[image: image242.png]




[image: image243.png]FriestandCampina ain




[image: image240.png]



[image: image240.png]

[image: image241.png]FriestandCampina ain




[image: image241.png]

	FactoryLink 7.5.2 server en client

	

	
	

	Installatie procedure FactoryLink 7.5.2. 

Server en (web) client

	DMV International
Project
FactoryLink server 7.5.2
Document ID
Install proc FLserver 7.5.2 V1.1.docx
Projectnummer
-

Status
Definitief
Installatiecodes
-

Datum
23 oktober 2014
Auteur(s)
R.J. Simons

Classificatie
Algemeen
R. Nooyen

Versie
1.1
R. Verduin

M. Jordaan


	


  ADVANCE \y 600.40 
1. Samenvatting

In de detailed design fase wordt de software architectuur, componenten, modules, interfaces, testaanpak en data van een software systeem verder verfijnd om zo de gespecificeerde requirements te realiseren. Het fysieke model zoals beschreven in de AD fase wordt verder uitgewerkt om zo een gestructureerde set van component specificaties te verkrijgen welke consistent, eenduidig en compleet gespecificeerd zijn. 

Dit detailed design document beschrijft de set component specificaties voor het <…> systeem.

2. Distributie

	Versie
	Datum
	Aan
	Bedrijf

	0.1
	18 juni 2002
	Wiljan Kluytmans
	DMV

	0.2
	26 juli 2002
	Wiljan Kluytmans
	DMV

	0.3
	10 nov 2005
	Wiljan Kluytmans
	DMV

	0.4
	23 aug 2006
	Wiljan Kluytmans
	DMV

	0.5
	09 feb 2007
	Wiljan Kluytmans
	DMV

	0.6
	12 dec 2007
	Marcel Jordaan 
	MPE

	0.7
	10 apr 2009
	Marcel Jordaan
	RLD

	0.8
	10 apr 2010
	Marcel Jordaan
	RLD

	0.9
	17 jan 2011
	Marcel Jordaan
	RLD

	1.0
	10 apr 2009
	Marcel Jordaan
	RLD

	1.1
	23 okt 2014
	Marcel Jordaan
	RLD


3. Overzicht
	Versie
	Datum
	Auteur
	Beschrijving

	0.1
	18 juni 2002
	R.J. Simons
	Initiële versie

	0.2
	26 juli 2002
	R.Nooijen
	Bijgewerkte versie

	0.3
	10 nov 2005
	R.Verduin
	Migratie Licence toegevoegd

	0.4
	23 aug 2006
	P. de Jong (MPE)
	Migratie versie 6.6 naar 7.5

	0.5
	09 feb 2007
	P. de Jong (MPE)
	Vervolg migratie 6.6.naar 7.5

	0.6
	12 dec 2007
	M. Jordaan (MPE)
	Aanvullingen  migratie 6.6 naar 7.5

	0.7
	10 apr 2009
	M. Jordaan (RLD)
	Aanpassingen voor Wei servers

	0.8
	10 apr 2010
	M. Jordaan (RLD)
	Aanpassingen voor Melkeiwit servers

	0.9
	17 jan 2011
	M. Jordaan (RLD)
	Aanpassingen voor Spray2 servers

	1.0
	10 apr 2009
	M. Jordaan (RLD)
	Aanpassingen voor Wei servers

	1.1
	23 okt 2014
	M. Jordaan (RLD)
	Aanpassingen voor WKC server


ADVANCE \Y 623.60
Dit document is eigendom van DMV International. Niets uit deze uitgave mag worden verveelvoudigd en/of openbaar gemaakt door middel van druk, fotokopie, microfilm of op welke wijze ook, zonder voorafgaande schriftelijke toestemming van de eigenaar.

ADVANCE \U 2.80©  2011 DMV International. All rights reserved.
4. Inhoudsopgave

31.
Samenvatting

2.
Distributie
4
3.
Overzicht
5
4.
Inhoudsopgave
6
5.
Introductie
8
5.1
Doel
8
5.2
Scope
8
5.3
Termen en afkortingen
8
5.4
Referenties
9
5.5
Overzicht
9
6.
Project standaarden, conventions en procedures
10
6.1
Design standaarden
10
6.2
Documentatie standaarden
10
6.3
Naamgeving conventies
10
6.4
Programeerstandaarden
10
6.5
Software development tools
10
7.
Component beschrijving
11
7.1
Installatie operating system.
11
7.1.1
Installatie Operating System.
11
7.1.2
BIOS setup Z400.
11
7.1.3
Installatie Microsoft Windows 2003 Server Standaard.
12
7.1.4
Opbouw computernaam
18
7.1.5
Update Windows Server 2003
22
7.1.6
Matrox display driver
22
7.1.7
Beeldscherminstellingen.
23
7.1.8
Beeldscherm(en) resolutie instelling
24
7.1.9
Instellen firewall Windows Server 2003
25
7.1.10
Instellen power opties
27
7.1.11
Aanmaken user (groepen).
28
7.1.12
Vrijgeven remote desktop
37
7.1.13
Automatisch inloggen
38
7.1.14
Netwerk instellingen
39
7.1.15
Taakbalk instellingen.
41
7.1.16
Disable the Shutdown Event Tracker in Windows 2003.
42
7.2
Installeren en configureren bginfo.
47
7.3
Installeren en configureren pagedefrag.
51
7.4
Installatie Total Commander
52
7.5
Installatie USB Device manager
56
7.6
Installatie 7-Zip
60
7.7
Installatie diverse programma’s
63
7.8
Installatie SQL Sever 2000
64
7.8.1
Installatie SQL Server 2000
64
7.8.2
Installatie Serviceopack 4 SQL Server 2000
73
7.8.3
FactoryLink SQL Database
77
7.8.4
ODBC Koppeling SQL Database
91
7.9
Installatie FactoryLink 7.5.1.
94
7.9.1
Aanmaken data mappen voor applicatie
100
7.9.1.1
Datastruktuur aanmaken
100
7.9.1.2
ODBC koppeling voor info database
101
7.9.2
Installatie Sinec H1 driver
104
7.9.3
Systeem variabelen servers
118
7.9.3.1
Environment variabelen MASTER server (server 1)
118
7.9.3.2
Environment variabelen SLAVE server (server 2)
118
7.9.3.3
Host variabelen
118
7.9.3.4
Service variabelen
119
7.9.4
Debug informatie voor Licentie Server
120
7.9.5
Wijzigen MAC adres server
121
7.9.6
Opstartscript FactoryLink clients
122
7.9.6.1
Informatie programma pslist
123
7.10
SQL-server Database instellingen
125
7.10.1
SQL Server stored procdures en jobs
126
7.11
McAfee Virus scanner
127
7.12
Clamwin virus scanner
129
8.
Opnieuw aanvragen van een FactoryLink license
135
9.
Applicatie wijzigingen doorvoeren
138
Appendix A
Source code listings
139
Appendix B
"Stop error code 0x000000A5" installing Windows
141
Appendix C
Een GPT-schijf (GUID Partition Table) converteren naar een MBR-schijf
145

Via de Windows-interface
145

Met een opdrachtregel
145
Appendix D
Requirements tracebility matrix
147
Appendix E
Conventions
148


5. Introductie 

5.1 Doel 

Document heeft het doel om als installatie/configuratie leidraad te dienen voor FactoryLink servers van de Melkeiwit (T01C1). Uitgangspunt voor de server configursatie zijn HP Z400 systemen.
5.2 Scope 

De FactoryLik servers voor de Melkeiwit, zijn twee, redundant opgestelde, servers.
5.3 Termen en afkortingen 

	Term
	Omschrijving

	
	

	Afkorting
	Omschrijving

	AD
	Architectuur Design

	ADD
	Architectuur Design Document

	AT
	Acceptatie Test

	DD 
	Detailed Design and production

	DDD 
	Detailed Design Document

	DFD 
	Data Flow Diagram

	ESA
	European Space Agency

	IT 
	Integratie Test

	SCMP 
	Software Configuration Management Plan

	SQAP 
	Software Quality Assurance Plan

	SRD 
	Software Requirements Document

	ST 
	System Test

	SVVP
	Software Verification and Validation Plan

	TBC 
	To Be Confirmed

	TBD
	To Be Defined

	TR 
	Transfer

	URD
	User Requirements Document

	UT 
	Unit Test


5.4 Referenties 

	Ref.
	Doc.Id
	Omschrijving

	[ESA]
	ESA PSS-05-05
	Guide to the software detailed  design phase, ESA Board for Software Standardisation and Control, Issue 1.

	
	
	


5.5 Overzicht 

6. Project standaarden, conventions en procedures 

6.1 Design standaarden 
6.2 Documentatie standaarden 

6.3 Naamgeving conventies
6.4 Programeerstandaarden 
6.5 Software development tools 

7. Component beschrijving 

7.1 Installatie operating system.

7.1.1 Installatie Operating System.

Voor FactoryLink versie 7.5 wordt het Microsoft Server 2003 Standaard operating systeem geinstalleerd op de FactoryLink server(s).
Gebruik de CD rom van Microsoft met de software voor Windows Server 2003 Standard Edition R2. 

Let er op, dat de hard disk compleet wordt gewist en geformatteerd, m.a.w. copieer belangrijke data die nog aanwezig is op de harddisk naar een veilige plaats. (Diskette, memorystick, etc.)
1. Start de server.

2. Plaats CD-ROM in de lade.
3. Herstart server, installatie wordt automatisch gestart.
Indien naar additionele bestanden of installaties wordt verwezen kunnen deze gevonden worden op de extra CD behorende bij het te installeren systeem. Op de CD zijn vermeld welk systeem het betreft en dat het de extra installaties betreft. In dit document wordt deze CD verder aangeduid als de extra-CD.
7.1.2 BIOS setup Z400/Z220/Z230.

In de setup wordt aangegeven dat de harde schijf in SATA RAID modus aangestuurd dient te worden. Het systeem is uitgerust met twee gharde schijven van identieke grootte, deze worden in RAID 1 geconfigureerd.
RAID configuratie is mogeijk via de BIOS: Gebruik de toets combinatie Ctrl + I om tijdens opstart in 

Aanpassen modus harde schijf:

Herstart de machine na deze uitgeschakeld te hebben. Open “setup” instellingen met F10 direct na opstart. Kies in menu ‘storage’ en vervolgens ‘storgage option’. Stel de harddisk modus (SATA emulation) in op ‘RAID + AHCI’ onder ‘SATA emulation’. 
Pas eveneens de opstart opties aan, zodat eenvoudig wordt op de RAID configuratie met Ctrl + I te openen. Kies in mune ‘Advanced’ en vervolgens ‘Power-On Options’. Kies nu de volgende instellingen:



POST Messages




Enable



F9 Prompt






Displayed



F10 Prompt





Displayed



F12 Prompt





Hidden



Option ROM Prompt



Enable



Remote Wakeop Boot Source

Local Hard Drive



After Power Loss




On



Post Delay (in seconds)



None

Sluit af met F10 en ‘Save changes and exit’, vanuit de menu optie ‘File’.
7.1.3 Installatie Microsoft Windows 2003 Server Standaard.

Volg de navolgende aanwijzingen.
	[image: image1.png]Windous Server 2883, Standard Edition Setup

Welcome to Setup.
This portion of the Setup progran prepares Microsoft(R>
UindowsCR) to run on your computer.

* To set up Windows now, press ENTER.

« To repair a Windous installation using
Recovery Console, press R.

« To quit Setup without installing Windows, press F3.




	Boot van de Windows Server 2003 Standard Edition Service Pack 1 CD.

Bij de <F6> opgeven van extra hardware, dient deze optie geactiveerd te worden. Bij vragen om een extra driver, de Intel RAID controller via een floppy disk laden: 
"Z400/Z600/Z800 SATA and SATA RAID"
Zodra dit scherm verschijnt druk op <Enter> om door te gaan.



	[image: image2.png]Windows Licensing Agreement

END-USER LICENSE AGREEMENT FOR MICROSOFT
SOFTUARE

MICROSOFT WINDOWS SERUER 2003, STANDARD
EDITION WITH SERUICE PACK 1

MICROSOFT WINDOWS SERVER 2003, ENTERPRISE
EDITION WITH SERUICE PACK 1

MICROSOFT WINDOWS SERVER 2003, STANDARD x64
EDITION

MICROSOFT WINDOWS SERVER 2003, ENTERPRISE
x64 EDITION

PLEASE READ THIS END-USER LICENSE AGREEMENT
<’EULA’> CAREFULLY. BY INSTALLING OR USING
THE SOFTWARE THAT ACCOMPANIES THIS EULA
<’SOFTWARE’ >, YOU AGREE TO THE TERMS OF THIS
EULA. IF ¥0U DO NOT AGREE, DO NOT USE THE
SOFTUARE AND, IF APPLICABLE, RETURN IT TO THE
PLACE OF PURCHASE FOR A FULL REFUND.

1. GENERAL. This EULA is a legal agreement
betueen you Ceither an individual or a single
entity> and Microsoft Corporation
CHicrosoft’>. This EULA governs the Softuare,




	Druk vervolgens op <F8> om de gebruikers overeenkomst te accepteren.
 

	[image: image3.png]Windous Server 2883, Standard Edition Setup

The following list shows the existing partitions and
unpartitioned space on this computer.

Use the UP and DOUN ARROM keys to select an item in the list.
« To set up Windous on the selected item, press ENTER.
* To create a partition in the unpartitioned space, press C.

* To delete the selected partition, press D.

4895 MB Disk @ at I1d @ on bus @ on atapi [MBRI
Unpartitioned space 4095 HB





	Verwijder in ieder geval de ‘C’ partitie (bij voorkeur alle aanwezige partities) met <D> <Enter> <L> zodat er “Unpartitioned space” over is en geen ‘C’ schijf meer aanwezig is.

	[image: image4.png]Windous Server 2883, Standard Edition Setup

You asked Setup to create a new partition on
4895 MB Disk @ at Id @ on bus 8 on atapi [MBRI.

© To create the new partition, enter a size
press ENTER.

* To go back to the previous screen without
the partition, press ESC.

The mininum size for the new partition is s
The maximun size for the new partition is 4887
Create partition of size (in MB>:

belou and

creating

megabytes (HB).
megabytes CHB).




	Maak een nieuwe partitie aan met <C> en met een grootte van 110.000 Megabytes. Druk op <Enter> om de partitie aan te maken. Dit is de ‘C’ partitie
Volgende alleen uitvoeren indien niet alleen de ‘C’ partitie verwijderd is: Selecteer daarna vrije ruimte en gebruik alle beschikbare ruimte om de 2e partitie van te maken met <C> en dan <Enter>.

Selecteer hierna weer de 1e partitie (Partition1) en druk op <Enter> om Windows op deze partitie te installeren.

	[image: image5.png]Windous Server 2883, Standard Edition Setup

The partition you selected is not formatted. Setup will now
Format the partition.

Use the UP and DOUN ARROW keys to select the file system
you want, and then press ENTER.

If you want to select a different partition for Windous,
press ESC.

[Fornat the ng the NIFS file system CQuick)
Format the partition using the FAT file system C(Quick>
Format the partition using the NTFS File system
Format the partition using the FAT file system





	Kies de 1e formatteer optie: “Format the partition using the NTFS file system (Quick)”

Druk op <Enter> om door te gaan.

	[image: image6.png]Windous

Server 2083, Standard Edition Setup

Please wait while Setup copies files
to the Windows installation folders.
This might take several minutes to complete.

Setup is copying files...
12%





	De schijf wordt nu geformatteerd en Windows kopieert de benodigde bestanden om het grafische gedeelte van de installatie te kunnen starten.

De computer zal tussendoor opnieuw opstarten.

	[image: image7.png]£3 Windows

@) Colloctin | Improved security

© Dynamic ‘The Windows Server 2003 famiy delivers enhanced security
Update with Intemet Information Services 6.0, which offers

restricted, safer functionality by default, SmartCard

© Preparing authentication, and improved security technologies ta protect
installation your nfastruchure
Instaling

L) Windows.
Finsling

® installation

Setup wil complete in
‘approximately:
35 minutes

Instaling Devices

— |
LS





	Het grafische gedeelte van de installatie is nu gestart.

	[image: image8.png]#5 Windows

Regional and Language Options
“You can custamize Windows fordiferent regions and languages.

Collecting
information

Update

o
© Dynamic
o

Preparing
installation

‘approximately:
33 minutes

®

Regional and Langusge Optians allo you ta change the way numbers, dates,
curencies and the ine are displayed. You can afso add suppt for sddiional
languages, and change your location selting

The Standards and formats setting i set o Engish (United States), and the
lacalon s set o United Staes.

L g

TestInput Languages allw you to ente text in many diferent languages, using
 vaiety of inpul methods and devices.

Your defaul text input langusge and method s: US Keyboard ayout

Ta view or change your eurent canfiguraton  cick Detals. Detais.
<Back Neit>





	Wacht tot het volgende scherm verschijnt en klik hier op “Customize”

	[image: image9.png]Regional Optins | Languages | Advrced |

[-Standards and formats

This option affects how some programs format numbers, currencies,
dates, and tine.

Select an tem to match s preferences, ar click Customize ta chaose
your own formats:

sanes

Long date: [ maandag 7 augustus 2006

-Location

weather, select your present locaton:

To help services provids you with local informetion, ﬁth s news and

= | &





	Wijzig “Standards and formats” 

in “Dutch (Netherlands)”
Vul bij “Location”, “Netherlands, The” in.

Klik op “OK”.

	[image: image10.png]



	Ga daarna bij het vorige scherm naar “Details”.

	[image: image11.png]Jrext Services and Input Languages e 1 £
[

senos |

[ Defaultinput language
Select one ofthe instaled input languages t use wher you stat your
compuer.

[Dutch (Netherands) - United States-Intemational

[-Installd services
Selectthe services that you want for each input language shown in the
fist. se the Add and Remave bultans ta modiy tis s,

[ utch (Netherlands)
5 Keyboard
+ United States-Interational

| Add
Femove.

Pioperies.

- Preferences

. || s
Cone |t





	Verwijder daar alle toetsenbordindelingen op “Dutch (Nederlands)” na.

Zodra je de instellingen opslaat zal Windows met een melding komen dat deze installingen pas actief worden na een reboot.

Deze melding is veilig om te negeren.

Klik vervolgens op “Next” om door te gaan.

	[image: image12.png]Personalize Your Software

Selup uses the nformalion you pravide about yoursel to personalze your Windows
software,

ﬁ Type yout fll name an the name of your compary o orgarization.

Name: [DMVINTERNATIONAL

Digarizaton [CAMPINAMELKUNIE

<Back





	Naam: 

DMV-INTERNATIONAL

Organisatie:

CAMPINA-MELKUNIE

Druk op “Next” om door te gaan.

	[image: image13.png]N N N




FC7FV – 9B44H – FYTGY – X7Q9Q - HDKVM
	Voer de juiste product code in en druk op “Next”

	[image: image14.png]Licensing Modes
‘Windaws Server 2003 suppoits o liensing modes.

@ Selecttheliensing mads you want o use.

 Per seryer. Number of concurrent comnections: =
Each connection must have it ou Client Access License.

& PerDevice or Per User.
Each device or user must have s ou Client Access License.

To avoid violtion of the License Agteemen, use Licensing (which s ocated
in Admiistatve Tools) o record the number of Cent Access Licenses
purchased.

Bk





	Kies bij het volgende venster voor “Per Device or Per User” 

Klik weer op “Next” om door te gaan.




7.1.4 Opbouw computernaam

· FL_<afdeling>_S<n>
· afdeling Є {SPRAY2 }

· n =1 voor primary master, n = master 2 voor secondairy 

Primairy master is de master waar normaal de FactoryLink applicatie op draait. De secondairy master is de master welke wordt geactiveerd bij problemen met de primairy master.

	Vul de juiste computernaam in volgens voorgaande samenstelling en druk weer op “Next”.

Het administrator wachtwoord bi voorkeur niet blanco laten, indien er geen wachtwoord vastgelegd is kan het volgende gebruikt worden: bgktv.

Eventuele meldingen over het niet veilig zijn van een administrator wachtwoord of ongebruikelijke tekens in de computernaam kunnen genegeerd worden.

	
[image: image15.emf] 

- -



	[image: image242.png][image: image243.png][image: image16.png]Date and Time Settings
Set the correct date and time for you Windows computer

[ Date & Time

ime Zane.

11:35.38 =

& [G7-01.00 i, G o e sk v =]

I e S8 o g s e

<Back





	Stel de tijdzone in op 

“(GMT +1:00) Amsterdam…” 

Vink “Automatically adjust clock for daylight saving changes” uit.

Druk op “Next” om door te gaan.



	[image: image17.png]Networking Settings
Instaling nietwork software allows you to connect to oher computers, networks,
and the Inteme.

&8, Choose whetherto use tpical orcustom stfings
L

& Typical settings
Creaes newark conections using th Clen o Hitosot Networks, i
and PintShating for icrosoft Networks, an the TCP/IP ansport
frctocel wih aiomatic addessing

" Custom settings
Alows youto manaly corfigut retworking components

<ok





	Om de installatie af te kunnen ronden laten we de TCP/IP instellingen op “Typical settings” staan. 

Later kun je een vast IP adres aan de netwerk kaart(en) toekennen.

Druk op “Next” om door te gaan.

	[image: image18.png]Workgroup or Computer Domain
‘A woikgroup i a collcton of computers that have the same workgioup name. 4
domainis 3 collection of computers defined by a network administator.

Do you want this computer to be a member of s domain?
Yot may need to cbtain this informalion from your netwark acmiisrator)

i ;s Cormputer s ot o 3 netwoik, o1 i on & retwork wihout 3 dorai
‘Make this computer @ member of the following workaroup:

[WORKGROUP

s, make this computer a member of the ollowing domai

<Back





	Laat de werkgroep instellingen ook staan en druk op “Next” om door te gaan.

	[image: image19.png][Welcome to Windows

7 7 WindowsServer200s

. Standard Edition





	Windows heeft nu genoeg informatie om de installatie af te ronden…

Na een herstart zal het inlog venster verschijnen.

Log hier in als administrator met het blanco wachtwoord

	Windows Server Post-Setup Security Updates -> Finish

Manage your server                                           -> Selekteer vinkje ‘Don’t display this page at logon’

Resolution                                                          -> 1024 x 768  32 bit  85 Hz.

	[image: image20.jpg]=lolx|

Bl Adion ew tep

crEEsemasna

= B
3 Computer

< Dikdives

2 Disly adepters

2 DVDJCD-ROM drives:

53 Floppy disk controlers

33 Human Inteface Devices

23 IDE ATA/ATAPT controllers

S IEEE 1394 Bus host controlers

5 Keyboards

) Mice and other pointing devices

Weniors
B Network adapters

. Ports (CoMaLPT)
A Processors
@, Sound, video and game controlrs
< Storage volues
System devices
556 Universal SerielBus controlers
Standard Enhanced PCT to LS8 Host Controller
Standard Enhanced PCT to USB Host Controller





	Ga naar “Start(Control Panel(System” 

En kies dan op het derde tabblad getiteld “Hardware” de “Device Manager”
Daar zul je zien dat er kaarten nog niet zijn geïnstalleerd.

	Op de deze website (www.hp.com) vind je de volgende drivers, tevens zijn deze op de extra-CD te vinden.

· Realtek Hight Definition Audio Driver -  WDM_R243.exe
· Intel Storage Manager – sp42696.exe

· sp45949.exe

· sp46138.exe

· sp46837.exe

· sp50118.exe

· 10-12_xp32_dd_ccc_enu.exe

· sp43515.exe

Installeer al deze drivers met de standard opties. Je hoeft tussendoor de computer niet opnieuw op te starten. Doe dit pas zodra alle drivers geïnstalleerd zijn.

	

	[image: image21.png]Copacity:
14,368 -
Bl system

|
allocation unitsze

[efauk alocationsize  ~|
Volume label

Data]

[Formatoptions ————————————|
IV Quick Format.

T~ Enable Compression

I Greate an 15-D05 startup disk

—





	Ga naar de Windows Explorer <Win+E> en klik op de 2e partitie Local Disk (D:)

Windows zal zeggen dat deze niet geformatteerd is en of je dat nu wil doen. Beantwoord deze vraag met “Yes”

In het formatteer programma geef je de Volume Label in en zet een vinkje bij Quick Format.

Klik vervolgens op “Start” om de partitie te formatteren.


7.1.5 Update Windows Server 2003

Een update van Windows Server 2003 SP2 wordt gedaan met behulp van WSUS-offline. Hiermee kunnen updates geinstalleerd worden zonder dat een internet connectie benodigd is.

Met het programma WSUSoffline, zie website www.wsusofline.net, 
7.1.6 Matrox display driver

Voor de FactoryLink server van de WKC (fl-util-s1), wordt een matrox display driver kaart toegepast. De server wordt diret gebruikt als een client, en wordt met een Matrox video kaart uitgerust. De huidige configuratie is met twee schermen boven elkaar, voor zowel de hardware installatie als de software voor deze kaart wordt verwezen naar de handleiding “Matrox® M-Series™ User guide”, bestandsnaam: ‘Matrox_en_mseries_guide.pdf’.

De toegepaste driver voor de matrox video kaart is: xddm32_213_03_002_se_u_whql.exe.

7.1.7 Beeldscherminstellingen.
	Rechtermuisknop op het desktopscherm en selekteer ‘Properties”.



	[image: image22.png]Themes | Desktop | Screen Saver | Appearance | Setiings |

Atheme s a background plus a set of sounds, icons, and other elements.
1o help you personalize your computer ith one cick.

Theme:
liodiedThere RS Delte
Sanple

active =lojx|

Normal _DcaoledSelected

2
Cancel i)





	Selekteer het tabblad ‘Screen Saver’.

	[image: image23.jpg]Display Properties

Bl

Soreen saver

Blerk ~] _ setings Preview

Wit [ 10 minutes [~ Onresume, password protect

Moritor power

T adiust monitor power setings and save energy.

el Pover,

Fower.

[ Cancel





	Selekteer ‘Blank’, Wait 10 minutes en activeer een wachtwoord bij hervatten.

Klik op ‘Apply’ en ‘OK’.




7.1.8 Beeldscherm(en) resolutie instelling
	Rechtermuisknop op het desktopscherm en selekteer ‘Propertys”.



	[image: image24.png]Themes | Desktop | Screen Saver | Appearance | Setiings |

Atheme s a background plus a set of sounds, icons, and other elements.
1o help you personalize your computer ith one cick.

Theme:
liodiedThere RS Delte
Sanple

active =lojx|

Normal _DcaoledSelected

2
Cancel i)





	Selekteer het tabblad ‘Settings’.

	[image: image25.png]21X
Themes | Deskiop | Screen Saver | Appearance Setings |

Drag the moritor cons to match the physical arangement of yout moritors.

Disply
1. Plug and Play Moritor on AT|FieGL V3100
Screen rssolon Colr qualty
Less = f—— More || [iighes (260 =

1024 by 768 pivels (S By B

¥ s s dyioe 2 the prima manit
19| Evter iy Windouss desktop cria s o

Identy | Troubleshoo.. | Advanced
Cancel i)





	· Color Quality: ‘highest 32 bit’. 

· Screen resolutie voor alle schermen ‘1280 by 1024 pixels’. Dit de enige resolutie die overeenkomt met de pixel instelling van de grafische plaatjes in FactoryLink.

· Herhaal dit voor alle schermen (indien van toepassing). Selekteer het scherm door in het venster op het desbetreffende scherm te kliken..

Klik op ‘Apply’ en ‘OK’.
WKC Server: In de WKC server wordt een matrox video kaart toegepast, zet scherm 1 en 2 boven elkaar (met 4 schermen, deactiveer scherm 3 en 4). Scherm 1 is het bovenste scherm, extend de desktop over het tweede scherm. De resolutie voor de WKC server is 1024 * 768, daar d esrver ook als (beeldvullende) client wordt ingezet.



7.1.9 Instellen firewall Windows Server 2003

	Instellen en activeren windows firewall: (Inloggen met administrator rechten)
Ga naar: Start > Control Panel > Windows Firewall. Voorwaarde is dat de server niet ingericht als domein controller en derhalve geen Active directory actief heft.

	[image: image26.jpg]General | Exceptions | Advanced |

Windows Fiewalis helping o protect your PC

‘Windows Frewal helps protect your computer by preventing unsuthorzed users
from gaining access to your computer thiough the Itemnet or a network.

@ng.;

This seting blacks al autside saurces fiom conneeting to ths
compute, with the exception o those selected on the Exceptions tab

I Don't allow exceptions

Select s when you connect o publ networks inss secure
locstons, such as aipots. You il not be ntiied when Windows
Fiwall bosks progtams. Selectons an the Exceptions tab il be
igrored

@ C ot

Avoid using tis seting. Turming off Windows Fiewallmay make this
computer more vunerable o viuses and intuders

For informalion, see the biciosolt Web site

Cance





	Het scherm ‘Windows Firewall’ wordt geopend. Er zijn drie tabs aanwezig, selekteer de eerste tab ‘General’.
Activeer de firewall door ‘On’ te selecteren.



	[image: image27.jpg]e

‘Windows Firewalis blocking incoming network connectons, except or the

progtams and services selected below. Adding exceptons alows some programs
o work better bt mightncrease your secuy sk

Programs and Services:

Name.

emole Desklop

PR Framework

Add Program. Add Pt Edit Dt

I Display a notfication when Windows Firewall blocks a program

Cance





	Vink alle aanwezige uitzonderingen aan, zodat deze toegestaan worden.

	[image: image28.jpg]el Excpions Advanced |

- Network Connection Settings

Windows Firewalis enabled o1 the connections selected below. To add
xceptions fo an individual conecton, select . and then cick Settings:

Setings.
Local Area Cornestion 2.

|- Secuity Logging

You can create 3 log il for troubleshosting purposes. S

[ICMP.

Wi Internet Cortrol Message Frotocol (ICMF), the
computers on a network can shre eror and staus
infomaton.

Setings.

[~ Default Setings

T restore allWindaws Firewal seffings o a defaul state,
clck Restare Default

Restore Defauls





	De firewall wordt voor alle netwerken kaarten ingeschakeld.


7.1.10 Instellen power opties

	De power instelling dient op ‘altijd aan’ ingesteld te zijn: (Inloggen met administrator rechten)

	Ga naar: Start > Control Panel > Power Options. 

	[image: image29.jpg]Pows Schenss | Advanced| Hberte| UPS |

Selectthe power scheme vith the mast appropiate setings for
this computer. Note that changing the setings below il macily
the selected scheme.

o

[~Poverschemes ————————————————————

[ —— |
Savehs Delete

[~ Selingsfor Always O pawer scheme |

Tun off monitor [After 20 mins -
Tun offhard dsks: ~ [Never =

System stancby: Never =

)

Cance





	In het tabblad ‘Power Schemes’ wordt het schema ‘Always On’ gekozen. Dit betekent dat het systeem nooit de harde schijven uitzet, en nooit in standby modus gaat! 

De Monitor mag na 20 minuten uitgeschakeld worden.
Voor de WKC server (fl-util-s1) wordt voor de monitor ook de optie ‘Never” gekozen, de WKC server is zowel server als client voor FactoryLink. Het is niet d ebdoeling dat na 20 minuten het scherm op ‘zwart’ gaat.



7.1.11 Aanmaken user (groepen).

	Aanmaken van Users: (Inloggen met administrator rechten)

	Ga naar: Start > rechtermuisknop op ‘My Computer’ en klik op ‘Manage’.

	[image: image30.png]=lolx|

/=) Fle acion vew Window He [ISETE|
A=
) Computer Mansgement (Loca) Nome

System Tods Tosystomtoos

@ Event vewer

Shared Folders
Lacal Users and Graups

Devie Manager
& &y siorage
Removale torage
Dk Defragnenter
Dbk Management
o Servces and Aplcations

g Performance Logs and Alert:

Sstorece

oserviesand Applcatons





	Het scherm ‘Computer management wordt geopend.

Selekteer de directory ‘Local Users and Groups’ > ‘Users’.



	[image: image31.png]5] Computer Manag -1o) x|
=) Fle action View window Help [ISETE|
EEal=IE] [2
8 Conpute Monagement (ocah | [ ams Trulame Toesaroten
T i Svstem Took hamnstator Bultin account For adminstering
0 Event vewer Bouws: Bultin accourt for quest access to
E Shared Folders Gee 3 member of Adminstratars
TG aperdcens | G b This user account s sed by the i
SUPPORT_38... Chi=Microsoft Corporatio... Thisis a vendar’s accountfo the -
2 Grops 4 . "

Performance Logs and Alert:
Device Manager

& &y siorage
Removale torage
Dk Defragnenter
Dbk Management
o Servces and Aplcations

Bovc_riaos

SVC_Flautostart

Factorylink Autostart Service Acco





	Klik met de rechtermuisknop in rechterscherm en selekteer ‘New User..’.

	[image: image32.png]Ueername:
Bivame: |
Descipon: |

Password
Coriim password:

[ User must change password at next lagon

I User cannot change password
I Passyord never expies.
I Accountis dsabled





	Het volgende scherm wordt geopend.

	[image: image33.png]hewuser

P —

Password

Coriim password:

I s st chenge s t e fogon
7 User cannotchange password

¥ P e ipied

T~ Account s disabled





	Vul het scherm in zoals hiernaast.

User name  = Operator

Full name   = Operator

Description = member of Operators

Password volgens paswoordenlijst bekend bij de ETD.

Bij ontbreken lijst gebruik als password: operator

Zet de vinkjes zoals hiernaast aangegeven.

Klik op ‘Create’

	[image: image34.png]Ueername:
Bivame: |
Descipon: |

Password
Coriim password:

[ User must change password at next lagon

I User cannot change password
I Passyord never expies.
I Accountis dsabled





	Het volgende scherm wordt geopend.



	[image: image35.png]hewuser

Eullname: [Engineer Factoylink |
Desciptiors [ member of Engineers |

Password

Coriim password:

I s st chenge s t e fogon
7 User cannotchange password

¥ P e ipied

T~ Account s disabled





	Vul het scherm in zoals hiernaast.

User name  = Engineer

Full name   = Engineer

Description = member of Engineers

Password volgens paswoordenlijst bekend bij de ETD.

Bij ontbreken lijst gebruik als password: engineer

Zet de vinkjes zoals hiernaast aangegeven.

Klik op ‘Create’

	[image: image36.png]Ueername:
Bivame: |
Descipon: |

Password
Coriim password:

[ User must change password at next lagon

I User cannot change password
I Passyord never expies.
I Accountis dsabled





	Het volgende scherm wordt geopend.



	[image: image37.jpg]rewuser -

Username:  [pms
Eullname: s

Desciptions [Admiistuator voor OFC
Password .

Coriim password:

T s st chenge s ot e logon
7 User cannotchange password

¥ P e sipies

T~ Account s disabled





	Vul het scherm in zoals hiernaast.

User name  = pms

Full name   = Ta_backup

Description = Administrator

Password volgens paswoordenlijst bekend bij de ETD.

Bij ontbreken lijst gebruik als password: engineer

Zet de vinkjes zoals hiernaast aangegeven.

Klik op ‘Create’

	[image: image38.png]Ueername:
Bivame: |
Descipon: |

Password
Coriim password:

[ User must change password at next lagon

I User cannot change password
I Passyord never expies.
I Accountis dsabled





	Het volgende scherm wordt geopend.

Klik op ‘Close’.

	[image: image39.png]5] Computer Manag =18
/=) Fle acion vew Window He [ISETE|
« > @@ |2

=] Computer Management (Lacal)
= iy system Tooks
@ Event vewer
SharedFolders
86 LocalUsers snd roups
DU
Groups
Perormance Logs and Al
Devie Manager
8 Storage
Removale torage
Dk Defragnenter
Ok ansgement
o Servces and Aplcations

Name

T Full Neme

T escrption

dministrator

r
Fowes
Hiwe
Loperaer
Fscivobueer
Fsuprorr 55
Bsvc riaies

Engineer mogen FactoryLi

3
Operstor mogen aleen Fa
5QLDebugzer
Ch=Microsaft Corporation,
SVC_Flautostart

Bullt-in account for administering
member of Engineers

Buitin account for quest access to
member of Administratars

mermber of Operators

This user account i used by the Vi
This i a vendor's account for the F
Factorylink Autostart Service Acco





	Selekteer de directory ‘Groups’.

	[image: image40.png]5] Computer Manag =18
/=) Fle acion vew Window He [ISETE|
« > @@ |2

=] Computer Management (Lacal)
= iy system Tooks
) Event viewer

Shared Folders
-8 Local Users and Groups
Users
a
g Performance Logs and Alert:

e Manor
5 5 Storege

Remavable Storage
Disk Defragmenter

Ok ansgement
b Services nd Appcstions

Name

T escrption

[ 4B naministrators
Bsackon Operators
Giovrniod com Users
Gowess

- (o
Gipertorance Log sers
Gipertomance vonkor U
Girower iers

Gpun operators
Diremote bestiop Usrs

Grepicar
Do

Bripserviescrous

Goperators
Grorciciens

‘Adrinistratars have complete and u.
Backup Operators can overids sec.
Members are allwed ta launch, acti
Guests have the same access as me,
Mermbers n this group can have som.
Mermbers of this group have remote:
Mermbers of this group have remote:
Power Users possess most adrinistr
Members can adminster domain prin
Mermbers nthis group are aranted .
Supports file replication i  domain
Users are prevented from making ac.
roup for the Help and support Center
Operators mogen llesn Factorylink
Mermbers of this roup have accesst.





	Klik met de rechtermuisknop in rechterscherm en selekteer ‘New Group..’.

	[image: image41.png]2ix
revcroe

Group name:

Description:

Members:

Add Herove

e Close.





	Het volgende scherm wordt geopend.

Klik op ‘Add..’

	[image: image42.png]TV zx
Groupneme:  [Opeaos
Dessrption[Operators mogen lken FactogLik bedenen

Members:

]

Herove

[





	Vul het scherm in zoals hiernaast.

Group name  = Operators

Description   = Operator mogen alleen FactoryLink bedienen

Klik op ‘Add..’

	[image: image43.png]fselectusers 21

Select this object type:

[Dsets or Buitin secuty principals Object Types.

Erom ths location
[CaMPINA BAUSGES Locations.

Enter the abiect names to select (esamples}:

(=i ||
(e |
[testiize]|

Ghesk Hiames.

Advanced.





[image: image44.png]Select this object type:

[Dsets or Buitin secuty principals Object Types.

Erom ths location

[CaMPINA BAUSGES Locations.

Enter the abiect names to select (esamples}:
[Operato] Check Names

e [





	Het volgende scherm wordt geopend.
Type in het tekstvak: ‘Operator’.

Klik op ‘OK’.

	[image: image45.png]TV zx
Groupneme:  [Opeaos
Dessrption[Operators mogen lken FactogLik bedenen

Members:

Operator

]

Herove

[





	Nu is user ‘Operator’ gekoppeld aan de group ‘Operators’.

Klik op ‘Create’.

	[image: image46.png]2ix
revcroe

Group name:

Description:

Members:

Add Herove

e Close.





	Het volgende scherm wordt geopend.

Klik op ‘Add..’

	[image: image47.png]reworow
Description: |Engineers magen applicaties installeren en wijzigen

Members:

]

Herove

[





	Vul het scherm in zoals hiernaast.

Group name  = Engineers

Description   = Engineers mogen alleen applicaties installeren en wijzigen.

Klik op ‘Add..’

	[image: image48.png]fselectusers 21

Select this object type:

[Dsets or Buitin secuty principals Object Types.

Erom ths location
[CaMPINA BAUSGES Locations.

Enter the abiect names to select (esamples}:

(=i ||
(e |
[testiize]|

Ghesk Hiames.

Advanced.





	Het volgende scherm wordt geopend.

	[image: image49.png]Select this object type:

[Dsets or Buitin secuty principals Object Types.

Erom ths location

[CaMPINA BAUSGES Locations.

Enter the abiect names to select (esamples}:
[Engineer Check Names

e [





	Type in het tekstvak: ‘Engineer’.

Klik op ‘OK’.

	[image: image50.png]reworow
Description: |Engineers magen applicaties installeren en wijzigen

Members:

Engieer

]

Herove

[





	Nu is user ‘Engineer’ gekoppeld aan de group ‘Engineers’.

Klik op ‘Create’.

Klik op ‘Close’



	[image: image51.png]5] Computer Manag =18
/=) Fle acion vew Window He [ISETE|
« > @@ |2

=] Computer Management (Lacal)
= iy system Tooks
@ Event vewer
SharedFolders
86 LocalUsers snd roups
Users
3 Grouss
Peformance Logs and Al
8 Storage
Removale torage
Dk Defragnenter
Ok ansgement
o Servces and Aplcations

Name

T escrption

[ 4B naministrators
Bsackon Operators
Giovrniod com Users
Gowess

- (o
Gipertorance Log sers
Gipertomance vonkor U
Girower iers

Gpun operators
Diremote bestiop Usrs
Grepicar

Do

Geogrens
Dricoservcessron

Goperators
Grerciciors

‘Adrinistratars have complete and u.
Backup Operators can overids sec.
Members are allwed ta launch, acti
Guests have the same access as me,
Mermbers n this group can have som.
Mermbers of this group have remote:
Mermbers of this group have remote:
Power Users possess most adrinistr
Members can adminster domain prin
Mermbers nthis group are aranted .
Supports file replication i  domain
Users are prevented from making ac.
Engineers mogen appicaties installer.
roup for the Help and support Center
Operators mogen llesn Factorylink
Mermbers of thi roup have access .





	Selekteer in de directory ‘Users’.

	[image: image52.png](D e acion yow wreow

Help

e | BA@EXEFR|R

=] Computer Management (Lacal)
= iy system Tooks
@ Event vewer
SharedFolders
86 LocalUsers snd roups
DU
Groups
Perormance Logs and Al
Devie Manager
8 Storage
Removale torage
Dk Defragnenter
Ok ansgement
o Servces and Aplcations

ez
Fowes
Hiwe
Loperaer
Fscivobueer
Fsuprorr 55
Bsvc riaies

Ful Name

Engineer Factorylink

3
Operator

SQLDebugzer
Ch=Microsaft Corporation,
SVC_Flautostart

member of Engingers
Buitin account for quest access to
member of Administratars

mermber of Operators

This user account i used by the Vi
This i a vendor's account for the F
Factorylink Autostart Service Acco





	Klik met de rechtermuisknop in rechterscherm op ‘Administrator’ en selekteer ‘Set password’.

	[image: image53.png]x|

‘ ‘You are logged in as "Adinistiatar". You have chasen ta reset the passwrd for your
lacal user accour.

Rieselfing ths password might cause ireversible loss of nformalion. For secury reasans,
Windaws protects cetain rformalion by making it mpossble o access f the passward s
resel.

This data lss will occur the nest ne you log of

‘Yo should use tis command arip i youive forgatten the password and do not have a
password esel k. you know the current password and want to change i, press
CTRL+ALT+DELETE and click Change Password

For addiional infomation, cick Help.

Cancel Help





	Klik op ‘Proceed’.

	[image: image54.png]New passord

Coriim password:

) Ityou clk OK. thefollwing il occur:

Yourlocal user account wil inmediately lose access o al o s
enciypted fles, stored passwords, and personal secuity cetiicates.

1fyou slick Cangel, the passward will ot be changed and no deta loss wil

==





	Vul het password twee maal in .

Password volgens paswoordenlijst bekend bij de TD.

Bij ontbreken lijst gebruik als password: bgktv

Klik op ‘OK’.

	[image: image55.png]1)  The password has been set.





	Melding dat password is gewijzigd.

Klik op ‘OK’.

	[image: image56.jpg]Gensl |

Desciption: [ mplete and unresticted access o the computer/domain

Members:

Administiatar

Lors

o
ok ==

Py





	De gebruiker ‘PMS’ dient toegevoegd te worden aan de groep ‘Administrators’. Open hiervoor in de directorie ‘Groups’, de eigenschappen van ‘Administrators’. Voeg de gebruiker ‘PMS’ via de knop ‘Add’ toe.

Sluit het venster af met de knop ‘OK’.

	[image: image57.png]=lolx|

(S oe scion von wdow b

BT

v |omBBI2

=] Computer Management (Lacal)
= iy system Tooks
@ Event vewer
SharedFolders
86 LocalUsers snd roups
DU
Groups
Perormance Logs and Al
Devie Manager
8 Storage
Removale torage
Dk Defragnenter
Ok ansgement
o Servces and Aplcations

Name [ FollName.

T escrption

dministrator

r
Fowes
Die v
Boresin ™ opeie
Gscuvenumer SaDsbugr

2 UpRORT 55... CHMiroset Corporation
Hisvc FLadtos.. SVC AAdostot

Engineer Factorylink

Bullt-in account for administering the,
member of Engineers

Buitin account for quest access to,
member of Administratars

member of Operators

This user account i used by the Vi
This i a vendor's accaunt for the He,
Factorylink Autostart Service Account





	Sluit alle schermen van ‘Computer Management’ af.


7.1.12 Vrijgeven remote desktop

	Vrijgeven remote desktop voor administrator: (Inloggen met administrator rechten)

	Ga naar: Start > Control Panel > System. 

	[image: image58.jpg]e N e e e
e I et

Selectthe ways that this computer can be used from another
localon.
- Remole Assitance

Tun on Remote Assistance and alow inviations to be sent fom this
computer

Leain more about Femole Assitance,

s

- Remote Desktop
I¥' Enable Remote Desktop an this computer

Impartart: To allow users ta cannect rematel o tis computer, cick.
Select Remote Lisers.

Full computer name:
(ER]

Leain mare about Remate Desklop.

0K Cancel Apply





	Het scherm ‘System Properties’ wordt geopend. Keis de tab ‘Remote’.

Geef  ‘Remote Desktop’ vrij door deze aan te vinken. En stel vervolgens ‘Administrator’ als gebruiker in waarvoor remote desktop is toegestaan.



	[image: image59.jpg]The usersfsted below can connect tothis compuler, and anp members of
the Admiistators group can connect even i they are ot ised.

FAdminsatar

Admiristitor aeady has access.

i

T create new user accounts or add users ta ather groups, go o Caniol

e e
Cance





	Op gelijke wijze dienen de gebruikers ‘Operator’en ‘Engineer’ toegevoegd te worden.


7.1.13 Automatisch inloggen
	Instellen en activeren automatisch inloggen voor administrator account: (Inloggen met administrator rechten)

	Ga naar: Start > Run. Start vervolgens de applet met ‘control userpasswords2’.

	[image: image60.jpg]ron -

Type the name of a program, older, docurent, or
Internet resource, and Windows wil open t For you.

e

e





	Met de tool "control userpasswords2" kunt u de gebruikersaccounts regelen, bijvoorbeeld dat iemand een wachtwoord moet invullen, als hij van zijn pc gebruik wil maken. Of juist niet. Ook kunt u gebruikers de toegang tot een pc geheel weigeren.

	[image: image61.jpg]user accounts 21|
Uses | advanced

Use the st below to grant or deny users access to yaur.
computer, and to change passwords and other settings.

I e kg e S s s i ot

Users or ths computer:

User Name Growp
dministrator ‘Administrators; Remote Desk

A gemove | | Properties
[-Passward for Adhinstrator

g To change your password, press Cirk-Ak-Del and select

Lt
R
S| e





	Verwijder de vink voor ‘Users must enter a user name …..’. Nu hoeft niet meer ingelogd te worden met een gebruikersnaam en een wachtwoord. Na bevestigen dient in het volgden venster de gebruiker en wachtwoord opgegeven te worden waarmee automatisch ingelogd dient te worden.

	[image: image62.jpg]| Automatically Log On'

% You can set up your computer 50 that users do not have to type

user name and password to log on. To do this, specfy a user thet
wil be automaticaly logged on below:

User name: Advinistrator

password

Confirm Password:

Cancel





	Vul het wachtwoord voor het administrator account in, en bevestig met ‘OK’.


7.1.14 Netwerk instellingen

	Instellen netwerk IP-adressen: (Inloggen met administrator rechten)

	Ga naar: Start > Control Panel, open vervolgens ‘Network Connections’.

	[image: image63.jpg]_=loix|

Bl Edt Vew Favortes

Tools

Advanced _telp

| &

QBack - ) - T | search

i Foers 13 X 9| @

[ yer—

EE

Folders

5 captures
© dbogz
& s
Sty
5 st Dversen
& o207, 178
5 sau server
Toc1_InFod7s 5.2
25 BRMSVOL_EN (E1)
& B Control panet

§ acinistratve Toos
& Fonts
Y v oo
) portabe Medi Devi
., Printers and Faxes
S scomners snd Camer
A schecded Toss
2y Network lces
 Recycein

LAN or High-Speed Internet

Lacal frea Connection
Connected, Frewaled
T, Erosdcon fetirems Gigabi E.

wizard

@ New Connection Wtzard

PLC Connection
etwork cable unplugged, Fire.
T, Brosdcon etireme Ggabi E.





	Indine nodig hernoem de netwerk verbinding die voor het PLC koppelnetwerk wordt gebruikt tot ‘PLC Connection’.

Vervolgens worden de (vaste) ip adressen van de server ingesteld, volgens de IP adressen lijst.

	[image: image64.jpg]General | Auhentcation| Advanced |

Connest using

89 Bioadoom Nebdreme Gigabi Etherme | Configure

This cornection uses the foloning tems:
1 B Clent for Microsoft Networks.

1 /S Netverk Load Balaning

{23 Fic and Printer Sharin for Micrsof etworks
Intmet Protocol (TCPAF)

sl | Uninsal Fiopertiss
[ Descipion
Allowsyour computer 0 acsess fsousces on aMicosolt
netwak

¥ St con oo iea wher Soresied
% Noliy me when tis connection has ied or o coectivty

oK Cancel





	Open voor iedere netwerk connectie de instellingen door met de rechtermuis toets de ‘properties’ van een netwerkverbinding te openen. Stel vervolgens ip-adres en subnet mask in. Het subnetmask is voor iedere systeem gelijk: 255.255.254.0

	[image: image65.jpg]o |

You can get IP settings assigned automtically I your network supports:
this capabilty. Otherwise, you need to ask.your network dnistrator
for the appropriate IP sektings.

 Qbtain an IP adsess automaticaly

Use the follwing IP sddress:
1P address: 0.7 4 et
Subnet mask: 25 255 .25 0

Default gateway:

€ Obtain D3 server address autamaticall
[+ Use the follawing DI server addresses;

Breferred DS server:

alernate DS server





	Vul voor beide netwerkkaarten het juiste IP-adres in, en let op het subnet-mask. De overige velden kunnen leeg blijven.

	[image: image66.jpg]General | Auhentcation| Advanced |

Connest using

89 Bioadoom Nebdreme Gigabi Etherme | Configure

This cornection uses the foloning tems:
1 B Clent for Microsoft Networks.

1 /S Netverk Load Balaning

{23 Fic and Printer Sharin for Micrsof etworks
Intmet Protocol (TCPAF)

sl | Uninsal Fiopertiss
[ Descipion
Allowsyour computer 0 acsess fsousces on aMicosolt
netwak

¥ St con oo iea wher Soresied
% Noliy me when tis connection has ied or o coectivty

oK Cancel





	Open de opties voor de ‘PLC Connection’ verbinding en stel het ‘Internet Protocol (TCP/IP)’ in.

Door de optie ‘Show icon in notification area when connected’ aan te vinken is altijd de verbindingsstatus zichtbaar rechtsonder in de taakbalk.

	De WKC server (fl-util-s1) wordt voorzien van een derde netwerkaart, deze derde netwerkaart wordt gebrikt voor het ‘printer’ netwerk. Het netwerk adres op deze kaart is voor de WKC server:

192.168.254.10 met subnet mask 255.255.255.0. De kantoor printer heeft in dit sub-netwerk een netwerk adres: 192.168.254.110. 


7.1.15 Taakbalk instellingen.

	Open: Rechtermuisknop op ‘Start’ en selekteer Propertys.

Het navolgende scherm wordt geopend.



	[image: image67.png]Testr | st

[~ Taskbar appearance
¥ Lock the taskbar
IV Auto-hide the taskbar
[V Keep the taskbar on top of other windows

19" Groupsimier taskbar butons
¥ Show Quick Launch

[~ Notfication area

¥ Show the clock

‘Yo can keep the nolfication area uncuttered by iding cans that you
have nat licked receny.

I~ fide nacive sand Costnize
Cancel Aoy





	Selkteer tabblad ‘Taskbar’.

Vink de vinkjes aan zoals hiernaast is aangegeven.

Klik op ‘Apply’ en ‘OK’


7.1.16 Disable the Shutdown Event Tracker in Windows 2003.

Introduction

    "The Shutdown Event Tracker is a Microsoft Windows Server 2003 and Microsoft Windows XP feature that you can use to consistently track the reason for system shutdowns. You can then use this information to analyze shutdowns and to develop a more comprehensive understanding of your system environment." microsoft.com

The idea behind the shutdown event tracker is that a server isn’t meant to be restarted or shutdown regularly. Therefore, when it is, Administrators should keep a log of exactly why the machine was powered down. Essentially, this can be a good thing since it allows you to store a database of shutdown events for future reference.

For some people, especially those that use Windows 2003 as a client operating system or in a test environment - where restarting or shutting down a machine can be a common procedure - it might get to be quite annoying.

Note: This feature does come with Windows XP Professional as well, but is disabled by default. 

When you click on Shut Down… from the Start menu, the Shutdown Event Tracker pops up asking whether you want to Log Off, Restart or Shut down the computer. 

[image: image68.png]@ What do you want the computer to do?

St down -

Ends your session and shuts dawn Windows so that you
can safely turm off power.

Shutdonn Event Tracker
Select the option Ehat best describes why you wank to
Shut down the computer

Option ¥ planned

[Othr (Planmed) 5]

A shutdown orrestart fo an unknown reasan

Comment:

o el |





Note: When logging off, the Shutdown Event Tracker is grayed out. 

If you decide to Shut down or Restart the machine, you will be given seven Shutdown Event Tracker options to choose from. These will allow you to best describe why the computer is to be shutdown or restarted. You can also add a comment in the Comment box which is very useful for helping you to determine the reason for the shutdown. The following are the seven event tracker options available, and an example of what might normally be written in the Comment box.

Other (Planned) – A shutdown or restart for an unknown reason.

This is usually chosen when the other options do not describe why a shutdown or restart of the machine is taking place.

Comment: Shut down virtual test machine. Time to go home!

Hardware: Maintenance (Planned) – A restart or shutdown to service hardware on the system.

Choose this option when you want to carry out planned maintenance on the machine’s hardware.

Comment: Change Serial ATA cable.

Hardware: Installation (Planned) – A restart or shutdown to begin or complete hardware installation.

Choose this option when you plan to upgrade or install additional hardware on the machine.

Comment: Install a new 200GB hard drive.

Operating System: Reconfiguration (Planned) – A restart or shutdown to change the operating system configuration.

This option is for when you have made operating system changes that require a restart or shutdown of the machine. When you rename a computer or install an additional component, for example.

Comment: Installation of DNS Server Service.

Application: Maintenance (Planned) – A restart or shutdown to perform planned maintenance on an application.

This option would be chosen when a planned upgrade or re-configuration of an application took place.

Comment: Upgraded to ISA 2004 Service Pack 1. Restart required.

Application: Installation (Planned) – A restart or shutdown to perform application installation.

Choose this option when a planned installation of a new application has taken place.

Comment: Installed SQL Server 2000. Restart required.

Security issue – The computer needs to be shut down due to a security issue.

This option would be chosen when the machine needs to be restarted or shut down for security reasons.

Comment: DOS Attack.

Viewing Shutdown Event Tracker events

To view previous Shutdown Event Tracker event logs, go to the Event Viewer (Start > Programs > Administrative Tools > Event Viewer or Control Panel - Administrative Tools - Event Viewer) and under the System Log, search for Information Events with ID 1074 or 1076. Double click the event to bring up the Event Properties page.

Note: 1074 Events are logged when you manually shutdown the machine using the Event Tracker. 1076 Events are logged when the machine shuts down unexpectedly and the Event Tracker pops up when the Administrator (or first user with shutdown rights) logs on to the machine.

[image: image69.png]Date  [i2/06/2005 Souce:  USER32
Tipe: 131920 Catego: Nome
Tpe  Infomalion EventD: 1074
User  WINZ003\Adririststor
Computer: ZTABONA

Dessiptor:

[The pracess Explrer EXE has niialed the shuldawn of compuler
[ZTABONA on behalf of user WIN2003Admislrator for the following
isason: Hardware: Instaltion (Planned]

Fisason Cade: 0434010002

Shutdonn Type: shutdown

Comment: Add a 168 RAM mode.

For more information, see Help and Suppot Certer at
Iito: /g0, mictosolt com/fwink /2verts:asp.

Data (" Bytes & Words
o0 s4010002 |





As you can see in the image above, the Description indicates the reason for the shutdown, the time, the user that initiated the shutdown, as well as the comment that was typed in the Comment box.

Disable the Shutdown Event Tracker
If the event tracker is of no use to you then you can disable it. To do this, open the Group Policy Object Editor Console. Go to Start > Run…, type gpedit.msc and press OK.

Navigate to Computer Configuration > Administrative Templates > System and in the right hand pane, select the “Display Shutdown Event Tracker” setting.

Double Click this setting to open the Properties page. You are now given the option to leave it in a default state of Not Configured, set it to Always Enabled, Enabled for Servers/Workstations (Windows XP Pro) or Disabled completely (as the image below demonstrates).

[image: image70.png]e Aton yew teb

s OEEB @

[ el Carper oy
8 connaerCotiuration
2 ot St
&1 (2 Windows Settings
= (2 Administrative Templates.
520 Vv Coperents
® 3 System
& (2 Network

& Aduritrate Tenplates

[oeting

[ Dauser roies

| Ssarpts

| Stogn

| S0k Quotes.

| et Logon

| ShGroup polcy

| Saremate asstance

| SisystemRestore

| SierorRepartion

| Savinons Fi Protection

| Srente procedure Cal

| Cawindows Time Service:

134 Restrct potentialy unsafe HTM. Help functions to spectied fodars
00 ot dply Mansge Your Server page &t ogen

Acvate Shsdoun Event TrackerSystem State Datafesure
Enatle Persstent T Stanp.
Sy Windows nstlatn e ocation

Sy Windows Sevce Pack stalton e locaton

Nt confiured
ot confiured

Nt confred
ot confipred
ot confiured
]

Btendsd ) stanivd





[image: image71.png]Display Shutdown Event Tracker Properties! 2%

ating ||

28 Disply Shltdown Event Tracker

 Not Configured
" Enabled
& Disabied

Shutdonn Evert Tracker should be displayed:

Supported on: At lsast Mictosoft Windows XP Professionalor Windo.

BoviSatg_| et
==

ply





Note: When you enable the Group Policy for Server only, the Shutdown Event Tracker appears when you shut down a computer running Windows 2003, whereas for Workstation only, the Shutdown Event Tracker appears when a computer running Windows XP Professional is shut down.
[image: image72.png]Copyign @ 1985.2005 Ao Corporson

@ What do you want the computer to do?

Ends your session and shuts dawn Windows so that you
can safely turm off power.

coee | sen





After you make the change to the Group Policy, open the Command Prompt and run the gpupdate /force command to refresh the policy and have your settings be applied straight away. Alternatively you can just restart the machine.

When you next attempt to shutdown or restart the machine, the Shutdown event tracker will no longer be visible and the normal shutdown prompt will appear (as seen in the image below).

Missing folders in Group Policy Editor
If folders are missing in group policy editor, like the ones mentioned above, it is probably caused by installing IE7, do the following to correct this issue:

· Right Click on Administrative templates.

· When the Add Remove Templates Dialog appears, click the ADD Button.

· This will bring up the Policy Templates dialog. Highlight “system.adm” and click the Open button.

· Click the Close button on the Templates Dialog.

How to enable and disable Shutdown Event Tracker from the registry
1. To open Registry Editor, click Start, click Run, type regedit, and then click OK.

2. In Registry Editor, navigate to the following registry key: HKEY_LOCAL_MACHINE\Software\Microsoft\Windows\CurrentVersion\Reliability
3. Name:ShutdownReasonUI
Type:REG_DWORD
Value:1=enable;0=disable
7.2 Installeren en configureren bginfo.

	Installeren en configureen bginfo.exe: (Inloggen met administrator rechten)

	Copieer vanaf de extra-CD het programma bginfo.exe naar ‘C:\Program Files\Sysinternals’.

	[image: image73.jpg]_=lojx|

Fle Edt Yew Favortes Toos teb | &

Qack ~ ) + ¥ | O search [ Foldrs | 15 3 X 9 | @~

Aderess [ ciprogram Flesisysrermls ELE]

= | See [Type Date Hodried aurbutes
53 Documents and et o] | BBorfe o 942K8 pplcation Zemvrsz A

&l 2 Program Files

122 Analog Devices

(22 AT Technologies

122 Broadeom

2 emak

122 Common Files

1 ComPus Applicat

12 Hoversnap

(2D Instalishield Inst.

D el

5 Tnternet Explorer

12D NetMesting

122 Online Services

5 Outlook Express.
tornal

Uninstal Informa

= 2





	Indien de directory Sysinternals nog niet bestaat, maak deze dan aan. Via rechter muistoets op ‘Program Files’ en keuze ‘New Folder’.


	[image: image74.jpg]2 BGInfo - Default configuration

Fle Btmap Edt

Logon Domain
Logon Server:
MAC Addres:
Machine Domain:

<Boot Time>
<CPU>

<Default Gateway>
<DHCP Server>
<DNS Server>
<Free Space>
<Host Name>

<IE Version>

<IP Address>
<Logon Domain>
<Logon Server>
<MAC Address>
<Machine Domain>
<Memory>
<Network Card>
<Network Speed>
<Network Type>
<08 Version>
<Service Pack>
<Snapshot Time>
<Subnet Mask>
<System Type>
<User Name>
<Volumes>





	Start het programma bginfo door hier dubbel op te klikken. Het volgende venster verschijnt.

	[image: image75.jpg]2 BGInfo - Default configuration 3

Ele Btmep Ed Format b

<Boot Time>
<CPU>

<Host Name>
<IE Version>
<IP Address>
<Subnet Mask>
<MAC Address>
<08 Version>
<Service Pack>
<System Type>
<User Name>





	Wijzig de instellingen zoals hiernaast aangegeven, kies vervolgens ‘Position’ en stel in zoals aangegeven.

	[image: image76.jpg]Locate on scieen Linitines to
¢ o 9 =] 220 inches wide

¢ o

Compensate for
Taskbar posiion

v e @

Multple Monitor Configuraton.

Cance





[image: image77.jpg][Multiple Monitor Selection:

O systems with more than one display monitor you can have.
the textdisplay on a single moritor, o on ol moritors

Display the text o

 Biimay diplay moniey | Moriorlocaled ot

Al disley monitors L
R
R

)





	Vastleggen schermpositie.

	[image: image78.jpg]=loix]|

Ele Edt View Favortes Toos Help

| &

QBock = )+ ¥ | O search [ Folders | 1% 3 X 9| @~

Adress [ c:\program Fies|sysinternals

EE

Folders
122 ComPlus Applications
122 Hoversnap

12 Instalishield Installstion Informatior
D mtel

5 Tntermet Explorer

122 NetMesting

122 Online Services

(22 Openoffice.org 2.3

12 Opera

2 Outlook Express

L) Sysinternals

122 Ukratic

12 Uninstall Information

Name -

5o [ Type

Dte Modfied

Hsaroecs

942K8Applcation
1B Shorteut

27-9-2007 9:21
16-12:2007 21:29





	

	[image: image79.jpg]Gererl Short | Compatity | Securty |

Shortcut to Bainlo.exe.

Targettype:  Applcation
Target location: Sysitemals

Target

Statin \Program Fies\Gpsiieinals’

Shotcutkey:  [None.

Bun Nomal vindon 5

Comment

Eind Target.. | Change loon Advanced





	Maak een snelkoppeling (shortcut) aan die wijst naar bginfo.exe in de ‘C:\Program Files\Sysinternals’ directorie. Open de properties van deze snelkoppeling en voeg als parameter ‘/timer:0’ toe. Dit zorgt er voor dat de gegevens op de desktop eenmalig bijgewerkt worden.

Vervolgens de snelkoppeling verplaatsen naar de map opstart map van alle gebruikers. Dan wordt deze actie eenmalig bij opstart van het systeem uitgevoerd en is zichtbaar om welk systeem het gaat, en wanneer deze herstart is.

De opstart map voor alle gebruikers is:

‘C:\Documents and Settings\All Users\Start Menu\Programs\Startup’

	[image: image80.jpg] =loix|

Ble gt

Vew Favortes Tooks Help

| &

Qbock - ) + ¥ | O search [ polders | 1% 3 X 9| @~

‘ddress [[7) C:\Documents and Settings!all Usersitart MenulPrograms!startup

EE

= @ prograns

0 Acessories
D Adminstratie Toos
) AT HoRAISION
D Autortiey
(=] openofice.org 23
3 oo
© srtp

0 Toal Commander

@ e

Flders R Sz [Tyee Dt Hedfed
T < [lesarier K6 shona \sz 2
B i oeumelis 16 shortas 161220072129
5 & ot Mo [Fl5hortcat to tertup.bat 166 shortaut 16-12:2007 22:37





	


7.3 Installeren en configureren pagedefrag.

	Installeren en configureen pagedfrg.exe: (Inloggen met administrator rechten)

	Copieer vanaf de extra-CD het programma pagedfrg.exe naar ‘C:\Program Files\Sysinternals’. Het programma bevindt zich in een gecomprimeerd (zip) bestand, te samen met een help file, PageDefrag.zip

	[image: image81.jpg]~=lolx|

Ele Edt View Favortes Toos Help

| &

QBack + ) - (| ) search [ Foiders | 1

o X9 @

Adress [ c:\program Fies|sysinternals

EBE

Folders
5 Tntermet Explorer
122 Microsoft Office.

15 Microsoft QL Server
2 ML 4.0
[SLEIEY)

122 NetMesting

(22 Network Associates
1D Notepad++

122 Online Services

(22 Openoffice.org 2.3
12 Opera

5 Outlook Express

0 Sysintermals

122 Tecnomatix

Name -

S

Type

Hsaroecs

@pegedivap
Blshortct o Bafo.exe

<

942K8
211k

BT
1ke

Appll
Appll
Help

Shart

IDescription: System defragmenter Company: Sysinternals Fie ersion: 2,32, 210 KB

J ¥y Conpuer

L]
7





	Indien de directory Sysinternals nog niet bestaat, maak deze dan aan. Via rechter muistoets op ‘Program Files’ en keuze ‘New Folder’. Copieer de bestanden ‘pagedfrg.exe’ en ‘pagedfrg.hlp’ naar de directorie ‘C:\Program Files\Sysinternals'.



	[image: image82.jpg]File Clusterls) _Fragment(s]
Clpacelle e e 1
CNDOWS ysematcontg\ppEvent v &0 1
CAWINDOWStsysem32iconfpdeul I
CWINDOWS\paen32tconfialieinetExplorer.evt i1
p CAWINDOWStepslen 2\coni\SAM w1
W | cvnoovssiemeia et enen T2 3
t | [CONDOWS e SECURITY 64 1
(05§ | |CUWINDOWS \system32\confictsoftrere a1 -
gl ;| |COWINDOWS system3ziconis &t 1
i | IERRR e & 1
©i! | | oeteonensicnons
-Ej ! Delragment a et bact c
i Defignentever oot ©
i Defig abot souridown [3 2] seconds
KN |
Don't defragment (uninstall] L

Coppight © 20022004 Mark Russinovichi

Susinleials - w sysintemals com o [





	PageDefrag defragmenteert het wissel bestand, tijdens de opstart van het systeem. Selecteer deze optie, met als afbreek optie 3 seconden.


7.4 Installatie Total Commander
	Start de installatie: Dubbelklik op “Total Commander\ tcmdr655.exe” 



	[image: image83.jpg]Installation Total Commander 6.55
% Install: Total Commander

Please select alanguage/
Bite wahlen Sie eine Sprache.
5.v.p. choisssez une langue:

Engish
Devtsch (Geman)

Dansk Danish]
Norsk (Nowegiar
Itaiano (taian)

Svenska Swedsh)

B =





	De installatie start met een keuze van taal instelling. Kies Nederlands, en ga vervolgens door de installatie zoals in de volgende schermen aangegeven. De default instellingen kunnen aangehouden worden, zorg ervoor dat dezelfde instellingen als in de figuren worden gekozen!



	[image: image84.jpg]Installation Total Commander 6.55

% Install: Total Commander

Do you also want to nstal al other languages?
Vol can then swich between these anguages while the
progiam i ruring.

i)
z &

ok





	

	[image: image85.jpg]Installation Total Commander 6.55
% Install: Total Commander

Flease enter target dectory forinstallaion





	

	[image: image86.jpg]Installation Total Commander 6.55
% Install: Total Commander

Yo can define you own lacation fo the seltings (i fils).
Thisis oy recommended for advanced users.

Changs e location





	

	[image: image87.jpg]Installation Total Commander 6.55
% Install: Total Commander

Do you want to create a gioup in the Start men.?

& Yes
C Mo





	

	[image: image88.jpg]Installation Total Commander 6.55
% Install: Total Commander

Do youwart ta create an con an the Deskiap?

& Yes
C Mo





	

	[image: image89.jpg]Installation Total Commane

1 ) Installation successfull





	Na voltooien van de installatie dient een licentie voor Total Commander in de programma map geplaatst te worden.

	[image: image90.jpg]=lolx|

Ele Edt Vew Favortes Took

Help

| &

QBock = )+ ¥ | O search [ Folders | 1% 3 X 9| @~

Address [ c:yotakomd

EE

Fodrs
0ty Documents
2y Compuer

] % System (C:)
© Documents and ettgs
© progam s
= 3 totskemd

© Lavaunce
& wivoows
© vt
[l % Data (D:)
© cotures
© sz
Ol
Sriag
= (2 Installatie Diversen
© Autoruns
Soama
Somes
Srust
©neys
5 Motepsdr
O
© totpad
S Tosl Commander
© wrusscansot
S Weivener_Winzoos
S

(] Hoversnap v08.2ip

e Sz [Tyee Dt Hedfed
ST Pl Folder 112200 2227
Slcsencon 1248 Applcaton Exterson 26
Slcarrio o6 i devkedver 26
Bcarmrsrs " o
e 16 e Fie %
EHcescription 26 1oNFic i
[S]FRerES32.0LL 88 Applation Extension 26
HisToRY. 1T 76 Tocbeament 2
) ermomeo.1ir o6 Tetboament 2
Hvosse 16 e Fic o
ReGITER ATF 46 Rich Toxt Document 26
e Py o
Claweee wreie Ko At %
Ercumst e 5@ Appcation i
Sjrcunmsr e 26 WL Fie o
Srenamon 100K8  Applcaton Extersion 26
Blromam.ee 2ks Appcation o
SlroraLci eve mavreesr K6 MANFEST Fe o
@ TOTALCMD.HP 490KB Help File 2
B roracm.ne 56 MRl o
Swacereou 76K Applcaton Extenson 26
Sluvearolt 154K8  Applcation Extension 26
Clwczrosee 440 st o
Bwermconsou 19K Applcaton Exterson 26
Slwerzraz o 45K Applcaton Extenson 26
[l e WL Fie o
1K KEvFie o





	

	Kopieer Total Commander\WINCMD.KEY naar de directory waar Total commander is geïnstalleerd, dit is de map ‘C:\totalcmd’.
	


7.5 Installatie USB Device manager 

	What it is

	  
USBDLM is a Windows service that gives control over Window's drive letter assingment for USB drives. Running as service makes it independent of the logged on user's privileges, so there is no need to give the users the previlege to change drive letters.
It automatically solves conficts between USB drives and network or subst drives of the currently logged on user.
Furthermore you can define new default letters for USB drives and much more. 
	

	 
	


	 
	Getting Started

	
	  
When a removable drive (USB flash drive, flash card reader, portable hard drive) is attached for the first time, Windows mounts it to the first available 'local' drive letter. If there is a network share on this letter, Windows XP will use it anyway for the new USB drive because since XP network shares are specific to the current user and not visible in the context of the system where the letter is assigned. The USB drive then appears to be invisible.
You can change the letter assignments in the Windows Disk Management Console with a lot of mouse clicks but you have to do it again for every new device.
And, for USB devices that have no serial number (in violation of the USB standards) you have to do it too when you attach it to a different USB port. 

USBDLM can for newly attached USB drives 

· check if the letter is used by a network share of the currently logged on user and assign the next letter that is really available 

· reserve letters, so they are not used for local drives 

· assign a letter from a list of new default letters, also dependend on many different criteria as the active user, drive type, connection (USB, FireWire), USB port, volume label, size and others 

· assign letters for a specific USB drive by putting an INI file on the drive 

· remove the drive letters of card readers until a card is inserted 

· show a balloon tip with the assigned drive letter(s) 

· define autorun events depending on many different criteria 

· many other things, see help file, available online as HTML version too 

All functions are applied to USB drives at the moment they are being attached, when the USBDLM service starts up and when a user loggs on. 

USBDLM runs as Win32 service under Windows 2000, XP, Server 2003 and Vista. 64 Bit versions are untested but should work too. 
	

	
	 
	


	 
	Installation

	
	  
Unzip the distribution (USBDLM.ZIP) file, e.g. to your programs folder (a USBDLM folder is included). 

Then enter your prefered configuration into the USBDLM.INI (see below). There is a sample file included called USBDLM_sample.INI. Either create a new USBDLM.INI or rename the sample. 

If you are logged on with administrator privileges you can simply install it. 

_install.cmd

Without administrator privileges you will be prompted for them. 

You can change the USBDLM.INI without restarting the service because it's read each time a new drive gets assigned a drive letter. 

You can uninstall the service using parameter -uninstall: 

_uninstall.cmd

Both can be done without the message box by using the following USBDLM commandline parameters 

USBDLM -silentinstall

USBDLM -silentuninstall

It returns Errorlevel 0 on success, 1 on failure. 

You can stop and start the installed service by means of the _stop.cmd and _start.cmd. 
	

	
	 
	


	 
	Configuration

	
	  
Running without a configuration USBDLM only prevents that an USB drive gets the letter of a network share or a subst drive of the currently logged on user. It remounts then to the next letter that is really available. 

USBDLM is configured thru a text file, the USBDLM.INI. The USBDLM.INI is read from the folder of the USBDLM.EXE.
If required, read at Wikipedia how INI files work. 


New default letters for new USB drives (flash or harddrive) 

[DriveLetters]

Letter1=U

Letter2=Y

Letter3=Z

USBDLM can have up to 9 'LetterX' entries in each section. They don't have to be continious. 

Have a look into the help file USBDLM_ENG.CHM for more details. If you get the 'cannot display the webpage' error then the help file has the NTFS file attibute 'downloaded from untrusted source' and Microsoft doesn't trust its own CHM file format. To fix this right click the USBDLM_E.CHM, select Properties and click Unblock.
The help file is available online as HTML version too.


card readers 

The typical 20-in-1 card reader eats one drive letter for each of its slots - if we have a card for or not. USBDLM can remove the reader's drive letter until a media is inserted. This does not work under Windows 2000. 

[Settings]

NoMediaNoLetter=1

USBDLM assings then a drive letter as configured. If you need different drive letters for a multislot cardreader, then use the criterion 'DeviceType' in a DriveLetters section (MSCR is short for MultiSlotCardReaders which can be used too): 

[DriveLetters1]

DeviceType=MSCR

Letter1=R

Letter2=W

Letter3=


Excluded drive letters 

You can prevent Windows mounting to certain letters by putting them into section ExcludedLetters. This is especially useful for letters of network shares which Windows may assign to USB drives while booting. Do not configure other letters than network, subst and TrueCrypt drives here! 

[ExcludedLetters]

Letter1=F

Letter2=

Letter3=


Balloon Tips 

USBDLM can show a balloon tip with information about the drive letter(s) or mount points used for a just attached drive: 

[BalloonTips]

Enabled=1

[image: image91.png]Corsair Flash Voyager USB Device [X|

mourtedta Ui\





	


	Standaard kunnen netwerk drives op de drive letter j, g en h aanwezig zijn, om conflicten met usb-sticks te vorkomen (of op te lossen) wordt de usb manager geïnstalleerd.


	[image: image92.jpg]=lolx|

Fle Edt Vew Fawortes Tods Help

| &

Qe - O - B | O search [ rokders |5 3 X 9 | [+

sz [ cawanoons

L

Name - See [ Trpe Date Hodfied aubutes
e Folder 12122007 025
e Folder 11122007 23043 |
e Folder 16122007 0:39
e Folder 12122007 05
e Folder ramas R
e ool b |





	Ga naar de USBDLM directory in de extra installatie map. Kopieer de volledge map en voeg deze toe in de directory: C:\Windows. Open de map en volg de volgende instructies:

	[image: image93.jpg]Bl Edt Formet ew Hep

EGEDITA

[HKEY_LOCAL_MACHINE\SOFTWARE\Uwe

[HKEY_LOCAL_MACHINE\SOFTWARE\Uwe
“NomeiiaNoLatter ="1

[HKEY_LOCAL_MACHINE\SOFTWARE\Uwe

[onarrival]
open-explorer %rootx

[8a]ToonTips]
Enabled-1

[

sieber\usepLm\or iveLetters]

sieber\usebLM\ExcTudedLetters]

sieber\usepLisettings]

sieber\usepLm\Ballaons]





	De instellingen voor de USB manager worden in het register van Windows opgeslagen, met het bestand ‘usbdlm.reg’ kunnen deze instellingen direct in het register geladen worden.



	[image: image94.jpg]~=lolx|

Ele Edt View Favortes Toos Help

| &

Qe - O - B | O search [ rokders |5 3 X 9 | [+

sz [ cammpowsssoL

EBE

s Sz [Tyee Dt Hedied Aives
] rstolend K6 Windows Commend . 25102005 1649 A
Sl strtcnd 1K6 Windows Command . 722007 1448 A
lsop.cnd 1K6 Windows Commend . 1422007 1425 A
Bl nisaband 1K6 Windows Command . 251020051958 A
Blsores o 4@ Appcation wemmms A
Sltistusborives_To_Notepad.cnd 16 Windows Commend . 2032007957 A
Slistusborives_To_Ntepad_d 1K6 Windows Commend . 2032007957 A
Museour 6 emetshotest | 112200705 A
Mussouroe 15210 aplcation w0 a A
16 Regsrtonrres 182200709 A
UsgoLo 26 Tetbeamen | iozwizes A
st eng.cim 46KE Compled VL H... 25102007 1238 A
3150l _ger.chm S3KG Compled VL b 2510207 217 A
USBOLIM Lience bt 6 Tecbeament | iStozwini A
USEOLM Lo Si6 Todboament  iel02007szs A
SO sampli 26 Corfigratinetings 3620071875 A





	Importeer de register instellingen door dubbel te klikken op het reg-bestand.

	[image: image95.jpg]\"/ Are you s you wank t0 add the formation in CAWINDOWS|USBDLI.sbeln reg o the registry?





	Na dubbel klikken op her eg bestand wordt gevraagd of de import uitgevoerd dient te worden, kies ‘Yes’.

	[image: image96.jpg]1) Ioformation in CAWINDOWS|USEDLM|usbdimreg has been successfull entered ino the recistry.





	Nadat de instellingen in het register opgeslagen zijn, wordt dit gemeld, bevestig met ‘OK’.

	[image: image97.jpg]~=lolx|

Ele Edt View Favortes Toos Help

| &

Qe - O - B | O search [ rokders |5 3 X 9 | [+

sz [ cammpowsssoL

EBE

o Sz [Tyee Dt Hedied Aives
K6 Windows Commend . 25102005 1649 A
[ strtcnd 1K6 Windows Command . 722007 1448 A
lsop.cnd 1K6 Windows Commend . 1422007 1425 A
Bl nisaband 1K6 Windows Command . 251020051958 A
Blsores o 4@ Appcation wemmms A
Sltistusborives_To_Notepad.cnd 16 Windows Commend . 2032007957 A
Slistusborives_To_Ntepad_d 1K6 Windows Commend . 2032007957 A
Museoun 6 emetshotest | 112200705 A
Mussoiroe 15210 _applcaton w0 a A
Blusbdmarea Ut dmve et e O |Erts loizz0nrose A
2] USBDLM.bxt. http:/fuwwn. uwe-sieber dejusbdim_e.html Lot 29-10-2007 12:25 A
st eng.cim TR Corpleg I He... 2502007 3% A
3150l _ger.chm S3G Compled VL He. 25102007 217 A
USBOLIM Lience bt 6 Tecbeament | iStozwini A
USEOLM Lo Si6 Todboament  iel02007szs A
SO sampli 26 Corfigratinetings 3620071875 A





	Nu dient nog de service USBDLM geïnstalleerd te worden, dit wordt gedaan door het bestand ‘_install.cmd’ uit te voeren

	[image: image98.jpg]1)  Service installed and started





	Na installatie ven de service wordt dit gemeld, bevestig en alles is gereed.


7.6 Installatie 7-Zip 
	Start de installatie van 7-Zip vanaf de extra CD of ‘Installatie Diversen’ directorie:

Dubbelklik 7z432.exe

	[image: image99.jpg]1ol

Choose Install Location .
Crose the s i ol 25 4.3 (%)

Setup wilnstall7-2p 4.2 n the Following folder. To install in a dferent Folder, cick Browse
‘and select anather Foder. Click Install to tart the nstalaton

Destination Folder

Fee

Space required: 2,618
Space avaable: 11,968

w72 e

=






	Accepteer de in stallatie directorie.

Klik op “Install”

	[image: image100.jpg]Completing the 7-Zip 4.32 Setup
wizard

7-21p 4.32 has been installe on your computer,

Click Finsh ta close this wizard,






	Installatie is voltooid, afsluiten met “Finish”.

Indien een reboot gevraagd wordt, deze niet uitvoeren. Pas na installatie handmatig een reboot uitvoeren!

	[image: image101.jpg]emrzprieransger e < 1

Bestond Bowerken Beeld Favoristen Gereedschap Help

F = v = x i

Tosvoegen Utpakken  Testen ‘ Kopiéren  Verplastsen Verwidersn Info

[t object(en) geselecteerd 0B 08





	Start 7-zip op vanaf “Start ( All Programs ( 7-Zip ( 7-Zip File Manager”

Vervolgens dienen de extensie voorkeuren ingesteld te worden, ga naar “Gereedschap ( Opties”.

Selecteer allen in het opties scherm.

Instellingen opslaan en aflsuiten.



	[image: image102.jpg]21
oores

=
Systen | Pugins| Edo | nsteingen] Toal |

Assosiest 7Zip mek:

=
o 7Zip.
=
=
=
=
=
=
=
=
=
=
=
=
=

Als selesieien

“
Cancel Aeply Help
o |_cs





	


7.7 Installatie diverse programma’s

De volgende programma’s worden ter ondersteuning geinstalleerd, vooral deze programma’s kan de installatie routine als standaard worden doorlopen:

· Adobe Acrobat Reader

· Notepad++

· Microsoft Word-viewer

· Microsoft Excel-viewer

· Microsfot Visio-viewer

· Winmerge

· Infrarecorder

· Clamwin

· Windbf

· Autoit

7.8 Installatie SQL Sever 2000

· Installatie van SQL Server moet uitgevoerd worden voor de installatie van FactoryLink 7.5.

· Installeer voor een server met Windows Server 2003 de versie SQL Server 2000 Standaard edition.

· Installeer voor een pc met Windows XP de versie SQL server 2000 Personal edition. (Installatie procedure is niet opgenomen in dit document).

· Beide CD worden door Tecnomatix geleverd bij de Factorylink CD versie 7.xx.

7.8.1 Installatie SQL Server 2000

	Installatie procedure "SQL Server 2000" 



	Let op: Er zijn twee CD’s aanwezig in de aangeleverde verpakking van Tecnomatix.

Gebruik voor Windows 2003 uitsluitend de ‘ Standaard’ versie.



	Standard edition (voor installatie op Window Server 2003 OS)

Start installatie op door middel van de CD in de Cdrom drive te plaatsen.

Indien dit niet functioneerd Start . Run.. > Browse > 

navigeer naar de Cdrom drive en dubbelklik op autorun.exe.



	[image: image103.png]€S 5L Server 2000 Compernents

€S 50L server 2000 prereguisies

& Browse Setup/Upgrade Help

@ Read the Release Notes.

Exit





	Opstartscherm van de installatie.

Selekteer ‘ SQL Server 2000 Components’ 

	[image: image104.png]Install Components
M gérver
2000

Standard Edition

8 Install Database Server
SOL Server 2000 provides rich and robust
support for scalable database solutions
Install analysis Services
@ Install English Query

Back Exit





	Selekteer ‘ Install Database Server’ 

	[image: image105.png]SQL Server 2000

QL Server 2000 5p2 and below is ot supported by this version of
Windows. Please apply 5p3 after nstalation of SGL. Server 2000,

To run the program, cick Cantinue. For mare information, cick Detals.

T~ Don't display this message again

ot &=





	Zoals links te zien is, is deze versie van SQL 2000 niet geschikt voor Windows 2003.

Na de standaard installatie moet het meest recente servicepack geinstalleerd worden.

Klik op ‘Continue’.

	[image: image106.png]‘Weloome to the Microsoft SOL Server Instaltion
O Wizard.

The Instalston Wizard alows you o instala new instance of
SGL Server of modiy an existing nstarce.





	Wekomst scherm van SQL 2000

Klik op ‘Next’ 

	[image: image107.png][Computer Name

e the e o e capuder on which ou want o
e s e e of S Server oy o
O i tance of 0L Servr,

(07, et he name of e o exising Vitual S
Sty to manage,

[CEMPINA BAdaaE2

& LosalCampile}

 Bemote Computer

€ it Berve

e

Hep | <Baock [ HNew> Cancel





	Installatie wordt hier lokaal uitgevoerd.

Selekteer ‘ Local Computer’ 

Klik op ‘Next’



	[image: image108.png]Installation Selection

Select one of the following instalton options.

Wpacl, erove, o1 s corporents o an existing
instarce of B0 Gever

 Advanced options

Create aNew Instalation
This option alows youto create a new instance of SOL
Server 2000 orinstal Clert T aok on anp supported
Operating Systen.

Help <Back Next> Cancel





	Selekteer ‘Create a new instance of SQL Server, or install Client Tools’ 

Klik op ‘Next’ 

	[image: image109.png]User Information

Eter your name below. It s nct necessay ta e a company

(LR 011 1 TEFNATIONAL
Compary:  [CAMPINAMELKUNIE

Bk ==





	Vul indien nog niet aanwezig Name en Company in volgens linker scherm.

Klik op ‘Next’ 

	[image: image110.png]Software License Agreement

Fleass read the following License Agreement. Press the PAGE DOWN key to see.
the restof the agresment.

[END-USER LICENSE AGREEMENT :‘
SERVER LICENSE FOR MICROSDFT SOL SERVER PRODUCTS

IMPORTANT-READ CAREFULLY: This Endser License Agreement (ELILA) s a legal
agreement between you (sther an individa o a single entity) and Mistasoft Carporatian
fon the Microsoftsoftware product dentifed above, which includes compter software:
|and may include associsted meda, pinted matersis, and "oriine” or lectioric
(documentation ('Product’) An amendment or addendum to this ELILA may accampany
the Prodhuct. YOU AGREE T0 BE BOUND BY THE TERMS OF THIS ELILA BY
INSTALLING, COPYING, OR OTHERWISE USING THE PRODUCT. IF YDU DO NOT
|AGREE, DO NOT INSTALL DR USE THE PRODUCT: YOU MAY RETURN IT TO
IYOUR PLACE OF PURCHASE FOR A FULL REFUND.

The Product may contain the following soltware:

Do you accept allthe terms of the preceding Lizense Agreement? If you choose No, Setup
wil lose. To nstall Mictosoft SL Server 2000, you must accept this agreement.

<Back. Ho





	Bestudeer de licensie overeenkomst.

Klik hierna op ‘Yes’  

	[image: image111.png]Eter your 25-digt CDKey

mBvez  pesem Juxern {43c37  93TTH|

You canfind tis rumber on the yellow sticke i the CD e notes ot the CD slesve,

Bk ==





	Type de 25 delige CD-key in.

Opmerking: Deze key is voor alle instances binnen DMV t.b.v. FactoryLink hetzelfde.

Klik op ‘Next’ 

	[image: image112.png]Yo can select ane ofthe fallawing ypes o instalatons.

 Clent Took Only

oA TR

© Comectiviy Orly

This option alows you o instal a server and the cent
tools. Use this opton i you wa to set up a server with
adhnirislraton capabilties.

Help <Back Next> Cancel





	Selekteer ‘Server and Client Tools’ 

De optie ‘Tools’ is benodigd om de database te kunnen configureren,  programmeren en onderhouden.

Klik op ‘Next’

	[image: image113.png]Dot

For a default installaton, leave Defauk checked and
clck Next.

Toinstal o mairtain a named instance of SOL
Server on this computer cleat the Default checkbor
and type o select an nstarce rae.

A new name must be 16 characters ot less and
should tat ith a eteror cther acceptable
character. For mere iformaton, cick Help.

Istance nane:

e —

Hep | <Baock [ HNew> cancel |





	Vink ‘Default’ aan indien dat nog niet zo is en klik op ‘Next’

	[image: image114.png]Setup Type

Click the type of Setup you prefer then click Next.

Installed with the most comon optons. Recommended for most users

© e
T e e
~ Cusom Youmaychoos the optons o wer o sl ecommeded o

advanced users

Destinaton Folder

Frogram Files Bowse. C\Progiam FlestMicrosolt SOL Server
Data Fils Tm C\Progiam FlestMicrosolt SOL Server

Requred: Avalabl:
205K 1386700K
18917K 13BE00K
432K 13856700K.

Help <Back Next> Cancel

Space on program fes diive:
Space on system dive:
Space on data fes diive:





	Selekteer ‘Typical’ installatie.

Laat directory verwijzing voor ‘Program file’ en ‘Data files’ op hun default waarde staan.

Klik op ‘Next’

	[image: image115.png] Use the seme account for sach service. Auto sart SO Server Service.

 Customize the seltings for sach service.

Services Senvice Setings

€ 50l Server @ lise e Local Sy accaurt

€ 0L Serverbgent
e e  Use aDomain User account

Password: —
Domain: [CaMPING BASGES

I utg Start Service.

Hep | <Back New> cancel |





	Selekteer zoals op het linkse scherm te zien is. 

Klik op ‘Next’

	[image: image116.png]Choase the authenication mod.

 Windows Authentication Mods

& Mived Mode (Windows Autenticalion and SO Server Autheniication)

Add psswordforthe sa oai

Erter pessuord
Corfim passyard 1

[ Blark Password fnof recommended

Help <Back Next> Cancel





	Selekteer ‘Mixed mode’ en ‘Blank Password’ 

(Benodigd voor Factorylink)

Klik op ‘Next’

	[image: image117.png]Start Copying Files

Setup has enough informalion o promp you for the lcensing
O ‘mods information and start copying the progra fles. Click Back

1o teview of change the settings, oiherise cick Next 1o
proceed.

Bk ==





	De installatie configuratie is klaar.

Volgende stap is het daadwerkelijk installeren van SQL 2000 Server.

Klik op ‘Next’



	[image: image118.png]5L Server 2000 supports two ClientLicensing Modes

g} FerSeatEach deics hat accesss Mool S0L Servr
2000 requires a separate Cient Access License.

[Eig} FrocessorLicense - Eoch processor on i sever eites a
sepatate Processor License.

Pleas click Help to fuly understand the implications of the licensing
mode you choose.

Licensing Mode

 Pereatfor 3 = devices

€ Brocessor License for. = processors

Exit Setup Help





	Vul de licence mode in:
· Per Seat for 3 devices.

· Heeft server 1 processor dan optie ‘Processor Licence’ niet aanvinken.
· Heeft server 2 processoren dan optie ‘Processor Licence’ aanvinken en keuze op 2..
Klik op ‘Continiu’.

De Installatie van SQL server 2000 wordt uitgevoerd.

	[image: image119.png][Setup Complete

Setup hs finished instaling an instance of Mictosoft SO Server

O 200 on your computer.

Cick Firish to complete Setup.





	Scherm wordt getoond na installatie SQL Server 2000.

Klik op ‘Finish’.

	Herstart de server




7.8.2 Installatie Serviceopack 4 SQL Server 2000
	Installatie procedure Servicepack 4 SQL 2000.



	Start servicepack.

Er verschijnt een popup met de vraag waar de uitgepakte file’s moeten worden weggeschreven.

Voor een directory naam ‘C:\SQL2KSP4’ in.

Klik op ‘Next’.

	Open in ‘C:\SQL2KSP4’ -> ‘Setup.bat’.

Nu volgt het opstartscherm van de installatie.

Klik op ‘Next’

	[image: image120.png]Software License Agreement

Fleass read the following License Agreement. Press the PAGE DOWN key to see.
the restof the agresment.

[FERVICE PACK 4 FOR MICROSOFT SGL SERVER 2000 :‘
[SUPPLEMENTAL END USER LICENSE AGREEMENT

[SOFTWARE UPDATE FOR MICROSOFT SOFTWARE

IPLEASE READ THIS SUPPLEMENTAL END-USER LICENSE AGREEMENT
['SUPPLEMENTAL EULA") CAREFULLY. BY INSTALLING DR USING THE

[SOFTWARE THAT ACCOMPANIES THIS SUPPLEMENTAL EULA, YOU AGREE T0

[THE TERMS OF THIS SUPFLEMENTAL EULA. IF YOU DO NDT AGREE, DO NOT
INSTALL DR USE THE SOFTWARE.

1. Genera,

Do you accept allthe terms of the preceding License Agreement? If you choose No, Setup
willlose. To nstall Mictosoft SOL Server 2000 Service Pack 4, you mst acoept this

agieerent
<Back. Ho





	Lees de License agreement aandachtig door.
Klik vervolgens op ‘Yes’.

	[image: image121.png]¥ Dzt

To apply Service Pack to the defaul instance, check
the Default bor and olick Next

To appl Service Pack to a named instance of SOL
Server on this compute, clear the Defaul checkbox
and type o select an nstarce rae.

A new name must be 16 characters ot less and
should tat ith a eteror cther acceptable
character. For mere iformaton, cick Help.

Istance nane:

Hop | <Back [T | cancel |





	Klik op ‘Next’.

	[image: image122.png][Connect to Server

Selectthe authentication mod setup should use to connect

O 1050L Server.

" The SOL Server system adiristiator login informaton
(50 Server authenticaton)

Entersa passior

—

& The Windows accountinformation | use to log onfo my
computer with [Windows autheniicaton)

Bk ==





	Klik op ‘Next’.

	[image: image123.png]5a Password Warning

Service pack selup has detected that your SA password s set o blark. s highly recommended that you set a
stong password and secure your database server, even i you are using Windows authenticalion

©11do not want a blark password, set password

Erter 54 Password:
Confim SA Password:

 Tgrore the seculy ieal Waining, leava the passwoid Bark





	Vink ‘Ignore security warning, leave the password blank’ aan.

Klik op ‘OK’.

	[image: image124.png]SQL Server 2000 Service Pack 4 Setup.

Backward Compatbiy Checkist

Vet the follwing SP4 secuity enhancements. Click Help for more iformation.

Cross database ownership chaining i tumed off by defaut in SPA. Afer instalaion, you can enable
Gross database awneiship chairing for individual databases. I you must use crass-detabase ownership
chairing, select the check bor below.

I™ Enable cross-database ownership chaining for il dtabases (ot recommended)

P4 wil upgrad Mictosaft Search service and automalicaly rebuld all fulltst catalogs or all
applcations using the service. Duing the rebuld, futext functonalty may ot be fuly avaisble. Select
the check ba below ta conlinue upgrading to SP4.

¥ Upgrade Mictosolt Search and apply STL Server 2000 P4 [requied).

Click Cancel to exit Setup without instaling SP4.

Help | Coniinue | Concel





	Vink ‘Upgrade Microsoft Search and apply SQL 2000 SP4 (required)’ aan.

Klik op ‘Continue’.

	[image: image125.png]1fyou enable ST Server's eror reporting feature, SEL Server will b configured to
automaticaly send fata service eror report to Mictosoft or a Cotporate Erior Reporting server

1fa fotal etor occurs,the server sends nformation bout the etor o Mictosoft over &
secure connection, where it stored with mited access, orto  Corporate Ertor Repoting

To setup a Corporate Ertor Reporting server o cortrolthe informalion sent o Miosolt see.
hig://ocamictosoftcom/ceiiiro.asp. Miciosolt uses the reports only o improve STL Server,
and reats al irfomalion s confidental,

For more informaton on the eror reporting festure and the type of information sent, cick Help.

™ Automaticaly sen fetal efo repots to Microsoit

Help K





	Desekteer vinkje voor ‘Automaticaly send fatal error reports to Microsoft’.
Klik op ‘OK’.

	[image: image126.png]Start Copying Files

Selup has encugh informalion o start copying the program fes.
1fyou want o review of changs any setings, cick Back. If you
ate satisfied with the settings, cick Next o begin copying fes.

Bk ==





	Setup heeft nu genoeg informatie om de installatie te kunnen starten.
Klik op ‘Next’.

	[image: image127.png]1) You should now backup your master and msdb databases since this nstalation has pdated their content,





	Klik op ‘OK’. 

	[image: image128.png][Setup Complete

Setup hasfinished instaling an instance of Mictosoft SO Server

O 2000 Service Pack 4 on you computer

Cick Firish to complete Setup.





	De installatie is klaar.

Klik op ‘Finish’.

	Herstart de server.




7.8.3 FactoryLink SQL Database

	Indien nog niet aanwezig maak een database aan.


	[image: image129.png]0 prise Manager - [Console Root\Microsoft SOL S [=1 o3|

B Hle Acion Vew Ioos Window Help [ISETE|
e a@EXERR[2[% (N0 @B
3 Consol oot [5aLserver Goup_iTtom

Mirosoft sQL Servers
=g
&

(LOCAL) (Windows NT)

ocay)
{2 Databases (Windows NT)

{21 Data Transformation Set
{23 Management
{2 Replication

{2 security
{0 support Services
Mets Data Services.





	Clik met rechtermuisknop op de directory ‘Databases’ en selekteer ‘New Database…’.



	[image: image130.png]Database Properties.

Generl| Dat Fies | TinsacionLog|

I

Database
Status: WUrknown)
Qe WUrknown)
Date createdt (Urknown)
Size: WUrknown)
Space availble: (Urknown)
Number of users: WUrknown)

Backup.

Last database backup: None
Lastransaction lag backup: None

Maitenance

Maintenance plar: None

Collaion name: (Server defaul)

=





	Ul naam van database in volgens het volgende format:

DB_<projectnaam>_S<serverno>.

Klik op de tablad “Data Files”

	[image: image131.jpg]Database Properties -

Generel Dat Fiet | Tinsacion Loo|

Database fies
Fie Name Locaion Tl 522 (V5] Fiegoup
Fie propeties
% Automatcaly g e
Fie gronth Masimum e size-
" Inmegabytes: [ J @ Unresticted file growth
& By percent 2| € Bestict e giowt =
Bype t 10 = Bestrict fle growth B} 2 Fl

Concsl | Heb





	Klik hier op de knop met … erop bij location om de locatie waar de database wordt opgeslagen te kiezen.

Hierna opent het volgende venster

	[image: image132.jpg]Locate Database Fi

{0 CA(11139 MB free)
-3 DA (15665 M8 free)
{2 captures
{3 dblog2
@an
0 L data
{20 Installtis Diversen
{2 RECYCLER
/23 SAL server
{22 System Volume Information

5

Fie name: o 5]

Selected fl: DASQL server\DB_SSSC_S1

Cance





	Selecteer de map “d:\SQL server” en typ bij “file name” de naam van de database. DB_<projectnaam>_S<serverno>.

Klik dan op “ok” en klik op tabblad “Transaction Log”



	[image: image133.jpg]Database Properties -

Generl] DataFies TinsacionLog |

ATransaction log fles

Fil Name.

Location

)

Irifal size MB]

Delete

File proptiss
¥ Automaticaly row file

File growth

© Inmegabytes:

By percent

Masinum e size

& Unvesticted fle grawth

€ Beniafogonn e} [T

o Cesl |t





	Klik hier op de knop met … bij location.

Het volgende venster opent

	[image: image134.jpg]Locate Transaction Log Fi

{0 CA(11139 MB free)
-3 DA (15665 M8 free)
{2 captures
{3 dblog2
@an
0 L data
{20 Installtis Diversen
{2 RECYCLER
/23 SAL server
{22 System Volume Information

Fie name: D6 _555C_S11

Selected fl: DASQL server\DB_SSSC_S1

Cance





	Selecteer de map “d:\SQL server” en typ bij “file name” de naam van de database. DB_<projectnaam>_S<serverno>.

Klik dan op “Ok”



	[image: image135.jpg]Database Properties.

Generl] DataFies TinsacionLog |

ATransaction log fles

Fil Name.

Location

Irifal size MB]

Delete

File proptiss
¥ Automaticaly row file

File growth

© Inmegabytes:

By percent

Masinum e size

& Unvesticted fle grawth

€ Beniafogonn e} [T

o Cesl |t





	Klik “ok”

	[image: image136.png]=10l x|
“%) Bl Action Yew Took Window Help | =to1x]
e a@EARBR|*(NLIBRE
T oot Databases_1Tiem
= Hiroset s servers

2 20 sorver o ]

6 (LocAL (WindowsT)

& bt
IS ——
23 Horsgemert
23 Repcstion
Q seaurty
23 supportservies
Heta b Sevices

BB 5 51





	Verwijder niet ter zake doende databasis. (Nortwind, Pubs etc..).

Klik met rechtermuisknop op (LOCAL)(Windows NT) directory en selekteer ‘Edit SQL Server registration propertys…’.

	[image: image137.png][Registered SQL Server Prope:

Gerera |
L e i (|

& Use Windows authentication

 Use S0 Server authentication

LognMame: |
Passwod [

I Alisays prompt for login name and passiord)

Options

O 1= Eerra (|

97 Display SOL Server statein console
¥ (Sholtsplen dabases snd s/ obec

IV Automatically start SUL Server when connecting

=





	Haal vinkje weg voor optie ‘Show system databases and system objects’.



	[image: image138.png][Registered SQL Server Prope:

Gerera |
L e i (|

& Use Windows authentication

 Use S0 Server authentication

LognMame: |
Passwod [

I Alisays prompt for login name and passiord)

Options

O 1= Eerra (|

97 Display SOL Server statein console
I (Sholt e dabases and s/ obec

IV Automatically start SUL Server when connecting

=





	Klik op ‘OK’.

	[image: image139.png]=lolx|

BD & o ool

[ISETE

IR

B @k

NG, 0 @3

T3 Comele oot

5 mrosot saL ervers

4@ 5. server Gop
&

Databases

{2 Management

{2 Replication

{2 security

{0 support Services
Mets Data Services.

{1 Data Transformation Set

[ (LocaL) (windows NT) 6 Ttems
Name

Data Transformation Services
Management.

Security
Support Services

Meta Data Services





	Klik met rechtermuisknop op ‘(LOCAL)(Windowss NT)’.

	[image: image140.png]SqL Server Propert

Seversetings | DatsbaseSeivgs | Repiosion |
Gered | Memoy || Fiocessor | Secuy | Comectons |
@ Name: (local]

Product SOL Server Stanlad Edion

Operating system: Microsoft Windows NT - 5.2 (3730)
Product version: 8.00.2033 (SP4)

Language: English (United States)

Pt NI INTEL XS

0S memory: 1023(MB)

- 2

e C\Progan FlestWicosot SO
L SOL.Lai_Genee_CF1_CLAS

Aulostat poices when the operaing system sts
@] [ Audesiar SOl Seive
T ¥ AuostatMspTe

Statup Parameters |

Network Configuaton. |

Cancsl [ |_Hes





	Selekteer vinkje voor ‘Autostart SQL Server Agent’.

Selekteer tabblad ‘Memory’

	[image: image141.png]SQL Server Properties (Configure)
e [ s [ e |
Gered | Memoy | Fiocessor | Secuy | Comectons |
P

5L Server can be corfigured t use afiked smount of
memory orto dynamicaly adjus the amourt of memory used
based upon demand.

@ Binamically configurs SOL Server memorni

M (MB): 0

M8 | 10B
Masimum (ME]: 1023

4MB

| 10z3me

© Useafiredmemoy sz (B 511

——— izmiE

I Reserve physical memory for SOL Server

Minimum query memory (KB} Zs

& Configured values. © Runring valies

TBHE.

=





	Het nevenstaande scherm wordt geopend.

	[image: image142.png]SqL Server Propert

e [ s [ e |
Gered | Memoy | Fiocessor | Secuy | Comectons |

~Memory

5L Server can be corfigured t use afiked smount of
memory orto dynamicaly adjus the amourt of memory used
based upon demand.

" Dynanically configurs ST Server memory

Hiimum (48] 0
o o
iimum (6] 1023
4148 | oz
@ Use afiredmemoy sze (B 201

16 ME] —A— 1n23me

I Reserve physical memory for SOL Server

Minimum query memory (KB} Zs

& Configured values. © Runring valies

=





	Selekteer optie ‘Use a fixed memory size (MB)’ van maximaal 201MB.

Dit is het maximale geheugen wat SQL alocceerd voor uitvoering van zijn taken. Zodoende blijft er nog ruimschoots voldoende geheugen over voor andere applicatities zoals FactoryLink.



	[image: image143.jpg]SqL Server Propert

ers [ vemwnr [ e
Gononl | Memoy  Poossar | Secuty | Comecians

-Pracessar corlrol

In a symmetric muli processor (SMP) environment, specify
@ which processors SQL Server wil use

[TFossar =
[

v CPU 1-PROCESSOR_INTEL_PENTIUM

Mavinun worker teads: &3

T~ Boost SL Server priorty on Windows
I~ Use Windows NT fbers

Padlelsm—————————————————————————
@ Speciy the numbe of processors o use fo parallel execuion of

queres:

 Lse all available processors

& Use 1 ~] processarts)

i ey pon sl corsdoa 5=
Queries for paralel execulion (costestimate]

 Configured values. © Runring valies

=





	Selekteer optie ‘Use 1 processor’.

Maak eveneens een keuze welke processor gebruikt mag worden door SQL server. Door slecht 1 processor te gebruiken blijft voor andere programmas (FactoryLink) altijd voldoende processor ruimte over.

Klik ‘OK”.


	Maak een nieuwe gebruiker aan voor SQL-server, zie onderstaande figuur, het menu wordt opgeroepen met de rechtermuistoets.

	[image: image144.png]=10] x|

B e fcton Uow Ioks windom te

JISETE

c-BmFRR|@E| sk

NG, 0 @3

Somere 0
5 mrosot s ervers

@ oe_toici st
0 e
2 Dats Transformaton 521
2 pansgement
23 Repication
= secuty
€ Loans
Server Roles
Unked servers
5 Remote Sarvers
22 Support servies
ks Dats Services

| — (T

Refresh
vew »

Ling up Teons

Export List,

Arrange Lcons >

Help

R Tipe Sorve Accoss Defout Database | DofoutLanguage
5@ 50 server wrop
28 (L0CA) (Windows T) BUILTINAdminstrators Windows Gre. Permit mastar Engish
B Db s Standrd  Pemk master Engih





Sluit ‘Enterprice Manager’ af.
	

	[image: image145.jpg][SQL Server Login Properties - New Logi

General | Server Roles | Database Access |

Qe |

Authertication

" Windows Authentication
Doman [—
Secuiy acces
45 Grant acoess:

€ Denyaccess

& S0L Sever Autheniication

Password

Defauits
Speciy the default language and database fo this login.

Database: DB_TOCT 51 <
Longuage: &

Concsl | Hes

—





	Maak de gebruiker ‘flink’ aan zonder wachtwoord. Let op het veld met password blijft leeg, echter bij openen worden altijd ****** getoond.

	[image: image146.jpg]SqL Server Login Properti 0

il |

SewerFoks —————————
‘Server oles are used o grant server-wide secuity pivieges o @
fogin.

Server Foke =

I71%5 Secury Admiistetors
I1% Server Adriistators
171 setup Adninistators
I % Pracess Adrmiistators -
I71% Disk Adrinistators
I71% Database Creators

-Desciption

[Can perfom any sctvy n the STIL Server installtion





	Stel de rol in voor de SQL gebruiker.

	[image: image147.jpg]SqL Server Login Properti 0

Genenl] Seve les Databaseicces |

ﬁ ‘Speciy which databases can be accessed by this ogin

[Femit_|Database [User 2
[7]

OF master
O model
OB msb
OF tempdy
OB st

Database rols for DB_TOTCT ST

[Permitin Database Foke &

£ public

'€ db_accessadmin
I db_secuiyacin
IC & db dlachmin

Propetes

Concsl | Hes





	Leg vast welke database door de gebruiker benaderd kan worden, dit is alleen de FactoryLink database. Vul ook de links aangegeven rechten in.

	[image: image148.jpg]0 passior

Confim new passworc:

¥ ke ot onmere]





	Na bevestigen van de gegevens voor de nieuwe gebruiker, dient het wachtwoord bevestigd te worden, of zoals hier wordt aangegeven dat er geen wachtwoord wordt gebruikt.

	Stel de eigenschappen van de database in:

	[image: image149.jpg][DB_T01C1_51 Properties

Gereral | DtaFies | Tronsacton Lo Fiegoups | Optons | Permisios |

i Meme:  DB_TOICT 81

Database.
Status: Normal
Ourer s
Dae created 18122007 215725
Size: 334.25M8
Space availble: 0.00MB
Number of users: 3
Backup -
Last detabase backup None
Lastransaction lag backup: None
Matenance
Maintenance plar: None
Collaion name: 50L_Latin1_General CF1_C1_AS

Cancsl | Hes





	Klik met rechtermuisknop op (LOCAL)(Windows NT)/Databases/ DB_<projectnaam>_S<serverno> directory en selekteer ‘Properties’.

De eigenschappen van de database worden geopend. Neem de instellingen over zoals deze in de linker figuren zijn weergegeven.

	[image: image150.jpg][DB_T01C1_51 Properties

o | el [ [ (e

Database fies

Fil Name. Location Space alocated (MB]_Fiegroup.
DB_TO1C1_81 Data
[ ]

Delete

File proptiss
¥ Automaticaly row file
File growth Masium e size

" Inmegabytes: & Unvesticted fle grawth

© Bestiotflegowh B} [

By percent

Cancsl | Hes





	

	[image: image151.jpg][DB_T01C1_51 Properties

Genea | DataFies  Tratsaction Log | iegoupe | Opins | Pemisos |

ATransaction log fles

Fil Name.

Location

DB_TO1C1_51_Log

[ ]

Space alosated (ME]

Delete

File proptiss
¥ Automaticaly row file

File growth

© Inmegabytes:

By percent

Masinum e size

& Unvesticted fle grawth

€ Beniafogonn e} [T

Cancsl | Hes





	

	[image: image152.jpg][DB_T01C1_51 Properties

Geneal| DataFies | TransactonLog. Flegoups | Optons | Pemissios |

Fiegroups

Name.

PRIMARY

Fies

Fead Only Default

Cance

Help





	

	[image: image153.jpg]o i oo ] e o

Access

T~ Restict access

| embers of db_owrier, dosieato, o1 sysadrin

£ Single user

I~ Readonly
o

i
S

™ AN NULL defouk I Augciss

I Recusiv tiggers ¥ Ao shik

¥ Aot vpdte st ¥ futo rate st
¥ Tanpage deectin ™ Use ueted s

T llow cross-database ownership chaining

Compatity

Level Database compatbily evel 50

Cance

Help





	Aandachtspunten:

Stel het recoverymodel in op ‘simple’, in verband met gebruik van schijfruimte.

Zorg ervoor dat de optie ‘Auto shrink’ actief is, deze zorgt ervoor dat SQL –server voor deze database vrije ruimte hergebruikt.

	[image: image154.jpg][DB_T01C1_51 Properties

Gereral DataFies | Tronsacton Lo Fiegoups| Optons Permisos |

[Userficle [Cieate Table [CreateView |Creats SP_|Create Defauk | Cioat Ale [ Creat
b

€ ik

€ O o o o 8]

Cancsl | Hes





	Neem de rechten voor gebruiker ‘flink’ over zoals aangegeven.

Sla de instellingen op met de ‘OK’ toets.


7.8.4 ODBC Koppeling SQL Database

	Voor FactoryLink wordt een (systeem) ODBC koppeling naar de FactoryLink SQL database aangemaakt.


	[image: image155.jpg][Microsoft SQL Server DSN Configuration

This wizard wil help you create an DDBC deta source that you can use to
connect to ST Server.

What name do you want {0 use 1o refer 0 the deta source?

Neme: [EEESIGE]

How do you want o descibe the data source?

Dessiptior:

Which STL Server do you want to comnect to?

Server: [local |

== |





	Open ‘Start ( Administrative tools ( Data Sources (ODBC)’. Selecteer de tab ‘System DSN’ en voeg een nieuwe koppeling van het type ‘SQL server’ toe.

Stel vervolgens in, zoals in de volgende figuren is aangeven. De naam van de koppeling is als volgt opgebouwd: odbc<projectnaam>.

Let op: geen server nummer, zodat applicatie voor master en slave gelijk blijft!

	[image: image156.jpg][Microsoft SQL Server DSN Configuration

How shoud SOL Server veiiy the authentcity of the login D7

€ WithWindaws NT authertication using the netork agi .

= Wil ST Seiver aLthericaion s & 1agi D and passierd]
entered by the .

To change the network bray used to commricate with SOL Server,

el Clert Carfiguraton
Clent Configuation

Connect to S Server o obtain defaul settings for the.
ddiional configuration oplions.

Login D: [fink

Password

<ok == |





	Gebruik SQL authenticatie voor gebruikers.

	[image: image157.jpg][Microsoft SQL Server DSN Configuration

7 {Change the defail daabase o]
DB_T01C1 51 =
I™ Atach databse fiename:

et emporay stored procedites for prepered GG staerents
P2 o the tored procedires
@Gl e you disconect

e you discorect and =5 sppopiate e you
Cenected.

IV Use ANSI quoted ideniers.
IV Use ANSI nulls, paddings and warrings.

17 Use e oo S Syt iy S Sy o

<ok == |





	Kies de database  voor FactoryLink als de standaard database.

	[image: image158.jpg][Microsoft SQL Server DSN Configuration

I g e isae of S Server e oessages )

Engish

Use stiong encryption for data

Perform anslaton fr character data

Use regional setings when outpuling currency, numbers, dates and
fimes.

LR B<N

Save longrunring queres o the log e
[CADOCUME - T\ADMIN T\ OCALS T\ Temp @0 Browse
Long aueny time (milseconds]: [T777

T~ Log ODBC driver statistcs to the log file:
[CADOCUME-TVADMIN

<ok == |





	

	[image: image159.jpg]0DBC Microsoft SQL Server Setup

A new DDBC data saurce wil be created with the follwing
confiuaiion:

Fitost SaL Server GDBE v Verson 1366 3958 |

Data Source Name: odbeTOICT
Data Source Desciption

Server: local)

Database: DB_TOICT_S1

Language: [Défaul)

Translale Character Data; Yes

Log Long Rurning Queres: No

Log Diiver Stalistcs: No

Use Integrated Secuy: No

Use Regonal Setings: No

Prepared Statements Optior: Diop temporay procedures on
discannect

Use Fallover Server No

Use ANS Duoted Ideniers: Yes

Use ANSI Nul, Paddings and Warrings: Yes

[Data Encypion: No

Tes{Data Souce. ok | _cancel |





	Test de verbinding met de database!

	[image: image160.jpg]"0DBC Data Source Administrator

User DN SysemDSH | i 05N | e | Trecing | Cornestion Posing | About

System Data Sources:

2|

Narne. Diiver Add
NFOcon Wictoso Access Diver )

LocaServer SO Server

SOL Server

TECNOMATIXNS  Micrasolt Access Difver (-mdb)

USDATANS  icrasft Access Diiver (- mdb)

the indicated data provider. A System data source is visble o al users

An DDBC System dta source stores information about how o connect to
on this machine, including NT services.

oK Cancel

Help





	ODBC koppeling is nu gereed


Installatie FactoryLink 7.5.1. 

	Open Start > Run..

	[image: image161.png]N 21X

Type the name of a program, older, docurent, or
Internet resource, and Windows wil open t For you.

e

coce | o





	Vul REGEDIT32.exe.

Klik op “OK”

	[image: image162.png]=1BIx|
Fle Edt Vew Favortes Hep
0 symesto Name [Type [Data
& symesxx (@8] (pefaul) REG_SZ (value not set)
2 symmpi [ab]DataBasePath REG_EXPAND_SZ % SystemRactas|Systen
g ;vsaumi DeadGWDetectDefau\t REG_DWORD. 0:00000001 (1)
yamarlog Joemain REG 52
=it [EJoontadioefatGatewsypefauk REG_DWORD ox0000000 ()
=0 T [EBJEnsbletchPRedrect REG_DWORD axo0000001 (1)
S nalesecutyiters REG DWoRD oo 0
e [ —— ReG_DwoRD oxcannnnnn o)
20 Mdopters EBcsname recsz copinsbatSats
O o eorechdators [ PenabiRoster REG_DWORD 0100000000 ()
=fim [BJiameserver Rec_5z
) Podstenthutes [ Hostname [y campina basos3
= | Brerens REG_MULTLSZ 14351434 3403340
{2 performance [28]searchist REG 52
O securty [H]UseDomartianeDevoltion  REG_DWORD oxoo000001 (1)
20 ServiceProvider REG_DWORD ox0000001 (1)
{2 ToPIPE

o

My Computer{HKEY_LOCAL MACHINE|SYSTEM|CLrrentControlSet|ServicestTcpipiParameters

]
P





	Het registry editer scherm wordt geopend.

Navigeer naar: HKEY_LOCAL_MACHINE\System\

CurrentControlSet\Services\Tcpip\

Parameters.

Rechtermuisknop op rechterscherm.

Kies ‘Add new Dword value’.

Name = DisableDHCPMediaSense

Value = 1 (Decimal)
Sluit programma af.

	Run de FactoryLink installatie CD. (\Autorun\Autorun.exe)



	[image: image163.png]6 Macromedia Flash Player 7

FactorylLink 7.5 o

Tocnomatix

ENGLISH

Eeocis | B RN | e oaiscH

INSTALLATION

Gick erefoinsalyour Fadorylink Syser,
e, anel Opins.

RELEASE NOTES
Clck bere o rece e s Releoss Nokes:
for Fodoryink.

INSTALLATION GUIDE
Click bore o view i guic You must
hove Ade Reodes intlled

INSTALLATION

Welcome fo Factorytink. This instllafion progrom
‘guides you through installing your Factorytink System,
Client, and Options. Beforo installing your Factorylink
System, we strongly recommend that you read the
Releas Notes.





	Taalkeuze is ‘English’
Klik op ‘INSTALLATION’

	[image: image164.png]Welcome to the InstallShield Wizard for Tecnomatis FactoryLink 7.5

The InstalShild® Wizard il nstal Tecromlis FactoryLink 7.5 on your computer. To continue,
clck Next.





	Het welkomstscherm opent.

Klik op ‘Next’.

	[image: image165.png]Tecnomatix FactoryLink

[TECNOMATIX TECHNDLDGIES, INC. (Tecnomati)
FactonyLink Scware Agreement

IF YOU 'LICENSEE") DD NOT AGREE WITH THE TERMS AND CONDITIONS
[CONTAINED IN THIS AGREEMENT, FROMPTLY RETURN THE SDFTWARE PRODUCT.
T0 WHICH THIS AGREEMENT APPLIES FOR A REFUND OF THE LICENSE FEE PAID BY
LICENSEE FOR THE SOFTWARE PRODLICT.

LICENSEE SHALL EVIDENCE TS AGREEMENT T0 THE TERMS AND CONDITIONS OF
[THIS AGREEMENT BY (&) A PROCESS OR FROCEDURE DURING DRDERING.
IDOWNLOADING OR INSTALLATION OF THE SOFTWARE PRODUICT, (B) OPENING THE
SEALED PACKAGE CONTAINING THE CD ROM AND/OR OTHER MEDIA ON WHICH

THE SOFTWARE PRODLCT IS RECORDED, (C) USING THE SOFTWARE PRODUCT. OR
(DJENTERING INTD A SEPARATE WRITTEN AGREEMENT APPROVED BY

TECNOMATIX. THE SOFTWARE PRODUCT IS PROTECTED BY COPYRIGHT LAWS

|AND INTERNATIONAL COPYRIGHT TREATIES, AS WELL AS OTHER INTELLECTUAL
[PROPERTY LAWS AND TREATIES. THE SOFTWARE PRODLICT IS LICENSED (NOT
S0LD) T0 LICENSEE PURSUANT T0 THIS AGREEMENT. THE SOFTWARE PRODUCT. |





	Lees de license agreement  en klik op ‘Yes’

	[image: image166.png]Click the type of setup you preer

9 FactoLink Syste

Select s option t instal typical cei/server components. Recommended for
most server or slandalone instals.

# FactoLink Dlert

‘ Select which program features you want installed. Recommended for advanced

|—Deshnahan Folder





	Maak keuze ‘FactoryLink System’
Klik op ‘Next’.

	[image: image167.png]Click the type of Setup you prefer then click Next.

# Typical Development Program s installed with all components and is configured as
=  development sstem/client. Recommended for most
developrent nsallations.

® Typical Bunime Frogram s installed with ll components and s corfigured
s aunfime system/ciet, Fecommended for most
untime instalators.

o Only selected components areinstaled and conligured as
a development system/cent. Orly or use by advanced
® Custom Rurine Orip selected components are installed and configured as

e system/client. Driy for use by advanced users





	Maak keuze ‘Custom Development’

Klik op ‘Next’.

	[image: image168.png]Tecnomatix Factorylink 7.5 - InstallShield Wizard |

Select Features
Selectthe features setup wil insta.

Selectthe features you want 1o instal, and deselect the features you do ot want o nstal

Deseiption————————

Tecnomatix The ECS Graptics remote
—

\

35133 MB of space requied on the C diive
1271580 M8 of space avaiable on the C diive

Bk (=t Cancel





	Vink aan volgens nevenstaand scherm.

Schuif de slider naar beneden.

	[image: image169.jpg]Tecnomatix FactoryLink 7.5 - InstallShield Wizard

Select Features
Selectthe features setup wil instal.

Selectthe features you want 1o instal, and deselect the features you do it want o nstal

Deseiption—————————

Tecnomatix The core ecuied modules of
Clent Buider
EDI Drivers
4PD Di
UKD Di
/BEM Diver

bt
o
oo

405,83 MB of space requited on the C diive
1074345 M8 of space avaiable on the C diive

—————
R e e





	Vink aan volgens nevenstaand scherm.

Klik op ‘Next’.

	[image: image170.jpg]has enaush rfo
etings, cck Bach

oting

Setup Tope:
Development System

Destinaton Dit I
C:\Progiam FestTecnomati,

Component List
Server
Base

Diivers

Clent
Clent Buider |
Corfiguraton Explorer
Web Clert |





	In het nevenstaand scherm wordt weergegeven wat er gewenst is aan installatie opties.
Klik op ‘Next’.

	FactoryLink wordt nu geinstalleerd.



	[image: image171.png]Welcome to the FactoryLink License Wizard

Ha licensing currently exists on this machine. Enter your seri
number and configuration sequence {o begin the 10 day grace
period.

Select which lcensing apton you would ike o perfor:

@ e S Nimber s Coraation Saienca

€ uttoree Yo Syster

€ iew Purchased Optors

Bk ==





	Nu moet de licentie ingevoerd worden.
Klik op ‘Next’.

	[image: image172.png]Enter Configuration Information

Ente the Serial Number and Configurstion Sequence foryour system.
This information may be found on the “Exbit A" icensing document
piovided with your system.

Incamplete o inaceurate infamatian il be displayed i red.

Seial Number

Configuration Sequence:
[EDUCBTTE RalY GCHU DWT3 GRWT WEDH UDSS SE

Advanced.

Bk ==





	Voor ‘Serial Number’ en ‘Configuration Sequence’ in zoals hiernaast is aangegeven. Dit is een development licentie. Nadat de server compleet is ingericht, dient de development licentie vervangen te worden door een runtime licentie. 
(Deze licenties worden bewaard in de grote kluis).

Klik op ‘Next’.



	[image: image173.png]Enter Authorization Information

The system will unfor 10 days without uther authorzation. To
completely authoizs the syster, lease enter the Autorizalion Code and
Licensed Company Name below a5 it appers on the authoriation el

To obtain your authorization exmai, please vist the ollwing URL:

it/ Secnomalis com/services/authorization Hinl

You may select "Later” to defe tis process, but you must authorze the
syste using th ficense wizard within 10 days.

Incamplete o inaceurate infamatian il be displayed i red.

This is your Registiation Code:

[Ci7Z Sh6 2E S 7602 Z96T J346 7E

Please enter the Authorization Code below:

Licensed Corpany Name.

i =





	Klik op ‘Later’.

	[image: image174.png]1) This systemis full functionalFor 2 grace period of 10 day(s). This system must be authorized before the end of the 10 day grace period.





	Klik op ‘OK’.



	[image: image175.png]) Would you lke to launch the DCOMWWE Lty now?

To assure proper commurication between the FactoryLink System and it clents, t s recommended that you run the above utiky.





	Klik op ‘No’.

	[image: image176.png]InstaliShield Wizard Complete

Setup has finished instaling Tecromatis FactoLink 7.5 on you computer.





	Installatie van FactoryLink is nu voltooid.

Klik op ‘Finish’.

Reboot de server !!

Installeer het meest recente servicepack met bijbehorende patches!!


7.8.5 Aanmaken data mappen voor applicatie
7.8.5.1 Datastruktuur aanmaken

	Het bestand ‘Dir-<project-naam>.bat’ maakt alle benodigde directories op de D-schijf aan. Dit bestand bevindt zich op de extra CD\Installatie diversen\data.


	[image: image177.jpg]=loix|

Ele Edt Yiew Favortes Toos el

| &

Qack ~ ) - (¥ | O search [i5 polders | 1% 3 X 9 [ (@

Adress [ p:projectinstalatie DiversenipATA

EBE

Folders
6 project

(=)

@) Documerten

%) FAT SAT Documerten

@ Info database

) Insalate iversen

(=)

) AutoRuns

190 DATA

0 Drwvers

@ Frst

@ FiTsys

%) PonerChute

& sa

@ textpad

5o [ Type

FieF
2k8 wind

[Type: Windows Batch File Author: miordaan Date Modfied: 13-1-2008 22:0:[1,20 K8

| L]
7

[ iy Computer





	Dubbelklik op het bestand Dir-<project-naam>.bat om aanmaken van mappen uit te voeren.

	Voorbeeld bestand voor installatie deel G01C2:

@echo off

REM Create data directories voor FactoryLink

REM Installatie onderdeel: G01C2

d:

cd \

md d:\fl_data

md d:\fl_data\alarmen

md d:\fl_data\fl_database

md d:\fl_data\plc_data

md d:\fl_data\plc_data\p0

md d:\fl_data\plc_data\p1

md d:\fl_data\plc_data\p2

md d:\fl_data\plc_data\p3

md d:\fl_data\plc_data\p4

md d:\fl_data\plc_data\p5

md d:\fl_data\plc_data\p6

md d:\fl_data\plc_data\p7

md d:\fl_data\plc_data\p8

md d:\fl_data\plc_data\p9

md d:\fl_data\plc_data\p10

md d:\fl_data\plc_data\p11

md d:\fl_data\plc_data\p12

md d:\fl_data\trend

md d:\fl_data\visio

md “d:\fl_data\weekbackup db”

md “d:\fl_data\weekbackup db \Weekbackup Spray2”


7.8.5.2 ODBC koppeling voor info database

	De FactoryLink applicatie G01C1 gebruikt een MS Access database om recept date te laden in de applicatiegegevens van ondermeer de stappen programma’s weer te geven. Alle relevante informatie is in deze info database opgeslagen.

Plaats een kopie van de database ‘info.mdb’ in de map ‘D:\f_ldata’.
De FactoryLink applicatie G01C2 gebruikt een MS Access datbase om recept gegevens uit te lezen, voor deze database wordt een vergelijkbare ODBC koppeling aangemaakt met de Fl_rececpt, de database ‘recept.mdb’ is geplaatst in de directory ‘d:\fl_data\fl_database’.



	[image: image178.jpg]Suaapoganmes | Ticern | Grospsgouizevebinding | nio
GebrukersDSN SyteonDIN | BeandsDSN

Systeemgegevensbionner:

Naam Stuprogiomma Tosyosgen
FLecent Microsaft Access Dver (-mdb]

Locaberver 5L Server Vepwideren
odbeBOICT 50L Server —_—
TECNOMATIXNS  Miasct Access Diivr (-] Corligureren
USDATANS  Misrasot Access Diiver ("] e

brengen van een verbinding me! d apgegeven gegevensvoorering.
Een systeemgegevensbron fs zichtbaar voor alle gebrikers van deze
compute, inchusief NT services.

G ||| e

Een DDBC-ysteemgegevensbron beval gegevens aver hettotstand





[image: image179.jpg]Create New Data Source

Select a diiver for which you want 1o set up a data solice.

Name. Ve
Driver da Microsoft para arquivos tewto (- “csv) 4.
Diiver do Microsoft Access [ mckb]
Diiver do Microsoft dBase [~.cbi)
Diiver do Micrasof Excel[sis)
Diiver do Miasolt Paracs b )
Diiver para o Microsolt Visual FosPro

A0 i r
VOB, B | Vieco AcceseTrober (bl
s Mictosoft dB.ase Ditver (. dbf)

B





	Open ‘Start ( Administrative tools ( Data Sources (ODBC)’. Selecteer de tab ‘System DSN’ en voeg een nieuwe koppeling van het type ‘Microsoft Access Driver’ toe.

Stel vervolgens in, zoals in de volgende figuren is aangeven. De naam van de koppeling is ‘INFOcon’.

	[image: image180.jpg]DB

stellingen voor Microsoft Access

Haam van gegevensbror: [FL_recept oK
Beschiving [FactonyLink Recepten database
Annuleren
- Database
Database: D:fldatalf_detabase\Receptenmdh Help

Selecteren. Maken Repareren... | Comprimeren

Geavanceerd

[-Systeemdatabase.

& Geen

© Dtabase:

Susteemdatabase.

- Stuurprogiamma

Paginatime-out

—

I™ Aleentezen

Bulfergostte: [2045

I Exclusief





[image: image181.jpg]Dt uthorzation (o]
oK
Loginnane: =
Passwod
~Gptors
Tope Value

PageTmeant T =
ReadOrny E
ST cions

UseiCammitsync

Valueof  Thieads





	Gebruik de ‘Select’ knop om de juiste database aan de ODBC connectie te koppelen. Kies het bestand ‘INFO.MDB’.

	[image: image182.jpg]Suaapoganmes | Ticern | Grospsgouizevebinding | nio
GebrukersDSN SyteonDIN | BeandsDSN

Systeemgegevensbionner:

Naam Stuprogiomma Tosyosgen
FLecent Microsaft Access Dver (-mdb]

Locaberver 5L Server Vepwideren
odbeBOICT 50L Server —_—
TECNOMATIXNS  Miasct Access Diivr (-] Corligureren
USDATANS  Misrasot Access Diiver ("] e

brengen van een verbinding me! d apgegeven gegevensvoorering.
Een systeemgegevensbron fs zichtbaar voor alle gebrikers van deze
compute, inchusief NT services.

G ||| e

Een DDBC-ysteemgegevensbron beval gegevens aver hettotstand





	De ODBC koppeling voor de Access database INFO.MDB is nu gereed.
Maak op vergelijkbare wijze de overige koppelingen: INFOcon1 en INFOcon2. Alle drie de koppelingen verwijzen naar hetzelfde access bestand. 
Bovenstaande geldt voor de T01C1 applicatie. 

Voor G01C1 hoeft alleen een verwijzing naar de recepten.mdb aangemaakt te worden.


7.8.6 Installatie Sinec H1 driver

	Installatie procedure driver TP2000 t.b.v. H1 communicatie met een TI controller:


	Inloggen met Administator rechten.

Open: \\ControlPanel\Netwerk Connections\Local Area connection.



	[image: image183.png]Gonor | uppon]|

|- Connecton
Status Comnected
Dusstions 005753
Spestt 100.0 Mbps
[-Activy
Sent — 0| — Received
-
=

Packets: 5 115





	Klik op ‘Properties’.

	[image: image184.png]General | Auhentcation | Advanced |

Connest using

89 Bioadoom Nebreme Gigabi Etherme | Configure

This cannection uses the follwing tems:

£ (=l Nctwork Load Bancng
12 e it St o Microsct Hetwarks
¥ Intemet Protocol (TCPAP)

sl | Uninsal Fioperiss
[ Descipion
Allowsyour computer o access fesotsces on aMicosolt
netwak

T Show icon in notfication area when connected
I Notity me when this connection has limited or no connectivity

= [ e





	Selecteer Internet Protocol (TCP/IP)



	[image: image185.png]General | Auhentcation | Advanced |

Connest using

89 Bioadoom Nebreme Gigabi Etherme | Configure

This cannection uses the follwing tems:

sl | Uit Fioperiss

- Desciption
Transrission Cantal Patacal/Iteret Protocol. The default
wide area netviork prtacal tha pravides cmmunication
acoss dverse inerconnected netwarks

T Show icon in notfication area when connected
I Notity me when this connection has limited or no connectivity

= [ e





	Klik op Install



	[image: image186.png]iic the type of network component you want o instal

Hsevce

3 Pratocal

Desciption

A\ ce provides acoess to computers and fes on
the network you are connectingto.

Add Cancel





	Selecteer Protocol



	[image: image187.png]Click the type of network component you want to instal:
Bl Cient

Bseiee
R

[~ Desciption

A protocol i a language your computer uses to
commricats with other compuers.

Add Cancel





	Klik op Add



	[image: image188.png]Cick the Network Protocol that you want to instal, then click OK. 1f you have
% aninstalation disk for this companent, cick Have Disk

] [tmak Petzee
~ bopieTak oo
St TCPP vasion &

B Network Moritor Diiver
B N Link IPX/SPEX/NetBIOS Compatibls Trans v

T — | D
5 This divris digely s

Tellme why diver siqing i inportant
oK Cancel

Manufacturer
Microsaft





	Klik op Have Disk



	[image: image189.png]B e e S |

‘Copy manufacture’s fles from:

- -





	Klik op Browse

Ga naar de directory Install\Program Files\Tecnomatix\FactoryLink\Installs\
IsoTsp2000 directory op de FactoryLink installatie disk



	[image: image190.png]Locate File 2]
Lock i [ 55 IsoTsp2000 — FHoge
Sopsint
[ pbitoh.nf
B updicant
File pame: = Toen
Files of type: | Setup Information (*.inf) [ Cancel !





	Selecteer pbjosi.inf



	[image: image191.png]Fegare  [pount 5 =
Files of type: | Setup Information (*.inf) [ Cancel !





	Klik op open



	[image: image192.png]x|

B e e e |

‘Copy manufacture’s fles from:

CProgrom Fles\TecnomatiFactopLnkintals, =] [ Biaseee ]





	Klik op OK



	[image: image193.png]S ERMENE R ST Tl

Network Protocat I
IMPART IS0 8073 Transpot

This driver is nat digitally signed!
Tellme why diver siqring i inportant

==





	Selekteer IMPART ISO 8073 Transport



	[image: image194.png]S ERMENE R ST Tl

Network Protocat

This driver is nat digitally signed!
Tellme why diver siqring i inportant

==





	Klik op OK

Driver wordt geinstalleerd



	[image: image195.png]Boldon James Licence Agreement

Flease read the following ience agreement. Press the PAGE DOWN keyto
see the restof the agreemen.

[END-USER LICENCE AGREEMENT AND LIMITED WARRANTY FOR
BOLDON JAMES

IMPORTANT-READ CAREFUILLY: This Software Licence Agreement
['SLA")is alegal agreement between you [either an individual ot a single:
entity) and Boldon James Ltd or Leading Technology limited trading as
[Boldon James. In this SLA the term SOFTWARE means computer
software, the assaciated media, any pinted documentation, and any
"online” or electronic documentation. By instaling, copying or othervise
Jusing the SOFTWARE. you agree to be bound by the terms of this SLA. If
lvou do not agree to the terms of ths SLA, then Boldon James are:
unwiling to ioense the SOFTWARE to you. In such event, you may not
use o copy the SOFTWARE, and you should contact promptly Boldon
Wames for nstructions on retun of the unused product(s) fo a refund.

Thvoughaut this SLA and urless the contest requites athervise,
eferences o the singulat includs the piral and vice verss, references to
ry gender include each ofher gender, eferences to persons nclude
incividuals, bodies corporete, urincorporated associatons, patnerships
and other legal ntiies.

Do you accept al of the tems of the preceding Licence Agreement? f you
choose No, Instal wil close. To insallyou st sccept ths agieement.

Yes No





	Klik op Yes



	[image: image196.png][Boldon James Installation

New instalatons:

1 Bodon James has supplied you vith a separate icence for
this prodct,select it locaton below. Otherwise accept the
defaiticence fiom the insallalion media

Upgrading an exsting instaltior:

1 Bodon James has supplied you vith a separate icence for
this prodct,select it locaton below. Othenwise accept the
defaulticenice from the installation mecia, which wil Upgrade
the exsting icence i necessay.

 Defaul cence from nstalation media
& Newlcence fiomthe fis entered here.

[ Bowse
Cancel





	Selecteer

‘New licence from file entered here’.

Klik op Browse



	[image: image197.png][Select Licence.

Lookin: [ 55 IsoTsp2000 oz
Erzs

Fomms ] H =
Flesoftype: [Liosncs i () 2 =]





	Selekteer impCLI.lic

Dit is de meegeleverde licentie op de FactoryLink installatie CD.



	[image: image198.png][Select Licence.

Lookin: [ 55 IsoTsp2000 oz

Fegare  fmoriiic 5 =
e = R— = |





	Clik op Open



	[image: image199.png][Boldon James Installation

New instalatons:

1 Bodon James has supplied you vith a separate icence for
this prodct,select it locaton below. Otherwise accept the
defaiticence fiom the insallalion media

Upgrading an exsting instaltior:

1 Bodon James has supplied you vith a separate icence for
this prodct,select it locaton below. Othenwise accept the
defaulticenice from the installation mecia, which wil Upgrade
the exsting icence i necessay.

 Defaul cence from nstalation media
& Newlcence fiomthe fis entered here.

[E:\Program FieshTecnomais\Factor.
oK Cancel





	Klik op OK



	[image: image200.png]=lolx|

Ele Edt View Favortes Took

Help

| &

Qesck - © - (¥ | L search [ Folders | & 2 X 9 | (@~

Actdess [ CaDocuments and settingstall UsersiStart MenulPrograms\IMPART 32

EE

None - S [T Dote Modiied Atbtes
B coms U Manoger | o shatc a2z 1103 A
)OS Help KB shortcut 922007 11:09 A
B ostRelesse Note e shota s2zmiies A
Eransportdiagnostics e shota s2zmiies A

e shota s2zmiies A

B transprtparameters

5 objects

4,06 kB

[ Compter





	Scherm met geinstalleerde shortcuts wordt getoond. 

Sluit deze af.

	[image: image201.png]Local Network

. ou must shut down and restert your computer before the new settings wil ke ffect.

0 you want to restart your computer now?





	Klik op No


	[image: image202.png]21X
General | Auhentcation | Advanced |

Connest using

89 Bioadoom Nebreme Gigabi Etherme | Configure

i s i S

i o i

e T8

£ Sl hetwork LoadBaning

21 il and Printer Sharing for Microsoft Netwarks _';I
»

sl | Uniosal | Puopeties

- Desciption

Allows your computer o access resources on s Microsoit
retuiork.

I Show con in notfication area when connected
I Notity me when this connection has limited or no connectivity

o || Gt





	Deselekteer ‘Imparth TBH’.



	[image: image203.png]General | Auhentcation | Advanced |

Connest using

89 Bioadoom Nebreme Gigabi Etherme | Configure

This cannection uses the follwing tems:
et for Microsoft Networks f’

LSl TiMPART TBH
1 1= Network LoadBalncng

LElFile and Pinter Shaina for Microsot Networks E
K ) 3

sl | unisal | P

[Descipon
Pat of Boldon James IMPART 150 8073 Transport

I Show con in notfication area when connected
I Notity me when this connection has limited or no connectivity

o || Gt





	Schuif slider naar beneden



	[image: image204.png]General | Auhentcation | Advanced |

Connest using

89 Bioadoom Nebreme Gigabi Etherme | Configure

This cannection uses the follwing tems:

“IWPART 150 8073 Trnspot =
= Bioadcom Advenced Seve Progam Diver

= Ineret Potocal (TCP/P)
] ——

sl | unisal | P

- Desciption
Pat of Boldon James IMPART 150 8073 Transport

I Show con in notfication area when connected
I Notity me when this connection has limited or no connectivity

o || Gt





	Deselecteer IMPART ISO 8073 transport



	[image: image205.png]General | Auhentcation | Advanced |

Connest using

89 Bioadoom Nebreme Gigabi Etherme | Configure

This cannection uses the follwing tems:

IMPART anspart
% Bioadcom Advanced Server Program Diiver

= Intermet Protocol (TCP/IP)
e — |

sl | unisal | P
CDesciplon
1508073 Transpot Ptocl o Boldon ames MPART
appicaions

I Show con in notfication area when connected
I Notity me when this connection has limited or no connectivity

o || Gt





	Klik op OK



	[image: image206.png]=lolx|

Ele Edt View Favortes Took

Help

| &

Qesck - © - (¥ | L search [ Folders | & 2 X 9 | (@~

Actdess [ CaDocuments and settingstall UsersiStart MenulPrograms\IMPART 32

EE

None - S [T Dote Modiied Atbtes
B coms U Manoger | o shatc S22 1143 A
)OS Help KB shortcut 922007 11:13 A
B ostRelesse Note e shota s2wiias A
Eansport isgnostcs e shota s2wiias A
2 Tanspor: Parameters e shota s2miias A

5 objects

4,06 kB

[ Compter





	Sluit scherm af.

	[image: image207.png]Local Network

. ou must shut down and restert your computer before the new settings wil ke ffect.

0 you want to restart your computer now?





	Clik op ‘No’.

Sluit overige schermen af.




	Configureren BoldonJames ISO driver.


	

	Navigeer naar ‘C:\Program Files\Boldon James\Impart’


	

	[image: image208.png]=lolx|

Ele Edt View Favortes Toos Help

| &

Qack ~ ) - (| S search [i5 polders | 1% 3 X 9| [

Aderess [ c:\program Fies)goldon Jamestimpart

EBE

(Folders  X||Mame - size [ Type Date Modfied
(B Deskop o[ Sdocs File Folder 9-2:2007 11:06,
{3 My Documents [S]bjpert.dl 44KB  Application Extens... 5-9-20032:11
= My Computer Flciman.exe 18KB  Application 592003 2:11
5 % Loca Disk () Clereatesnee  6K6 Applcation 92003 2:11
& Dorumets and sttings icient.dl 23K Applcaton Extens... 552006 2111
122 MsOCache Dlicserverexe  30kB  Application 592003 2:11
) Program Files [Simpart.di 20K Application Extens... 5-9-2003 211
& Adobe B mpciiic 1KB LICFie 16-1-2007 8:57
() Anelog Devices (Snetbrows.dl  S2K8  Application Extens... 5-9-20032:11
() AT Technolagies [Bnsaps 3K Fie 592003 2:11
1 ) Boldon James S osipars 28K8 Fie 592003 2:11
55 Inpart Slroutes.exe 18KB Applcation 5320032111
2 docs rudiag.exe 43KB  Application 592003 2:11
122 Broadeom it exe 16KB  Applcation 592003 2:11
5 Commn Fies 116 Fle 3112007 15048
{2 ComPlus Applcations “ tsppars.exe B4KE  Application 5-9-2003 2:11
() Instalshield Instalation Infe | | CJtspservexe  1SKB  pplication 592003 2:11
& Intel [Sespsrv.d 84KB Application Extens... 5-9-2003 2111
& iternet Explrer (| B 536 Applcation Extens. 592003 211
T | ;l_l « | |
Type: Fie Date Modfied: 31-1-2007 1548 Size: 135 bytes 13Sbytes | J My Computer .





	Delete de file ‘subnets’, indien deze aanwezig is.



	[image: image209.jpg]Select network adapters to use with 0SI Transport

Broadoom Netxireme Gigabit Ethemet (P=10.71.50.123) S

Cancel





	Start nu het programma ‘multinic’ via Start ( Run.

Druk op de knop regenerate, en selekteer alleen de netwerkkaart die voor H1 (PLC communicatie) wordt gebruikt. Sluit vervolgens multinic af. 

Let op nu is het bestand subnets aangemaakt, en is goed!


	[image: image210.png]=lolx|

Ele Edt View Favortes Toos Help

| &

Qack ~ ) - (| S search [i5 polders | 1% 3 X 9| [

Aderess [ c:\program Fies)goldon Jamestimpart

EBE

Foders x
(£ my Documents A
2 3 by Computer
[ % Local Disk (C:)
5 Docunents and setings
S maocache
= 5 progran s
© ade
55 fnaog Devices
5 AT Technobges
= 55 bokdon 23mes
55 npart
s
 erosdcom
S common s
5 Complus Agpcatons
S

. Sl ;rl

e sz [Tyee Dot Hodfed_=
S Pl Flder s22007 1105
Sbpertl 44K Applcation Extension
Clamnoe 1560 pplcation
Dlacsnes 516 ppliation
Sicert @ 23K Applcation Extension
Cliesenercre 5010 pplcation
Bimpart i K6 Applcation Extension
g 1Ko Ul
Slretbronsdl 525 Aplcation Etension
S ko Fie
Sosrs w6 e
Cloesee 188 Appliation
Dludogoe 416 plcation
Elnrk.ere 1610 Applcation
Bsuirets e Fie L
AT i plcation
15K Aplcaton

Elrspserv.exe
et
<

R |

IDescription: IMPART transport parameters configuration Company: Protek File Vers 84,0 kB

T v Corpiter 7





	Run program tsppars.exe

. 

	[image: image211.png][IMPART 150 Transport Propes

Schenes | Tanspot Clos 4 Network| E515 | Advarced]

 Conputer
DHesssETS2

- Curtent Parameter Set:

ICL Canformance Level 0 (Read Orip) Save As

(Read Onl)

Ferove Get

oK Cancel Apply Help





	Klik op ‘Save As..’.

Vul in DMV.



	[image: image212.png][IMPART 150 Transport Propes

Schenes | Tanspot Clos 4 Network| E515 | Advarced]

- Computer

DMV-SSSCT52 Select.

- Curtent Parameter Set:

Remove Set

i3 Cancel Apply Help





	Selekteer tabblad 

‘Transport class 4’.

	[image: image213.png][IMPART 150 Transport Propes

Soheres  Trarspo Cess & | Network | ESIS | Advareed]

Class 4 Pratocol Dptions
Masimum TPDLU size [102¢ =]
Masimum Credt =
T4 (Connect Flequest Tiner) =
T1 DaaRetaenision Tme) 7 2
N (Retransrission Count] Emat
W (Window Timer) [ =]
1 inactiviy Times) [ =
Allow na checksums V.

il extarded famad





	Wijzig instellingen volgens onderstaand voorbeeld.

Maak de volgende wijzigingen, deze staan anders op de afbeelding:

T1 (connect) = 5 seconden

T1 (retrans) = 5 seconden

N = 2

Met deze instellingen worden disconnects sneller gedetecteerd, hetgeen de overall respons ten goede komt.

	[image: image214.png][IMPART 150 Transport Propes

Schenes| Trnspon Class § Netwark | E515 | Advanced|

CLNP Potocol Dptons
e chedkains r
Genarae o repat PDU r

‘Segmentation permission mandatory [~

efaul ffetime. =
Defaut IPDU Liet =
eassembly tier m =
Resssembly ER=|
Enable route recording r

Cancel Epply Help





	Selekteer tabblad ‘Network’.

Vergelijk of de instellingen gelijk zijn.

Open tabblad ‘ESIS’.

	[image: image215.png][IMPART 150 Transport Propes

Schenes| Transpot Clos ] Network ESIS | dvarced]

SIS Type

& HIGESTS Sippatt
© End System

© Intemedite System
[ Masimu g e [500 =]
 BOES andis o

Corfiuationtiner 120 =

- Protoco] Options
IV hcoept suagested corfiguraion et
I Eieceive £5 Hellos ot ozl E5:

715 Protocol Hpticns
= erd sugested corfiguraton e
1= FeceivelG Helos aticosllS:

-9 Al 19 Protocol Opions

I Eneble configuraton rofficaton

[ Cancel Apply Help





	Vergelijk of de instellingen gelijk zijn.

Open tabblad ‘Advanced’.

	[image: image216.png][IMPART 150 Transport Propes

Schenes| Tanspot Clos 4 Network| E515  Advareed |

[~ Transpott PDU Gueies

Transtit TPDU queve high water mark:

Rieceive TPDU queve high water mark:

[~ Transport T1 timer adustment

Enable dynamic T1 timer adustment r

[~ Mulle Adspters on the same network

LAN Frame disable it o=
Wavimum %, 25/Cs per Line =]

oK Cocel | dop | Hebp





	Vergelijk of de instellingen gelijk zijn.

Klik op ‘OK’.

Reboot machine.

Na rebooten is driver geconfigureerd.


7.8.7 Systeem variabelen servers
Open via Start/My Computer/Propertys het tabblad ‘Advanced’. Klik op de button ‘Environment Variables’. 

7.8.7.1 Environment variabelen MASTER server (server 1)
Maak onder ‘User variables for Administrator’ de volgende environment variabelen aan:

· FLUSER
met waarde fluser1
Maak onder ‘System variables’ de volgende environment variabelen aan: 

· FLHOST

met waarde <computernaam server> (bijv. DMV-SSSC1-S1)
· FLPARTNER  met waarde ‘slave server naam’*
· FLAPP

met waarde project inclusief path (bijv. D:\FL\SSSC1)

· FLSTATION
met waarde 1

· FLSINECH1
met waarde 1

7.8.7.2 Environment variabelen SLAVE server (server 2)

Maak onder ‘User variables for Administrator’ de volgende environment variabelen aan:

· FLUSER
met waarde fluser1

Maak onder ‘System variables’ de volgende environment variabelen aan: 

· FLHOST

met waarde <computernaam server> (bijv. DMV-SSSC1-S2)

· FLPARTNER
met waarde ‘master server naam’*
· FLAPP

met waarde project inclusief path (bijv. D:\FL\SSSC1)

· FLSTATION
met waarde 2
· FLSINECH1
met waarde 2

*) De master of slave server naame is de machine naam van de partner server, partner voor redundantie.

7.8.7.3 Host variabelen

Open via c:\WINDOWS\system32\drivers\etc\ de file hosts.

Voeg deze toe voor de MASTER en de SLAVE server.

Voeg de volgende variabelen toe (indien al niet reeds aanwezig):

· 127.0.0.1




localhost
7.8.7.4 Service variabelen

Open via c:\WINDOWS\system32\drivers\etc\ de file services.

Voeg deze toe voor de MASTER en de SLAVE server.

Voeg de volgende variabelen toe (indien al niet reeds aanwezig):
	USDCVRN
	7579/tcp   
	usdcvrn
	#FactoryLink VRN service

	FLLICSRVC
	8000/tcp    
	fllicsrv
	#FactoryLink License Server

	FLCONNSRVC
	6096/tcp    
	flconnsrvc
	#FactoryLink Connection Server

	FLDATASRVC001
	6097/tcp    
	fldatasrvc001
	#FactoryLink Data Server

	FLDATASRVC002
	6098/tcp    
	fldatasrvc002
	#FactoryLink Data Server

	FLDATASRVC003
	6099/tcp    
	fldatasrvc003
	#FactoryLink Data Server

	FLDATASRVC004
	6100/tcp    
	fldatasrvc004
	#FactoryLink Data Server

	FLDATASRVC005
	6101/tcp    
	fldatasrvc005
	#FactoryLink Data Server

	FLDATASRVC006
	6102/tcp    
	fldatasrvc006
	#FactoryLink Data Server

	FLDATASRVC007
	6103/tcp    
	fldatasrvc007
	#FactoryLink Data Server

	FLDATASRVC008
	6104/tcp    
	fldatasrvc008
	#FactoryLink Data Server

	FLDATASRVC009
	6105/tcp    
	fldatasrvc009
	#FactoryLink Data Server

	FLDATASRVC010
	6106/tcp    
	fldatasrvc010
	#FactoryLink Data Server


De server is nu voorbereid op:

· VRN server (Veld communicatie en communicatie tussen redundante servers).
· License server (Server voor afhandeling toekenning licenties FactoryLink).

· Connection server (Server voor communicatie met derden systemen)

· Dataservers (Servers voor communicatie met FactoryLink clients).
7.8.8 Debug informatie voor Licentie Server

The license server debug messages are printed to the Win32 console and can be viewed with DebugView which can be downloaded for free from SysInternals (now Microsoft):  

http://www.microsoft.com/technet/sysinternals/Miscellaneous/DebugView.mspx  

To debug FactoryLink 7.0:  

 Debugging is enabled by default, use DebugView to see the CAL transaction messages when clients connect and disconnect.  

To debug FactoryLink 7.1 and 7.2 Pre SP2:  

 The license service in 7.1 was changed so it no longer automatically outputs information that can be picked up by a debug program. It can be started with an argument from the command line to show you this information.  

1) Stop the Factory License Server  

2) Run LicServer -v4 from command line  

3) Run dbgview.exe to see CAL transaction messages when clients connect and disconnect.  

To debug FactoryLink 7.2 with FactoryLink SP2, 7.5, 7.5 SP1, and 7.5 SP2:  

 The license service in 7.2.2 (SP2) and above was changed to add redundant licensing. To facilitate debugging on the license server PC, you must change the following registry entry to TRUE: HKEY_LOCAL_MACHINE\SOFTWARE\Tecnomatix\FactoryLink\ Licensing\Debug License Server. The only valid values are TRUE or FALSE. TRUE = turn on debugging, FALSE = no debugging.  

 If the registry entry can not be found, run the LicUtility.exe then click OK. The registry entry will then be added. Then you should be able to edit it.  

 Once you have made the registry change, you must stop and restart the license server so that it will read the new registry changes. Then you can run the dbgview.exe utility and see the CAL transaction messages.  

To debug FactoryLink 8.0 and above:  

 Run the License Utility and in the Local License Server Settings section, click on the check box next to Enable debug trace so that a check mark is in the box. Then click on the OK button to close the License Utility. You will get a warning message that pops up telling you the License Server needs to be restarted. Click on the Yes button and it will restart the License Server for you. Now you can run the dbgview.exe utility and see the CAL transaction messages.

7.8.9 Wijzigen MAC adres server

	Let op: Het wijzigen van het MAC adres van een server is niet geheel zonder gevaar.

Indien de wijziging toegepast wordt om een server te vervangen betekend dit dat er twee servers in omloop zijn met hetzelfde MAC adres. Als de vervangen server al dan niet gerepareerd weer in omloop komt kan dit bij connectie op hetzelfde netwerk als waar zich de actuele server bevindt dit ernstige netwerkproblemen veroorzaken. 

****** GA MET EEN MAC ADRES WIJZIGING HEEL VOORZICHTIG OM  ******



	[image: image217.png]General Advarced | iver | Detas | Resources | Pover Managermen |

The follwing praperies are avaiabl fo this network adapter. Cick
the property you wari to change on the ef, an then select it value
onthe ight.

Property:
6021p 005
Checksum Ofioad

Ethermet@wieSpesd
Flow Corirol © Not Present

Laige Gend Offoad

Value:
© [ooi7asssEsE0

Speed & Duplex
Wake Up Capabilles
WOL Speed

=





	Rechtermuisknop op ‘My Computer’

Select ‘Manage’

Select ‘Device Manager’ linker panel

Select ‘Network adapters’ rechter panel

Rechtermuisknop op bovenste network kaart (moederboard)

Select ‘Propertys’

Select tabblad ‘Advandced’

Select ‘Local Administered Address’

Select selectie naast invulveld ‘Value’

Vul in op invulveld (onder ‘Value’) het MAC address in (12 karakters)

Clik op ‘OK’.

	Om het orginele MAC adres weer terug te zetten (Hard coded op netwerkaart) , herhaal de handelingen zoals boven vernoemd. 

In plaats van het MAC adres in te vullen selekteer ‘Not Present’ en klik op ‘OK’.


7.8.10  Opstartscript FactoryLink clients
Voer de volgende items uit:

· Maak op D:\ een directory aan ‘FL Client shortcut’.

· Copieer het programma pslist.exe in deze directory.

· Copieer het logon script in de file ‘FactoryLink client logon.bat’.

· Plaats een shortcut van voornoemde batfile onder de naam ‘FactoryLink client start’ op de desktop van de client. Link aan deze shortcut het orginele FLRUN icon van FactoryLink. (Path: C:\Program Files\Tecnomatix\FactoryLink\Server\BIN\flrun.ico.)
Werk directory: "C:\Program Files\Tecnomatix\FactoryLink\Client\WebClient\Bin"

Als voorbeeld een client login voor de G01C2 installatie, in regel 2 en 3 dient de juiste server naam ingevuld te worden: 

@echo off

rem Batchfile voor het automatisch opstarten van clients

SET Server1=FL_WEI_S1

SET Server2=FL_WEI_S2

SET path=C:\Program Files\Tecnomatix\FactoryLink\Client\WebClient\Bin;%path%

rem controleer of weblcient reeds is gestart, en stop batch file als dat het geval is

pslist.exe graph > nul

if %errorlevel% == 0 goto end

rem start facorylink weblcient

:start

cls

echo *****************************************************************************

echo * Webclient applictie voor Server 1 wordt gestart. Even Geduld...           *

echo *                                                                           *

echo * Laat Dosbox open staan, zodat bij uitval FL master automatisch geprobeerd *

echo * wordt om op de andere server in te loggen                                 *

echo *****************************************************************************

rem net use \\%Server1% /delete

rem net use \\%Server1% /user:Admin bgktv

flweb.exe -n-m30,-z5000:100 -h%Server1%:6096

rem Controleer nogmaals op dubbele webclient

pslist.exe graph > nul

if %errorlevel% == 0 goto end

cls

echo *****************************************************************************

echo * Webclient applictie voor Server 2 wordt gestart. Even Geduld...           *

echo *                                                                           *

echo * Laat Dosbox open staan, zodat bij uitval FL slave  automatisch geprobeerd *

echo * wordt om op de FL master in te loggen                                     *

echo *****************************************************************************

rem net use \\%Server2% /delete

rem net use \\%Server2% /user:Admin bgktv

flweb.exe -n-m30,-z5000:100 -h%Server2%:6096

rem blijf dit script herhalen!!!!

goto start

:end

7.8.10.1 Informatie programma pslist

Most UNIX operating systems ship with a command-line tool called "ps" (or something equivalent) that administrators use to view detailed information about process CPU and memory usage. Windows NT/2K comes with no such tool natively, but you can obtain similar tools with the Windows NT Workstation or Server Resource Kits. The tools in the Resource Kits, pstat and pmon, show you different types of information, and will only display data regarding the processes on the system on which you run the tools. 

PsList is utility that shows you a combination of the information obtainable individually with pmon and pstat. You can view process CPU and memory information, or thread statistics. What makes PsList more powerful than the Resource Kit tools is that you can view process and thread statistics on a remote computer. 

Installation
Copy PsList onto your executable path and type "pslist".

Usage
The default behavior of PsList is to show CPU-oriented information for all the processes that are currently running on the local system. The information listed for each process includes the time the process has executed, the amount of time the process has executed in kernel and user modes, and the amount of physical memory that the OS has assigned the process. Command-line switches allow you to view memory-oriented process information, thread statistics, or all three types of data.

usage: pslist [-?] [-d] [-m] [-x][-t][-s [n] [-r n]  [\\computer [-u username] [-p password]] [[-e] name | pid]

	 
	-d
	This switch has PsList show statistics for all active threads on the system, grouping threads with their owning process.

	 
	-m 
	This switch has PsList show memory-oriented information for each process, rather than the default of CPU-oriented information.

	 
	-x
	With this switch PsList shows CPU, memory and thread information for each of the processes specified.

	 
	-t
	Show process tree.

	 
	-s [n]
	Run in task-manager mode, for optional seconds specified.

Press Escape to abort.

	 
	-r n
	Task-manager mode refresh rate in seconds (default is 1).

	 
	-u
	Specifies optional user name for login to remote computer.

	 
	-p
	Specifies optional password for user name. If you omit this you will be prompted to enter a hidden password.

	 
	name
	Instead of listing all the running processes in the system, this parameter narrows PsList's scan to those processes that begin with the name process. Thus:

pslist exp
 

would statistics for all the processes that start with "exp", which would include Explorer.

	 
	-e
	Use the -e switch if you want the process name to be treated as an exact match instead of a partial match.

	 
	pid
	Instead of listing all the running processes in the system, this parameter narrows PsList's scan to the process that has the specified PID. Thus:

pslist 53
 

would dump statistics for the process with the PID 53.


7.9 SQL-server Database instellingen

De SQL server database instellingen en configuratie voor replicatie en onderhoudstaken worden gedaan nadat alle tabellen door FacotryLink aangemaakt zijn.
Voordat replicatie en onderhoudstaken geconfigureerd worden dient de tabel LOGBOOK aangepast te worden. Aan deze tabel dient een kolom toegevoegd te worden, namelijk TMSTAMP, met type ‘datetime’, de standaard (default) waarde van de kolom wordt opgegeven als GETDATE(). De kolom mag geen lege vleden bevatten. Deze extra kolom is nodig om ‘oude’ records met een onderhouds-script te kunnen verwijderen.
7.9.1 SQL Server stored procdures en jobs

	Voor SQL server onderhoud dienen een drietal jobs aangemaakt te worden. Start de ‘Query Analyzer’, via Start > All Programs > Microsoft SQL Server > Query Analyzer.

	[image: image218.jpg][
@ R (o |

T~ Start SQL Server i itis stopped

Connest using
© Windows authertication
& 50 Server autentcation

Login name: 2

Password: I

o Cwesl | Heb





	Na opstarten van de ‘Query Analyzer’ dient ingelogd te worden. Log in op de localhost (gebruik … om de juiste server te vinden), de login naam is sa, zonder wachtwoord.

	Nu wordt de Query Analyzer zichtbaar, in de menubalk dient de juiste database geselcteerd te worden, doe de keuze.

Er dienen drie jobs aangemaakt te worden, hiervoor worden achter een volgens drie sql bestanden geopend en eenmalig uitgevoerd. Deze bestanden Zijn te vinden op de extra-CD\Installatie diversen\SQL\Server<server nummer>, met extensie sql. Het betreft de volgende bestanden:

T01C1


OpschonenALARMS.sql





OpschonenLogbook.sql





OpschonenTrending.sql

A01C1


OpschonenALARMS.sql





OpschonenLogbook.sql





OpschonenLoops.sql





OpschonenTrending.sql





ClearConflictTables.sql





sp_ClearConflictTable.sql

G01C2


OpschonenALARMS.sql





OpschonenTrending.sql




IdentiekeDatabasesG01C1.sql




mp_check_remote_server.sql




mp_copy_table.sql




Weekbackup spray2.sql

Na openen van een bestand dient op de knop met de groene pijl gedrukt te worden, dan wordt het script eenmalig uitgevoerd. Herhaal dit voor alle te openen bestanden, de sql code wordt nu als job (of stored procedure) opgeslagen.



	[image: image219.jpg]5QL Query Analyzer - [Query Installatie Diversen\S0L\OpschonenALARMS.sal] —[ol x|

53 6le et Query Took window_tes —l8ix|
B-FEE| ) bREM|o|E-|v e ([0osocs & &8s | 55
Obict Browser x| Server: riervrr [beare Quer 5)

T

seom TaNsAcTION J

3 FLEMWIT ST DECLARE BJobID BINARY(16)
@ oe_tonct_s1 DECLARE BReturnCode INT
U master SELECT @ReturnCode = 0
U model IF (SELECT COUNT(*) FRON msch.dbo.syscategories UHERE name = N'Database Naintenance') < 1
g b, EXECUTE mscb.dbo.sp_add_category @name = N'Database Haintenance'
e

ommon Objects
Configuration Functior
Cursor Functions
Date and Time Functi
Mathematical Functior
ggregate Furnctons
Metadata Functions
Secuty Functions.
Sting Functions
System Functions
System Statistcal Fun
Test and Image Funct
Fowset

System Data Types

- Delete the job with the same name (if it exists)
SELECT @J0bID = job_id

T e

WHERE (name = N'Opschonen ALARMS')

IF (BIGHID IS NOT HULL)

BEGIN

The comuand(s) completed successtully.

T |

kil |
“ 2| [ Gds £ Messages
5 tiecs BT Toroes] [0y ot complotea FL_ETWIT_51(3.0) 5 (5% [DB_ToIC1 51 [0:00:00 [0rows [n2s, Col3t

‘ [Comnections: 1 [ [ [,






	Na uitvoeren van alle scripts kan in de SQL Manager de zojuist ingevoerde jobs bekeken worden, zie onderstaande figuur. Controller of deze jobs aanwezig zijn, en op enabled staan.

	[image: image220.jpg]=lolx|

@ oe_toici st
2 Dats Transformation
c@ s

593 saL server Ager
@ aes

@ Operstors
2obs
@ Bk

g curen: actvry
Database Maite
SQt severLogs

{23 Replcation _,;]
‘ 3

‘B Fle Actin View Toos Window Help [ISETE|
e | BEFRR[(@m(k[NE 0BG
Consde Roct ob
Miroscf sQL Servers
=9 O o e/ Categon Enstled | rumnatle | sched... [ status Lot un . [Nt Run .
=8 (0C) (Windowstry || 2]0pschonen auaris Dotabase Mainten... Yes  Yes Ves  NotRumwing Unknown (1912200
£ Databases [Slopschonen LogBoOK Database Mainten... Yes Yes Yes Nok Running  Unknown (19-12-200,
[8]opschonen TRENDING Database Mainten. Yes Yes Yes Not Running  Unknown (19-12-200.







7.10 McAfee Virus scanner
De McAfee virus scanner is de defacto standaard van DMV (althans op het moment van aanmaken van dit document). Echter tijdens testen van de geïnstalleerde servers bleek dat de standaard instellingen van de virus scanner een beperking aan FactoryLink oplegden. Dit uit zich door de volgende meldingen en of niet kunnen uitvoeren van handelingen:

· Save en Restore van een mps file van de applicatie duurt met geïnstalleerde virus scanner veel langer dan zonder virusscanner.

· Openen van een remote applicatie met de Configuration Explorer werkt niet bij geïnstalleerde virusscanner.

· Vermoeden is dat licentie server soms lang wacht met vrijgegeven van een ingebruik genomen licentie.

Met bovenstaande gedragingen is besloten om de McAfee virus scanner voorlopig niet te installeren.
7.11 Clamwin virus scanner
	ClamWin Free Antivirus is een gratis Antivirus programma voor Microsoft Windows 98/Me/2000/XP/2003/Vista. ClamWin biedt een grafische interface naar de Clam AntiVirus scanning engine. Het ClamWin Free Antivirus programma is voorzien van een gebruiksvriendelijke installatie procedure en ontwikkeld in open source code.

Belangrijkste kenmerken: 

· Geplande scans. 

· Automatische Updates van de Virus Database. Het ClamAV team werkt de Virus Database regelmatig bij, en bijna altijd onmiddellijk nadat er een nieuw virus of een nieuwe variant uitkomt. 

· Zelfstandige Virus Scanner 

· Context Menu Integratie voor de Windows Verkenner 

· Add-in voor Microsoft Outlook. 

· De laatste versie van Clamwin Free Antivirus is 0.94.1 

Opgelet! ClamWin Free Antivirus ondersteund nog geen real time scanning. Bestanden die u opent of uitgevoert worden niet automatisch gescand. U moet dus manueel de bestanden scannen om een virus te vinden. De MS Outlook Add-in kan wel automatisch virussen onderscheppen in email-bijlagen.

	[image: image221.jpg]@ Antivirus Setup Wizard

This wil install ClarWin Free Antivius 0.94 on your computer.

Itis recommende that you close ol ther applications before
conliuing,

Click Nextto continue, o1 Cancel o exit Setup.

==





	Start de installatie met: clamwin-0.94-setup.exe
Ga verder met ‘Next’.

	[image: image222.jpg]& Setup. irus

License Agreement
Flease read the following important nformalion before contiving.

Flease read the following Lizense Agreement. You must accspt the tems o this
greement before continuing with the installaion

[GNU GENERAL PUBLIC LICENSE :‘
[Version 2, June 1991

\Copyright (C) 1989, 1991 Free Software Faundation, Inc.
/59 Temple Flace - Suite 330, Boston, MA 02111-1307, UsA

[Everyone is permitted to copy and distribute verbatim copies
lof this icense cocument, but changing it i ot alowel.

i 3ccep the agieement
1o not acoept the agreement

Bk =





	Accepteerd de licentie en ge verder met ‘Next’.

	[image: image223.jpg]& Setup.

Select Installation Options
Who should tis application be nstalled for?

Flease select whether you wish to make tis softwate avaiableto allusersor st
yoursel

& e e e T o [ i)

Ol for me (Adriististor)

Bk =






	Kies voor ‘Anyone who uses this computer (all users)’ setup, ga verder met ‘Next’.


	[image: image224.jpg]11 Setup irus

Select Destination Location
‘Where shauld Clanwin Free Aniivits be installed?

£ ettt o s i ol

To cantinue, click Nest If you would ke to select  diferent folder, lick Brawse.

Atleast 18.5 MB offee disk space s requied.

Bk =





	De standaard installatie directory kan geaccepteerd worden.
Ga verder met ‘Next’.

	[image: image225.jpg]ot Conponenic -
o st [Z

Selectthe components you want o nstal; cear the components you do not wart o
instal.Click Next when you ae read) to cortinue.

Custom Instalation

Clame Fils
Clanin Files

Itegration vith Windows Explorer
Itegration with Microsoft Dutlook. 0118
Itemational Help Fles 08ME
Duich Help e
O Fiench Help Files: TEMB

Curent selecton requies t least 39,8 MB of disk space.

Bk =





	Selekteer additioneel de Nederlandse taal, en g a verder met ‘Next’.

	[image: image226.jpg]& Setup. irus

Select Start Menu Folder
‘Whers should Setup place the progran's shortcuts?

Setup il cesteth proggansshotuts i th folowing et Menafler.

To cantinue, click Nest If you would ke to select  diferent folder, lick Brawse.

| Catin i

T~ Don't create a Start Menu folder

Bk =





	Accepteer de standaard om een programma groep aan te maken, en ga verder met ‘Next’..

	[image: image227.jpg]11 Setup irus

Select Additional Tasks
‘Which addional tasks should be performed?

Select the addiional tasks you would ke Setup o perform while nstaling Clariwin Free:
Antivius, then click Next

Dowrlosd
T~ Giiad Vi Dabase Fies [0 ot Seleti o soiee i & Py S
Addionslico:

IV Creste a desktopicon

Bk =





	

	[image: image228.jpg]=lolx]

Click Instll to continue with the instalation, o cick Back if you want o rvisw or
change any seltings.

[Destnation localior:
C:\Progiam Fles\Clariwin

Setup type:
Custom Installation

Selected components:
Clam Fils
Clanwin Fes
Itegraton wih Windows Explorer
Itegralion with Mictosoft Outlook
Iteinational Help Fies
Dutch Help Files

L o

Bk =





	Start de feitelijke installatie met ‘Install’.

	[image: image229.jpg]! Setup

Installing
Flease wait while Setup insalls Claniwin Fee Antivius on your computer.

Extracting fs.
C\Documents and Setings\All Users clamwindbmain cvd





	De voortgang van de installatie wordt met een voortgangsbalk weergegeven.


	[image: image230.jpg]=loix]

=1 Completing the ClamWin Free
@  Antivirus Setup Wizard

Setup has finished installng Claniwin Free Aniivis on your
computer. The application may be launched by selecting the
instaled icons.

Clck Firsh to st Seup.





	De installatie is voltooid, installeer vervlgens de laatse updates en virus definities. Dit kan direct vanaf internet, of via bestanden die vanaf een andere computer gedowlnload zijn..

	De laatste (nieuwste) virus definities kunnen gedownload worden via http://database.clamav.net/main.cvd en http://database.clamav.net/daily.cvd. 

Copiëer de gedownloadde main.cvd en daily.cvd naar je database locatie die ingesteld is in het menu "ClamWin Preferences", op de tab "File Locations". De standaard database locatie is: "C:\Documents and Settings\All Users\.clamwin\db"


8. Opnieuw aanvragen van een FactoryLink license
Een license kan om de volgende redenen opnieuw aangevraagd worden:

· De harddisk moet vervangen worden.
· Processor is vervangen

· Systeem is geüpgrade

· Vervangen van de PC

In het verleden moest de licentie worden gemigreerd m.b.v. de deinstall utility van FactoryLink.

Tegenwoordig (2005) gaat dit via de website. 

Na het installeren van FactoryLink moet via het programma “FactoryLink Install” de serial en configuratie sequence worden ingevoerd. Vul hier de oude gegevens in en bevestig deze volgens figuur 1.
[image: image231.png]Thark you for choosing FactoryLink Enterpise Contol System as your SCADA solton.

Installwil place the fcensed FactaryLink saftwar in the folowing defauit cive:\dieetory, or you can type a diferent
dive:\diector:

Instalto L]

It also create a Program Group.
yout own e

Frogram Group: [FactonyLink £C5
‘Application Ditector: [CiFLapP

Enter you seial rumber: e

isplaying the FactaryLink icans. YYou can use the folwing defaul tle,oryou can erter

Enter yourconliguration sequence:

[ o v e e v e
Enter your pimery language preference: En E

To quit Instal at any tine ding the instalston process, choose the EXIT icon on the background screen offunction key, F3.

Hee |





Figuur 1
Geef de licentie directory in (fig.2)

[image: image232.jpg]The FactorpLink license diectory cortains special s which hold information on alicensed options.
Yo may enter a diferent cense diector ot you may choose the defaul

[CAFLNTAGPTRUN

K CANCEL HELP

DEFAULT





Figuur 2
Kies in het menu van  fig 3 voor Additional Functions.
[image: image233.jpg]Chosse the type of installaton or FactoryLink by clicking one of the buttons:

EXPRESS Instalation

Installs ol censed Factoink software modlles. I requites a toal of 53341K' ytes ofdisk. space.

CUSTOM Installation/Deinstalation

Lets you decide which liensed FactorLink software modues to instaldeinstall. Disk space reuied/reed is
recomputed duing the nstallaton.

OPTION Instaliaion
Updates an existing FactoryLink system with optonal modkes. ( e Diivers, PAK. 3idpaity )

=]

Addiional Instal Functions

Invokes registralion, suthorzaton, operating system, network, program gioup, and font setups. Ako displys
documentatin.

CANCEL HELP





Figuur 3
In Additional Functions kies voor Software Registration (fig 4) 

[image: image234.jpg]Additional Install Functions

FacloLink ECS Selup Functons

[ wsra—] |

o | etk St |

T \ \

FactoyLink ECS Iformation

Intoled S Moes | \

AvlblSowacModies | NEW Fosas |

EEE | Il |

Trairing Couses
CANCEL HELP





Figuur 4
Bij het “Licence Agreement “ scherm geeft men vervolgens ACCEPT (fig.5)
[image: image235.jpg]The use of FactoryLink s resticted 10 3 single CFU
Each FactoyLink package includes a seral umber and a carfiguration sequence that must be

entered during the instalaion. The configuration sequence i used o instal authorized
FactoyLink mockes and programs.

Belore using any ofthe programs of moches, the user mst execute and deliver to United Stales.
Data Corporation the FactoryLink Software Agreement o a simia iense agreement spproved by
United States Data Corparaiion.

A copy ofthe FactonLink Soltwere Agresment s included in the docurentation

Flease acknowledge your acceptance and rafication of the terms and conditons of such
greement by typing the word shown below:

Flease type ‘ACCEPT a this tie:





Figuur 5
Noteer het registratie nummer van fig.6 en neem vervolgens via het internet contact op met de leverancier van FactoryLink.
[image: image236.png]The registratian information for tis Factorylink fcense is as folws:

Seal Number )
Configuration Sequence:  Fiarty - 110,65 PHEE TTHU 2eny
Riegistiation Number: CAEA UDKI 38T 6NGY H4MY TGIE ND.

For your converience, this nformaton has been recorded inthe fllowing tet fl:
CAFLNT\FAXTXT

To eceive your uthorzation number, FAX this text il or call the Urited States Data
Corparation autharization department. Onc you have received the autharizalion
numbe, proceed to Softwate Authorization

Phone rumer 800) 7564383 -Or-(372) 497.0382
FAX rumber: (872) 8631428

] HELP




 
Figuur 6
Op het moment van schrijven is dat het volgende adres:

http://webora.tecnomatix.com/oracle/auth/auth.asp waarna onderstaande formulier verschijnt.

Vul dit de gegevens in en na op “Continue” te hebben geactiveerd moeten nog enige gegevens ingevuld worden waaronder een e-mail adres. 

Software Registration and Authorization 

Bovenkant formulier

	To register and authorize your software, please enter the following information and klik the Continue button. Fields marked with an asterisk (*) are required input. 


	Serial Number: * 
	 
	[image: image237.wmf]



	Registration Code: * 
	 
	[image: image238.wmf]


	


[image: image239.wmf] Continue 


Onderkant formulier

In de e-mail die men vervolgens krijgt staat o.a. de nieuwe autorisatie code. Deze code moet ingevuld worden in het menu welke men kan activeren in met de button rechts boven in fig.6. 

Hierna moet het installatie menu afgesloten worden.
Nadat FL opgestart is, met als environment variabel SET FLOPT=C:\FLNT\OPTRUN, kan worden gecontroleerd of dat de code geaccepteerd is. Ga daarvoor naar het systeem menu en kontroleer daar of de DECODER in “running” staat. Is de code niet geaccepteerd of nog niet ingevuld dan krijgt men de melding “Running In Demo Mode”.
9. Applicatie wijzigingen doorvoeren

Een wijziging in een redundante applicatie dient volgens een vast patroon op de beide servers doorgevoerd te worden.

Wat kan er mis gaan? Uitwisseling van tag waarden tussen master en slave kan niet goed functioneren, dit wordt veroorzaakt hhet NIET gelijk zijn van de applicaties op master en slave systeem!Dit is zichtbaar door dat tags een verkeerde waarden hebben.

Voor redundantie maakt FactoryLink met VRN gebruik van (interne) id’s voor tags, er wordt dus niet met een naam gewerkt. Bij een opstart en een online herstart worden voor nieuwe tags deze id’s incrementeel bepaald door een vorm van compilatie (ctgen). Indien deze incrementele generatie niet op beide systemen identiek is zijn er tussen beide applicaties verschillen. En kunnen genoemde problemen optreden. Bij toevoegen, verwijderen of aanpassen tag definities kan het probleem optreden.

Wordt een dergelijke wijziging uitgevoerd dan is het volgende stappenplan een manier om probleemloos wijzigingen door te voeren:
1. Server 1 (master) en server 2 (slave) zijn normaal in gebruik;

2. Voer de wijziging op server 1 (master) uit;

3. Stop FactoryLink op server 1;

4. Voer op server 1 het volgende uit:
a. maak een Multi-platform save (mps) van de applicatie.
b. genereer de taglijst volledig opnieuw, door vanaf de command line in te geven: 
ctgen –c –r –v;

5. Stop FactoryLink op server 2;

6. Start FactoryLink op server 1;

7. Doe nu het volgende op server 2:
a. Copieer mps vanaf server 1 en restore deze;
b. Pas tSAP namen aan voor sever 2, maak applicatie van server 1 geschikt voor server 2;
    Iedere server is vorzien van de optie dat de H1 instellingen automatisch goed gezet
    worden, echter dit betekent dat de server 2 maal gestart moet worden!
c. start FactoryLink op server 2;
d. stop FactoryLink op server 2;
e. start FactoryLink op server 2;

8. Wijziging volledig doorgeveord.

Appendix A Source code listings
Source code autit3 script flweb.au3 tenbehoeve van weblcients.

;

; AutoIt Version: 3.0

; Language:       English

; Platform:       WinXP/2003/Vista

; Author:         Marcel Jordaan (mjordaan@rldautomation.eu)

;

; Script Function:

;   Starts the FactoryLink webclient using graph.exe, and accepts the the same commandline parameters as graph.exe.

;   This script will start the weblcient and close all popup messageboxes reporting errors, normally weblcient will close 

;   after accepting popup messages!

;   Note: the path variable should be set to the bin-directory where webclient is installed

; Usage:

;   Calling this script in cmd or batch file makes it possible to auto-close webclient after an error, 

;   and restart the same or an alternative weblcient connection.

;initialize the command line parameter string

$params = ""

;Get the commandline and build the parameter list for webclient

For $i = 1 to $CmdLine[0] step 1

  $params = $params & " " & $CmdLine[$i]

Next

;start webclient with parameters

Run("graph.exe" & $params)

;wait for graph to start, no longer then 20 seconds

$wcproc = ProcessWait( "graph.exe", 20)

If $wcproc = 0 Then

  MsgBox(0, "FactoryLink webclient", "'Graph' did not start. Bye!")

  Exit

EndIf

;get the PID for graph.exe, so we can check if the process is still running

$webclient = ProcessExists( "graph.exe")

;Wait for webclient toe report a failure

;weblcient should be running, if not quit

while ($webclient <> 0)

  ;weblcient uses standard messageboxes to report errors, other programs use the same

  ;class names for the standard messageboxes.

  ;if we find a standard message box, check if it is owned by webclient

  $var = WinList("[CLASS:#32770]")

  ;$array[0][0] = Number of windows returned

  ;$array[1][0] = 1st window title

  ;$array[1][1] = 1st window handle (HWND)

  ;$array[2][0] = 2nd window title

  ;$array[2][1] = 2nd window handle (HWND)

  ;loop through all the standard messageboxes

  For $i = 1 to $var[0][0] step 1

    ;check if the messagebox is owned by webcient, if not just ignore

    If $webclient = WinGetProcess($var[$i][0]) Then

      ;messagebox is owned by webclient, give user 2.5 seconds to read....

      Sleep( 2500)

      ;close the message box

      WinClose($var[$i][0])

      ;for severe erros weblcient shuts down

    EndIf

  Next

  ;do not use 100% CPU power, just go to sleep for half a second

  Sleep( 500)

  ;we are finished if weblcient is shutdown or has ended

  $webclient = ProcessExists( "graph.exe")

WEnd

; Finished!
Appendix B "Stop error code 0x000000A5" installing Windows
Article ID: 314830

System TipThis article applies to a different version of Windows than the one you are using. This article was previously published under Q314830.

SYMPTOMS

When you are installing Windows XP, you may receive the following Stop error: 

STOP: 0x000000A5 (Parameter1, Parameter2, Parameter3, Parameter4)

The ACPI Bios in this system is not fully compliant to the specification. Please read the Readme.txt for possible workarounds, or contact your system vendor for an updated bios.

The bios in this system is not fully ACPI compliant. Please contact your system vendor for an updated bios. If you are unable to obtain an updated bios or the latest bios supplied by your vendor is not ACPI compliant, you can turn off ACPI mode during text mode setup. To do this, simply press the F7 key when you are prompted to install storage drivers. The system will not notify you that the F7 key was pressed--it will silently disable ACPI and allow you to continue your installation." 

The parameters Parameter1, Parameter2, Parameter3, and Parameter4 vary from computer to computer. 

CAUSE

This behavior can occur if Windows has detected that the BIOS in the computer is not fully compliant with Advanced Configuration and Power Interface (ACPI). 

RESOLUTION

To resolve this behavior, contact the manufacturer of your computer to obtain a BIOS that is fully ACPI compliant.

To work around this behavior, manually install the Standard PC hardware abstraction layer (HAL): 

Restart the computer to restart Setup.

When Setup starts again, press F7 (not F6) when you see the "Press F6 if you need to install a third-party SCSI or RAID driver" screen.

Windows automatically disables the installation of the ACPI HAL and installs the Standard PC HAL. 

MORE INFORMATION

The following information can help you debug a Stop 0x000000A5 error. 

The cause of the Stop message is always errors in the ACPI BIOS. There is nothing that you can fix at the level of the operating system.

This Stop message can occur at other times. For example, you could receive this Stop error after you install Windows XP and later make a hardware change, such as adding a device. A computer that works properly with Microsoft Windows 98 may not work with Windows XP. Windows 98 does not necessarily use all of the ACPI functions that Windows XP uses. Windows 98 may allow workarounds that Windows XP cannot use because of its more stringent requirements for system stability).

The following information lists possible causes of the Stop 0x000000A5 error. To identify the cause of this Stop error, note Parameter1 and compare it to the numbers listed in the following paragraphs, to get a general idea of the problem. 

The other parameters (arguments) are not discussed in this article. You can view these other arguments only by connecting the system to a kernel debugger. This article provides a description of the issues that this Stop error can involve, without requiring more advanced debugging. For more information, refer to Acpidbg.h.

(0x00000001, Parameter2, Parameter3, Parameter4):

This argument is defined as an ACPI root resources failure. More specifically, ACPI could not find the system control interrupt (SCI) vector in any of the resources that ACPI received at startup. SCI is a special interrupt type that provides a more efficient way to deal with problems that are normally handled by system management interrupts (SMI). If SCIs cannot be initialized, ACPI cannot function. This can occur if no entry for this IRQ is found in the list or if no IRQ resource list was found at all.

(0x00000002, Parameter2, Parameter3, Parameter4):

This error is defined as ACPI root PCI resource failure. To discover what current resources are being used by PCI devices, ACPI must be able to query the CRS descriptor in the ACPI namespace. This error occurs if the BIOS lacks a pointer to the list, or if the list is empty, or if the list contains errors or conflicts.

(0x00000003, Parameter2, Parameter3, Parameter4):

This error is defined as "ACPI failed must succeed method." This error occurs if ACPI cannot build a control method to reference the ACPI namespace. Other arguments for this error reference the ACPI object that was being run and the name of the control method. One greatly simplified explanation (which might not be completely accurate) is that the system cannot find a way to get to the ACPI tables that define the Plug and Play and Power Management capabilities of the system.

(0x00000004, Parameter2, Parameter3, Parameter4):

This error occurs because the system received some data type other than the expected integer as a package element when defining a _PRW method in the ACPI namespace. _PRW is only defined for systems that have the ability to wake the system from Sleep mode. Among other things, _PRW defines the lowest power system state that the system can enter and be able to wake up. 

(0x00000005, Parameter2, Parameter3, Parameter4):

The _PRW referenced must contain at least two elements. When queried, the _PRW indicated that it was incomplete.

(0x00000006, Parameter2, Parameter3, Parameter4):

A power resource is named, but there is no information available in the namespace for this resource.

(0x00000007, Parameter2, Parameter3, Parameter4):

When the system referenced a method in the ACPI namespace, the system expected a data type of BUFFER but received some other data type.

(0x00000008, Parameter2, Parameter3, Parameter4):

The system expected a data type of INTEGER but received some other data type.

(0x00000009, Parameter2, Parameter3, Parameter4):

The system expected a data type of PACKAGE but received some other data type.

(0x0000000A, Parameter2, Parameter3, Parameter4):

The system expected a data type of STRING but received some other data type.

(0x0000000B, Parameter2, Parameter3, Parameter4):

An _EJD string referenced a nonexistent object. The _EJD object defines objects that are dependent on a specific ejection device. For example, if you have an add-in adapter in a docking station and you attempt to eject the laptop computer, ACPI references this namespace to determine whether the dependent adapter in the docking station supports the ejection. This error occurs because there is no object to define the name referenced in the _EJD string.

(0x0000000C, Parameter2, Parameter3, Parameter4):

The namespace defines support for docking but lacks enough information to implement it. This can be because it is not properly defined or because there are duplicate services defined.

(0x0000000D, Parameter2, Parameter3, Parameter4):

There are many objects that must be defined in the ACPI namespace, and one or more of these was not found. This error generally occurs if either the _HID or _ADR object is missing. An _ADR is used to define the address of a device on its parent bus. The _ADR is a static address that defines slot numbers for devices that reside on any number of bus types, including EISA, floppy, IDE controller, IDE channel, PCI, PCMCIA, and CardBus. The _HID contains objects that supply the operating system with Plug and Play hardware ID for a device. Technically, these objects are optional, but they absolutely must be present to describe any device that is enumerated by an ACPI driver.

(0x0000000E, Parameter2, Parameter3, Parameter4):

This error occurs because the system could not find a required method or object in the namespace for a power resource. These are anything other than a "device." If the BIOS does not supply either an ON, OFF, or STA for a power resource, the system stops with this error.

(0x0000000F, Parameter2, Parameter3, Parameter4):

This occurs when a Plug and Play resource descriptor is smaller than its defined size, indicating that the descriptor is either incomplete or corrupted. Resource descriptors describe the system resources that the computer is using, can use, or wants to use. Each descriptor entry has a predefined size, which must match the data returned.

(0x000000010, Parameter2, Parameter3, Parameter4):

When a system transitions from one power state to another, the system compares the states that are supported system-wide to those supported by the individual devices in the system. This error occurs if Power Resource maps to a nonexistent system-wide resource. You can use a kernel debugger to view the specifics of this error, including the system power state that is causing the problem.

(0x000000011, Parameter2, Parameter3, Parameter4):

The system cannot enter ACPI mode. There are many reasons for this, including: 

The system cannot initialize the AML interpreter.

The system cannot find the Root System Description table.

The system cannot allocate a critical driver.

The system cannot load the Root System Description table.

The system cannot load device descriptor blocks.

The system cannot connect an interrupt vector.

The SCI_EN (system control interrupt enable request) cannot be set (see 0x00000001).

The ACPI Table checksum is incorrect.

ACPI is a hierarchical arrangement of tables, each one building upon the next to define the complete capabilities of the system and of every device in the system. ACPI starts by looking for the Root System Description table, which points to the next table, which points to the next table, and so on. Usually, the 0x000000011 error occurs because these tables are damaged or missing.

(0x000000012, Parameter2, Parameter3, Parameter4):

ACPI is expecting a Power Resource Object that it cannot find. You can view the error through a kernel debugger to see what the exact object is.

(0x00002001, Parameter2, Parameter3, Parameter4):

ACPI tried to evaluate the control method for a specific type of Programmable Interrupt Controller and did not succeed.

(0x00010001, Parameter2, Parameter3, Parameter4):

ACPI tried to perform interrupt routing and did not succeed. This error is usually due to a damaged interrupt routing table.

(0x00010002, Parameter2, Parameter3, Parameter4):

ACPI could not find a link node that is referenced in the interrupt routing table.

(0x00010003, Parameter2, Parameter3, Parameter4):

A device is present in the system, but there is no entry for the device in the IRQ routing table. 

(0x00010005, Parameter2, Parameter3, Parameter4):

This error relates to the rules about how a PCI routing table is written. To prevent ambiguity, the device number must be specified in the table but the function number must not. To this end, the function field must be set to all F's. This error occurs when a function number is not all F's.

(0x00010006, Parameter2, Parameter3, Parameter4):

ACPI must able to disabling a link node to reprogram it. This error occurs if ACPI cannot disable a link node.
Appendix C Een GPT-schijf converteren naar een MBR-schijf
Een GPT-schijf (GUID Partition Table) converteren naar een MBR-schijf (Master Boot Record, hoofdopstartrecord)

 Via de Windows-interface
1. Maak een back-up van alle volumes op de GPT-standaardschijf die u naar een MBR-schijf wilt converteren of verplaats ze. Als de schijf geen partities of volumes bevat, gaat u door met stap 2.

2. Zie 'Verwante onderwerpen' voor meer informatie over het maken van back-ups van volumes.

3. Open Computerbeheer (lokaal).

4. Klik in de consolestructuur op Computerbeheer (lokaal), klik op Opslag en klik vervolgens op Schijfbeheer.

5. Als de schijf geen partities of volumes bevat, gaat u verder met stap 5. Klik anders met de rechtermuisknop op de volumes op de schijf en klik vervolgens op Volume verwijderen.

6. Klik met de rechtermuisknop op de GTP-schijf die u naar een MBR-schijf wilt converteren en klik vervolgens op Converteren naar MBR-schijf.

Opmerkingen

· U kunt deze procedure alleen uitvoeren op een lokale computer als u lid bent van de groep Back-upoperators of de groep Administrators op de lokale computer, of als de juiste machtiging aan u is overgedragen. U kunt deze procedure alleen extern uitvoeren als u lid bent van de groep Back-upoperators of de groep Administrators op de externe computer. Als de computer deel uitmaakt van een domein, kan het zijn dat ook leden van de groep Domeinadministrators deze procedure kunnen uitvoeren. Uit veiligheidsoverwegingen kunt u beter Run as gebruiken om deze procedure uit te voeren. Zie voor meer informatie Standaard lokale groepen, Standaardgroepen en Uitvoeren als gebruiken.

· U opent Computerbeheer als volgt: klik op Start, klik op Configuratiescherm, dubbelklik op Systeembeheer en dubbelklik op Computerbeheer.

· U kunt een schijf alleen van GPT naar MBR converteren als de schijf leeg is.

· Itanium-computers, x86-computers met Windows Server 2003 en Service Pack 1 (SP1) en x64-computers ondersteunen MBR- en GPT-schijven.

 Met een opdrachtregel

1. Maak een back-up van alle volumes op de GPT-standaardschijf die u naar een MBR-schijf wilt converteren of verplaats ze. Als de schijf geen partities of volumes bevat, gaat u door met stap 2.
Zie 'Verwante onderwerpen' voor meer informatie over het maken van back-ups van volumes.

2. Open een opdrachtprompt.
3. Typ het volgende:
diskpart
Als de schijf geen partities of volumes bevat, gaat u door met stap 7.

4. Typ het volgende achter de DISKPART-prompt:
list volume
Noteer het nummer van het volume dat u wilt verwijderen.

5. Typ het volgende achter de DISKPART-prompt:
select volume n
6. Typ het volgende achter de DISKPART-prompt:
delete volume
7. Typ het volgende achter de DISKPART-prompt:
list disk
Noteer het nummer van de schijf die u naar een MBR-schijf wilt converteren.

8. Typ het volgende achter de DISKPART-prompt:
select disk n
9. Typ het volgende achter de DISKPART-prompt:
convert mbr
Belangrijk

U kunt een schijf pas naar MBR converteren als deze leeg is. Maak een back-up van uw gegevens en verwijder vervolgens alle partities of volumes voordat u de schijf converteert.
	Waarde
	Beschrijving

	list volume 
	Hiermee wordt een overzicht weergegeven van standaardvolumes en dynamische volumes op alle schijven.

	select volume 
	Hiermee wordt het opgegeven volume geselecteerd en in focus gebracht, waarbij n het volumenummer is. Als er geen volume is opgegeven, wordt met de opdracht select het huidige volume in focus weergegeven. U kunt het volume opgeven aan de hand van het volumenummer, de stationsletter of het koppelpuntpad. Als u op een standaardschijf een volume selecteert, krijgt de bijbehorende partitie de focus.

	delete volume 
	Hiermee verwijdert u het geselecteerde volume. Het is niet mogelijk om een systeempartitie, opstartpartitie of een partitie met een actief wisselbestand of een crashdump (geheugendump) te verwijderen.

	list disk 
	Hiermee wordt een overzicht weergegeven van schijven en van informatie over deze schijven, zoals de grootte, de beschikbare hoeveelheid vrije ruimte, of de schijf een standaardschijf is of een dynamische schijf, en of de schijf gebruikmaakt van een MBR (Master Boot Record, hoofdopstartrecord) of een GPT (GUID Partition Table). De schijf die met een sterretje (*) is gemarkeerd, heeft de focus.

	select disk 
	Hiermee wordt de opgegeven schijf geselecteerd en in focus gebracht, waarbij n het nummer van de schijf is.

	convert mbr 
	Hiermee wordt een lege standaardschijf met de GPT-partitiestijl geconverteerd naar een standaardschijf met de MBR-partitiestijl.


Opmerkingen

· U kunt deze procedure alleen uitvoeren op een lokale computer als u lid bent van de groep Back-upoperators of de groep Administrators, of als de juiste machtiging aan u is overgedragen. Als de computer deel uitmaakt van een domein, kan het zijn dat ook leden van de groep Domeinadministrators deze procedure kunnen uitvoeren. Uit veiligheidsoverwegingen kunt u beter Run as gebruiken om deze procedure uit te voeren. Zie voor meer informatie Standaard lokale groepen, Standaardgroepen en Uitvoeren als gebruiken.

· U opent een opdrachtprompt als volgt: klik op Start, wijs achtereenvolgens Alle programma's en Bureau-accessoires aan en klik op Opdrachtprompt.

· U kunt een schijf alleen van GPT naar MBR converteren als de schijf leeg is.

· Itanium-computers, x86-computers met Windows Server 2003 en Service Pack 1 (SP1) en x64-computers ondersteunen MBR- en GPT-schijven.

· Zie 'Verwante onderwerpen' voor meer informatie over DiskPart.

Informatie over functionele verschillen

· Het is mogelijk dat uw server anders werkt. Dit hangt samen met de versie en editie van het geïnstalleerde besturingssysteem, de machtigingen van uw account en uw menu-instellingen. Zie Help-informatie bekijken op het web voor meer informatie.

Appendix D Requirements tracebility matrix

The table below presents the software requirements versus components traceability matrix.

[The table contains an example. Relations can be one-to-one or one-to-many.]

	Requirements traceability matrix 

DDD traced naar SRD
	Date:

dd mon yyyy

pagina 1 van <nn>

	Project:Projectnaam
	

	SRD Identifier
	DDD Identifier
	Samenvatting van de Software Requirement

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


Appendix E Conventions

<sub>.<doc>.<e>.<nnn>[(<type>)]

<sub>
project afhankelijk: sub-systeem binnen een project

<doc>
document: URD of SRD

<e>

gewicht van requirement




E = Essentieel, D = Wenselijk

<nnn>
serienummer requirement (opeenvolgend), 1..999

<type>
extra veld voor het tonen van het requirement type:


F
= functioneel,
            P
=    performance,

I      =  interface, 


O
= operationeel, 
            R
=    resource, 

V     =  verificatie, 


A
= acceptatie-test,              D
=    documentatie, 
RE   =  betrouwbaarheid, 


PO 
= portabiliteit, 
            Q
=    kwaliteit, 

SA   =  veiligheid.


M
= onderhoudbaarheid,      S    =    beveiliging


147-149
ADVANCE \D 2.85Installatie FactoryLink server PC Versie 7.5.2
P15450 Install proc FLserver 75.doc
ADVANCE \D 2.85Auteur(s): Marcel Jordaan (RLD)
Classificatie: Algemeen  Versie: 1.1 Status: Definitief




_1476352651.unknown

_1476352652.unknown

_1476352653.unknown

_1232777613.vsd
-


 


 


-



