This article describes a network connectivity issue with HP desktops with Intel 82567LM-3 network cards.

## Symptoms

HP computers have issues negotiating a link speed when the system is idle or coming out of a standby state. This can cause GPO objects, application connections, mapped drives, and other services to fail when the system resumes.

In the windows event viewer you may see the following errors or entries in series when the system resumes from standby:

- Event 27

Intel 82567LM-3 Gigabit Network Connection

Network link is disconnected.

- Event 33

Intel 82567LM-3 Gigabit Network Connection

Network link has been established at 1Gbps full duplex.

- Event 32

Intel 82567LM-3 Gigabit Network Connection

Network link has been established at 100Mbps full duplex.

The system may also display Event 1054 at this time.

You may also see the error "Time out of sync when waking from standby."

## Cause

This is a known issue that appears to be caused by network cabling and an older system BIOS.

## Solution

Upgrade to the latest system BIOS.

For HP 6000 computers, upgrading the 82567LM-3 network card driver to v11.12.36.0 also resolves this issue.

## Applies to

Product

- HP Compaq 8000 Elite with Intel 82567 series network cards

- HP Compaq 6000 Pro with Intel 82567 series network cards

- HP Compaq dc7900 with Intel 82567 series network cards

Operating system

- Windows XP

- Windows Vista

- Windows 7

HP desktop

‹ Fine-tuning your SURVEYOR deployment up How SmartProfiles work ›

- Log in or register to post comments

WORKAROUND:
Run regedit
Navigate to

[HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Control\Class\{4D36E972-E325-11CE-BFC1-08002bE10318}\XXXX

where XXXX is the key number (ie 0008) that pertains to the 82579LM adapter (do a search of the keys) 
find the subkey “LinkNegotiationProcess”.  If the subkey is present, change the value to 2.
If subkey is not present, create it, add a string value and set it to 2
Exit regedit, reboot the system.

## System specifications

| Specification | Description |
| --- | --- |
| Chassis dimensions (H x W x D) | - Standard minitower orientation : 447 x 178 x 455 mm (17.6 x 7 x 17.9 in) - Converted desktop orientation : 178 x 447 x 455 mm (7 x 17.6 x 17.9 in) |
| Weight1 | Exact weights depend upon configuration : - Minimum : 10.4 kg (22.9 lb) - Standard : 11.7 kg (25.8 lb) - Maximum : 14.8 kg (32.6 lb) |
| System board form factor | ATX 244 x 305 mm (9.6 x 12 in) |
| Processor socket | Single LGA 1155 |
| CPU bus speed | DMI |
| Chipset | Intel PCH C206 |
| Memory expansion slots | 4 DDR3 memory slots |
| Memory type supported | - DDR3 - UDIMM (Unbuffered) - ECC & non-ECC |
| Memory modes | Non-interleaved for single channel. Interleaved when both channels are populated. |
| Memory speed supported | 1333 MHz DDR3 |
| Memory protection | ECC available on data, parity on address and command |
| Maximum memory | 16GB |
| Memory configuration (supported) | 1GB, 2GB and 4GB ECC and non-ECC unbuffered DIMMs are supported, but not if mixed. |
| PCI express connectors | - 1 PCI Express Gen2 slot x8 mechanical/x4 electrical, open-ended - 1 PCI Express Gen2 slot x16 mechanical/x16 electrical (used for discrete graphics), open-ended - 1 PCI Express Gen2 slot x1 mechanical/x1 electrical - 1 PCI Express Gen2 slot x16 mechanical/x4 electrical - 1 PCI Express Gen2 slot x1 mechanical/x1 electrical |
| PCI connectors (5.0 V) | 2 PCI |
| Interfaces supported | Integrated (6) Serial ATA interfaces (2 x 6 Gb/s SATA, 4 x 3 Gb/s SATA). One port can optionally be used for eSATA). RAID 0 and 1 supported. (Factory integrated RAID is Microsoft Windows only). RAID 5 is supported by Software XOR. |
| Serial attached SCSI | None |
| Integrated graphics | - Integrated Intel HD Graphics 2000 (on Intel Core i3/Core i5/Core i7 processors). Integrated Intel HD Graphics P3000 (on Intel Xeon E3-12x5 processors). - Unified Memory Architecture (UMA)- A region of system memory is reserved and dedicated to the graphics display. DirectX 10.0 compliant; (OpenGL 3.0 on Intel HD Graphics P3000); 1 VGA & 1 DP graphics ports integrated in motherboard. - Integrated graphics can support dual display across DP & DVI-I outputs. |
| Network controller | Integrated Gbit LAN MAC by Intel PHY Lewisville 82579LM; Management capabilities WOL, PXE 2.1 and AMT 7 |
| External SATA (eSATA) | 1 port eSATA capable with optional eSATA After-Market Option cable kit. |
| IDE connector | No |
| Floppy connector | No |
| Network controller | Management capabilities WOL, PXE 2.1 and AMT 7 |
| Serial | 1 internal header (requires optional Serial Port Adapter Kit). |
| 2nd serial | No |
| Parallel | 1 internal header (optional parallel port adapter required). |
| HD integrated audio | Yes |
| CD-ROM input (Audio) | No |
| AUX input (Audio) | No |
| IEEE 1394 connector(s) | - Front : 1 IEEE 1394a (requires optional PCIe card to function) - Rear : No - Internal : No |
| USB connector(s) | - Front : 3 USB 2.0 - Rear : 6 USB 2.0 - Internal : 5 USB 2.0 ports available by two separate 2x5 and one 1x5 header: supports one HP Internal USB Port Kits, (one port on each Kit) for 1x5 pin header plus (a) up to two USB Media Card Readers, or (b) one Internal Port kit and one USB Media Card Reader. |
| HD integrated audio | Yes |
| CD-ROM input (Audio) | No |
| AUX input (Audio) | No |
| Chassis fan header | - 1 Rear System Chassis Fan Header - 1 Optional Front Chassis Fan Header |
| CPU fan header | Yes |
| Front PCI fan header | Yes |
| Front control panel/Speaker header | Yes |
| CMOS battery holder - Lithium | Yes |
| Integrated trusted platform module | Integrated TPM 1.2. The TPM module disabled where restricted by law, i.e. Russia. |
| Power supply headers | Yes |
| Power switch, power LED & Hard Drive LED header | Yes |
| Clear password jumper | Yes |
| Keyboard/Mouse | USB or PS/2 |
| ENERGY STAR qualified (Config Dependent) | Yes |
| 80 PLUS compliant | Yes, gold |
| NOTE: 1 : Configured with 1 hard drive, 1 optical drive and one NVIDIA Quadro NVS 295 low profile graphics card. |  |

top

## Electrical specifications

| Specification | Description |
| --- | --- |
| Power supply | 400 W wide ranging, Active PFC, 90% Efficient |
| Power supply fan | 92 x 25 mm variable speed |
| Operating voltage range | 90 to 269 V ac |
| Rated voltage range | 100 to 240 V ac |
| Rated line frequency | 50/60 Hz |
| Operating line frequency range | 47 to 66 Hz |
| Rated input current | 6 A @100-127 V |
| Heat dissipation | - Typical : 229.3 kg-cal/hr (910 BTU/hr) - Maximum : 361.6 kg-cal/hr (1435 BTU/hr) |
| FEMP standby power compliant | Yes, with Wake-on-LAN disabled: <2 W in S5-power Off |
| Power consumption in sleep mode (as defined by ENERGY STAR) - Suspend to RAM (S3) | <4 W |
| Surge tolerant full ranging power supply (withstands power surges up to 2000 V) | Yes |
| ErP Lot 6- Tier 1 Compliance @ 230 V (<1 W in S5- Power Off) | Yes |
| ErP Lot 6- Tier 2 Compliance @ 230 V (<0.5 W in S5- Power Off) | Yes |

top

## Environmental specifications

| Specification | Description |
| --- | --- |
| Temperature | - Operating : 5° to 35°C (40° to 95°F) - Non-operating : -40° to 60°C (-40° to 140°F) |
| Humidity | - Operating : 8% to 85% RH, non-condensing - Non-operating : 8% to 90% RH, non-condensing |
| Maximum altitude | - Operating : 3,000 m (10,000 ft) - Non-operating : 9,100 m (30,000 ft) |
| Dynamic shock | - Operating : Half-sine: 40 g, 2-3 ms - Non-operating : - Half-sine : 160 cm/s, 2-3 ms (upto 100 g) - Square : 422 cm/s, 20 g |
| Dynamic vibration1 | - Operating : 0.5 g (rms), 5-300 Hz - Non-operating : 2.0 g (rms), 10-500 Hz |
| Cooling | Above 1524 m (5,000 ft) altitude, maximum operating temperature is de-rated by 1°C (1.8°F) per 305 m (1,000 ft) elevation increase |
| NOTE: 1 : Values represent individual shock events and do not indicate repetitive shock events. Values do not indicate continuous vibration. |  |

top

## Multimedia

| Specification | Description |
| --- | --- |
| Multimedia and audio devices | - Integrated Intel/Realtek HD ALC261 Audio - HP Thin USB Powered Speakers, BFR-PVC free - Creative X-Fi Titanium PCIe Audio Card |

top

## Storage

| Specification | Description |
| --- | --- |
| Optical and removable storage | - HP 16X DVD-ROM SATA Drive - HP 16X DVD+-RW SuperMulti SATA Drive - HP 22-in-1 Media Card Reader Kit (Workstations) - HP Blu-ray Writer |

top

## Networking and communications

| Specification | Description |
| --- | --- |
| Intel Gigabit CT Desktop NIC | - Connector : RJ-45 - Controller : Intel WG82574L Gigabit Ethernet Controller - Memory : Integrated Dual 48K configurable transmit receive FIFO Buffers - Data Rates Supported : 10/100/1000 Mb/s - Compliance : IEEE 802.1P, 802.1Q, 802.2, 802.3, 802.3AB and 802.3u compliant, 802.3x flow control |
| Network transfer rate | - 10BASE-T (half-duplex) 10 Mb/s - 10BASE-T (full-duplex) 20 Mb/s - 100BASE-TX (half-duplex) 100 Mb/s - 100BASE-TX (full-duplex) 200 Mb/s - 1000BASE-T (full-duplex) 2000 Mb/s |

top

## Other hardware

| Features | Description |
| --- | --- |
| Monitors1 | - HP ZR30w 30-inch S-IPS LCD Monitor - HP DreamColor LP2480zx Professional Display - HP ZR24w 24-inch S-IPS LCD Monitor - HP ZR22w 21.5-inch S-IPS LCD Monitor - HP LP3065 30-inch Widescreen LCD Monitor - HP LP2475w 24-inch Widescreen LCD Monitor - HP LP2065 20-inch LCD Monitor |
| Input devices | - HP USB Laser Mouse - HP SpacePilot 3D USB Intelligent Controller - HP SpaceExplorer 3D USB Controller - HP USB 2-Button Optical Scroll Mouse - HP USB Standard Keyboard - HP PS/2 Optical Scroll Mouse - HP PS/2 Standard Keyboard - HP USB Optical 3-Button Mouse - HP USB Smart Card Keyboard - HP 2.4 GHz Wireless Keyboard and Mouse - HP USB CCID SmartCard Keyboard |
| Miscellaneous | - HP Power Cord Kit - HP Workstation Mouse Pad - HP Serial Port Adapter - HP Internal USB Port Kit - HP ENERGY STAR 5.0 Enabled Configuration - HP Parallel Port Adapter Kit |
| NOTE: 1 : Screen size is measured diagonally. |  |