## Writing a Library for Arduino

This document explains how to create a library for Arduino. It starts with a sketch for flashing Morse code and explains how to convert its functions into a library. This allows other people to easily use the code that you've written and to easily update it as you improve the library.

For more information, see the API Style Guide for information on making a good Arduino-style API for your library.

We start with a sketch that does simple Morse code:

int pin = 13;

void setup()
{
  pinMode(pin, OUTPUT);
}

void loop()
{
  dot(); dot(); dot();
  dash(); dash(); dash();
  dot(); dot(); dot();
  delay(3000);
}

void dot()
{
  digitalWrite(pin, HIGH);
  delay(250);
  digitalWrite(pin, LOW);
  delay(250);
}

void dash()
{
  digitalWrite(pin, HIGH);
  delay(1000);
  digitalWrite(pin, LOW);
  delay(250);
}

[Get Code]

If you run this sketch, it will flash out the code for SOS (a distress call) on pin 13.

The sketch has a few different parts that we'll need to bring into our library. First, of course, we have the dot() and dash() functions that do the actual blinking. Second, there's the ledPin variable which the functions use to determine which pin to use. Finally, there's the call to pinMode() that initializes the pin as an output.

Let's start turning the sketch into a library!

You need at least two files for a library: a header file (w/ the extension .h) and the source file (w/ extension .cpp). The header file has definitions for the library: basically a listing of everything that's inside; while the source file has the actual code. We'll call our library "Morse", so our header file will be Morse.h. Let's take a look at what goes in it. It might seem a bit strange at first, but it will make more sense once you see the source file that goes with it.

The core of the header file consists of a line for each function in the library, wrapped up in a class along with any variables you need:

class Morse
{
  public:
    Morse(int pin);
    void dot();
    void dash();
  private:
    int _pin;
};

[Get Code]

A class is simply a collection of functions and variables that are all kept together in one place. These functions and variables can be public, meaning that they can be accessed by people using your library, or private, meaning they can only be accessed from within the class itself. Each class has a special function known as a constructor, which is used to create an instance of the class. The constructor has the same name as the class, and no return type.

You need a couple of other things in the header file. One is an #include statement that gives you access to the standard types and constants of the Arduino language (this is automatically added to normal sketches, but not to libraries). It looks like this (and goes above the class definition given previously):

#include "Arduino.h"

[Get Code]

Finally, it's common to wrap the whole header file up in a weird looking construct:

#ifndef Morse_h
#define Morse_h

// the #include statment and code go here...

#endif

[Get Code]

Basically, this prevents problems if someone accidently #include's your library twice.

Finally, you usually put a comment at the top of the library with its name, a short description of what it does, who wrote it, the date, and the license.

Let's take a look at the complete header file:

/*
  Morse.h - Library for flashing Morse code.
  Created by David A. Mellis, November 2, 2007.
  Released into the public domain.
*/
#ifndef Morse_h
#define Morse_h

#include "Arduino.h"

class Morse
{
  public:
    Morse(int pin);
    void dot();
    void dash();
  private:
    int _pin;
};

#endif

[Get Code]

Now let's go through the various parts of the source file, Morse.cpp.

First comes a couple of #include statements. These give the rest of the code access to the standard Arduino functions, and to the definitions in your header file:

#include "Arduino.h"
#include "Morse.h"

[Get Code]

Then comes the constructor. Again, this explains what should happen when someone creates an instance of your class. In this case, the user specifies which pin they would like to use. We configure the pin as an output save it into a private variable for use in the other functions:

Morse::Morse(int pin)
{
  pinMode(pin, OUTPUT);
  _pin = pin;
}

[Get Code]

There are a couple of strange things in this code. First is the Morse:: before the name of the function. This says that the function is part of the Morse class. You'll see this again in the other functions in the class. The second unusual thing is the underscore in the name of our private variable, _pin. This variable can actually have any name you want, as long as it matches the definition in the header file. Adding an underscore to the start of the name is a common convention to make it clear which variables are private, and also to distinguish the name from that of the argument to the function (pin in this case).

Next comes the actual code from the sketch that you're turning into a library (finally!). It looks pretty much the same, except with Morse:: in front of the names of the functions, and _pin instead of pin:

void Morse::dot()
{
  digitalWrite(_pin, HIGH);
  delay(250);
  digitalWrite(_pin, LOW);
  delay(250);  
}

void Morse::dash()
{
  digitalWrite(_pin, HIGH);
  delay(1000);
  digitalWrite(_pin, LOW);
  delay(250);
}

[Get Code]

Finally, it's typical to include the comment header at the top of the source file as well. Let's see the whole thing:

/*
  Morse.cpp - Library for flashing Morse code.
  Created by David A. Mellis, November 2, 2007.
  Released into the public domain.
*/

#include "Arduino.h"
#include "Morse.h"

Morse::Morse(int pin)
{
  pinMode(pin, OUTPUT);
  _pin = pin;
}

void Morse::dot()
{
  digitalWrite(_pin, HIGH);
  delay(250);
  digitalWrite(_pin, LOW);
  delay(250);  
}

void Morse::dash()
{
  digitalWrite(_pin, HIGH);
  delay(1000);
  digitalWrite(_pin, LOW);
  delay(250);
}

[Get Code]

And that's all you need (there's some other nice optional stuff, but we'll talk about that later). Let's see how you use the library.

First, make a Morse directory inside of the libraries sub-directory of your sketchbook directory. Copy or move the Morse.h and Morse.cpp files into that directory. Now launch the Arduino environment. If you open the Sketch > Import Library menu, you should see Morse inside. The library will be compiled with sketches that use it. If the library doesn't seem to build, make sure that the files really end in .cpp and .h (with no extra .pde or .txt extension, for example).

Let's see how we can replicate our old SOS sketch using the new library:

#include <Morse.h>

Morse morse(13);

void setup()
{
}

void loop()
{
  morse.dot(); morse.dot(); morse.dot();
  morse.dash(); morse.dash(); morse.dash();
  morse.dot(); morse.dot(); morse.dot();
  delay(3000);
}

[Get Code]

There are a few differences from the old sketch (besides the fact that some of the code has moved to a library).

First, we've added an #include statement to the top of the sketch. This makes the Morse library available to the sketch and includes it in the code sent to the board. That means if you no longer need a library in a sketch, you should delete the #include statement to save space.

Second, we now create an instance of the Morse class called morse:

Morse morse(13);

[Get Code]

When this line gets executed (which actually happens even before the setup() function), the constructor for the Morse class will be called, and passed the argument you've given here (in this case, just 13).

Notice that our setup() is now empty; that's because the call to pinMode() happens inside the library (when the instance is constructed).

Finally, to call the dot() and dash() functions, we need to prefix them with morse. - the name of the instance we want to use. We could have multiple instances of the Morse class, each on their own pin stored in the _pin private variable of that instance. By calling a function on a particular instance, we specify which instance's variables should be used during that call to a function. That is, if we had both:

Morse morse(13);
Morse morse2(12);

[Get Code]

then inside a call to morse2.dot(), _pin would be 12.

If you tried the new sketch, you probably noticed that nothing from our library was recognized by the environment and highlighted in color. Unfortunately, the Arduino software can't automatically figure out what you've define in your library (though it would be a nice feature to have), so you have to give it a little help. To do this, create a file called keywords.txt in the Morse directory. It should look like this:

Morse   KEYWORD1
dash    KEYWORD2
dot     KEYWORD2

[Get Code]

Each line has the name of the keyword, followed by a tab (not spaces), followed by the kind of keyword. Classes should be KEYWORD1 and are colored orange; functions should be KEYWORD2 and will be brown. You'll have to restart the Arduino environment to get it to recognize the new keywords.

It's also nice to provide people with an example sketch that uses your library. To do this, create an examples directory inside the Morse directory. Then, move or copy the directory containing the sketch (let's call it SOS) we wrote above into the examples directory. (You can find the sketch using the Sketch > Show Sketch Folder command.) If you restart the Arduino environment (this is the last time, I promise) - you'll see a Library-Morse item inside the File > Sketchbook > Examples menu containing your example. You might want to add some comments that better explain how to use your library.

If you'd like to check out the complete library (with keywords and example), you can download it: Morse.zip.

That's all for now but I'll probably write an advanced library tutorial soon. In the meantime, if you have any problems or suggestions, please post them to the Software Development forum.

For more information, see the API Style Guide for information on making a good Arduino-style API for your library.

# Arduino Style Guide for Writing Libraries

This is a style guide to writing library APIs in an Arduino style. Some of these run counter to professional programming practice. We’re aware of that, but it’s what’s made it possible for so many beginners to get started with Arduino easily. So please code with these principles in mind. If you have suggestions on how to make Arduino libraries clearer for that core audience, please jump in the discussion. This is a work in progress.

Be kind to the end user. Assume you are writing an API for an intelligent person who has not programmed before. Come up with a clear mental model of the concept you’re working with, and the terms and functions you will use.

Match your API to the underlying capabilities. You don’t want to expose implementation details to the user but you also don’t want an API that suggests an inaccurate mental model of the possibilities. For example, if there are only a few possible options for a particular setting, don’t use a function that takes an int, as it implies you can use any value you want.

Organize your public functions around the data and functionality that the user wants. Quite often, the command set for a particular electronic module is overly complicated for the most common uses, or can be re-organized around higher level functionality. Think about what the average person thinks the thing does, and try to organise your API functions around that. Adafruit's BMP085 library is a good example. The readPressure() command performs all the necessary steps to get the final pressure. The library wraps this commonly executed series of functions into a high-level single command which returns the value the user's looking for in a format she expects. It abstracts away not only the low-level I2C commands, but also the mid-level temperature and pressure calculations, while still offering those mid-level functions as public functions for those who want them.

Use full, everyday words. Don’t be terse with your function names or variables. Use everyday terms instead of technical ones. Pick terms that correspond to popular perception of the concept at hand. Don’t assume specialized knowledge. For example, this is why we used analogWrite() rather than pwm(). Abbreviations are acceptable, though, if they’re in common use or are the primary name for something.For example, “HTML” is relatively common and “SPI” is effectively the name of that protocol (“serial-peripheral interface” is probably too long). (“Wire” was probably a mistake, as the protocol it uses is typically called “TWI” or “I2C”.)

Avoid words that have different meanings to the general public. For example, to programmers, an error is a notification that something happened. To the general public, errors are bad things.

When you have to use a domain-specific term, write a sentence or two describing it to the general public FIRST. You’ll likely come across a better term, and if not, you’ll have started the documentation on your library.

Document and comment as you go. When writing examples and documentation, follow this style guide: http://arduino.cc/en/Reference/StyleGuide

Use the established core libraries and styles.

- Use read() to read inputs, and write() to write to outputs, e.g. digitalRead(), analogWrite(), etc.

- Use the Stream.h and Print.h libraries when dealing with byte streams. If it’s not appropriate, at least try to use its API as a model. For more on this, see below

- For network applications, use the Client and Server libraries as the basis.

- Use begin() to initialize a library instance, usually with some settings. Use end() to stop it.

Use camel case function names, not underscore. For example, analogRead, not analog_read. Or myNewFunction, not my_new_function. We've adopted this from Processing.org for readability's sake.

LONG_CONSTANT_NAMES_FULL_OF_CAPS are hard to read. Try to simplify when possible, without being terse.

Try to avoid boolean arguments. Instead, consider providing two different functions with names the describe the differences between them.

Don’t assume knowledge of pointers. Beginning users of C find this the biggest roadblock, and get very confused by & and *, so whenever you can avoid having them hanging out in the API, do so. One way is to pass by reference using array notation rather than * notation, for example.

void printArray( char* array);

can be replaced by

void printArray(char[] array);

Though there are some libraries where we pass pointers by using structures like const chars, avoid anything that requires the user to pass them. For example,rather than:

foo.readAccel(&x, &y, &z);

use something like this:

xAxis = adxl.readX();

yAxis = adxl.readY();

zAxis = adxl.readZ();

When using serial communication, allow the user to specify any Stream object, rather than hard-coding "Serial". This will make your library compatible all serial ports on Mega and the Due, and can also use alternate interfaces like SoftwareSerial. The Stream object can be passed to your library's constructor or to a begin() function (as a reference, not a pointer). See Firmata 2.3 or XBee 0.4 for examples of each approach.

When writing a library that provides byte-stream communication, inherit Arduino's Stream class, so your library can be used with all other libraries that accept Stream objects. If possible, buffer incoming data, so that read() immediately accesses data the buffer but does not wait for more data to arrive. If possible, your write() method should store data to a transmit buffer, but write() must wait if the buffer does not have enough space to immediately store all outgoing data. The yield() function should be called while waiting.

Here are a few libraries that are exemplary from Adafruit. She breaks the functions of the devices down into their high-level activities really well. https://github.com/adafruit/Adafruit-BMP085-Library https://github.com/adafruit/DHT-sensor-library

This does a nice job of abstracting from the Wire (I2C) library: https://github.com/adafruit/RTClib Reference Home

Corrections, suggestions, and new documentation should be posted to the Forum.

The text of the Arduino reference is licensed under a Creative Commons Attribution-ShareAlike 3.0 License. Code samples in the reference are released into the public domain.