Procedure to assign an IP address to a M1E using the Bootp lite software

- Bootp lite software is a DOS based program.  Therefore you will need to launch a DOS Command Prompt to run it.   Go to Start menu and select RUN.  Type in CMD as shown below.

- Go to the path where the bootplt.exe file is in.

Type in the following command to execute the program.

Bootplt [mac address] [IP address]

The mac address is written on a white sticker on the M1E.

For example if the mac address is 000054101762 and you want to assign 192.168.141.6 for IP address you need to type in the following command:

Bootplt 00005410172 192.168.141.6

- Press enter and follow the direction

3) Make sure that the IP address does not already exist and press Y to continue.

- Cycle power the M1E.  Bootp lite will send 10 Bootp messages to assign the IP address, than it will ask you if you want to save the current IP address to flash ram. In this case, you will not be able to save the current IP address to memory because it is not a Momentum ENT module (170ENT11000).  If you want to save the IP address to memory you will need to use the PLC programming program like Concept or Proworks NXT.

Note: Make sure that you check with your Network Administrator before assigning or changing any IP addresses.

Customer Support

Schneider Electric